BIM Studio	Tel  +852 3468 5250

A2, 35/F, TML Tower	Fax  +852 3585 5599

3 Hoi Shing Road,	hongkong@a-c-i-d.com Tsuen Wan, N.T.	www.a-c-i-d.com

BIM Submission and Purpose

| Stage | Submit format and content | Information required | Submission purpose |
| --- | --- | --- | --- |
| Early Planning stage | Topography | Contour and coordinates for project area | For sunshade study, landscape study, excavation and back fill, even sub-structure analysis |
|  | Utilities analysis | Existing drainage and sewage system, electricity, gas and telecom system | Preparation for design of underground services |
|  | Traffic analysis | Road map | To locate the main and secondary entrance of the building |
|  | Build mass model to generate the preliminary area schedule | Master plan and floor plans | Preliminary area schedule could be use as early financial analysis |
| Design stage | Sunshade, wind, energy consumption analysis for project | Site contour, existing landscape analysis, includes tree location, species and ages | For design reference, building location and building design checking |
|  | Excavation analysis | Site topography, master plan and building plans, levels | Excavation analysis could be used as design checking, which could help to revise the design to make sure the excavation keep at minimum level |
|  | Landscape model, includes trees within project boundary | Site topography, site landscape report and arborist report | Could be used for landscape design, to relocate the purposed tree on the landscape model, and for visual checking |
|  | Tree crown, roots and growth analysis | Tree species, age; dimension of tree crown and roots | Tree crown analysis for landscape design visual analysis; Tree root analysis for utilities design |
|  | Clash detection between building and tree crown | Masterplan, building design; tree species, age and dimension of crown and roots | The clash detection will be used for landscape design |
|  | Design options | Masterplan, preliminary architectural and landscape design | Analyze design options to achieve quality design for architecture, master planning and landscape |
|  | Architectural BIM model: room names, function, numbers and area | Masterplan and general arrangement plan, elevations and roof design | For drawing production, material schedule and area schedule |

|  | Architectural drawings: Masterplan, general arrangement plans, elevations and sections (up to 1:100) | Masterplan and floor plans, elevations and roof design | For tender drawings and construction drawings |
| --- | --- | --- | --- |
|  | Area schedule | Masterplan, floor plan and levels | Area schedule from BIM model could provide accurate area instantly, not over or a lot less than the limitation |
|  | Schedule of quantities | Architectural design and materials | Schedule of windows, doors or louvres, help QS to start with preliminary quantity take-off |
|  | Structural BIM model: piling, pile cap, substructure and superstructure | Structural design for piling, pile cap, ELS, demolition plan, substructure and superstructure | For structural drawing production, export model for structural calculation |
|  | Hoarding design | Hoarding location and height | Commercial ads on hoarding |
|  | Export structural model | Structural design for piling, pile cap, substructure and superstructure | For structural calculation |
|  | MEP BIM model: MVAC, P&D, FS and electrical design | MEP design for MVAC, P&D, FS and Electrical, dimension and level of ducting and pipework | All MEP elements were model to make sure the design feasibility |
| Detail design | Drawing production for architecture, structure and MEP | Architectural, structural and MEP design | For government submission (GBP) |
|  | Clash detection | Architectural, structural and MEP design | Design coordination |
|  | Spatial validation checks for headroom and working space | Architectural, structural and MEP design (all levels shall be clearly mentioned) | For building and services design, building operations and maintenance activities checking |
|  | Drawing revision | Architectural, structural and MEP design revisions | Make sure all 2D and 3D information update and consistent |
|  | Schedule revision | Architectural, structural and MEP design revisions | Schedule updated as per design updates will avoid the time gap between design consultant and QS |
|  | animation | Architectural, structural, MEP and landscape design | To visualize the design in virtual reality for design checking |
|  | Update design analysis: sunshade, landscape etc | Architectural, structural, MEP and landscape design | To design improvement |
| Tender / Construction | Clash detection revision | Architectural, structural and MEP design revisions | Constantly clash checking to avoid design fault to minimize unnecessary construction cost |

|  | Drawing production | Architectural, structural and MEP design revisions | Construction drawings |
| --- | --- | --- | --- |
|  | Schedule revision | Architectural, structural and MEP design revisions, material revisions | For quantity take off |
|  | 4D animation for construction sequence | Architectural, structural and MEP design revisions; construction programme and method statement | Construction sequence checking, clash between trades could be detected before commencement. |
|  | Shop drawing | Shop drawing and method statement | Shop drawing produced from BIM model to ensure the drawing and model are consistent |
|  | CSD | Architectural, structural, MEP drawings from contractor | test the feasibility of MEP ductwork / pipework, clash detection between MEP and Architecture / Structure |
|  | Procurement, cash flow and payment | Contractor’s design for architecture, structure and MEP, material and equipment to be used | Procurement, lead time and payment will affect the construction programme |
|  | Construction safety check | Construction equipment | 4D Construction simulation to highlight the construction sequence, while at the same time to visualize the temporary works during construction to ensure the site safety, delivery route, protective measures etc. |
|  | Site management | Procurement schedule, lead time and site logistic planning, installation detail | To utilize the 4D Construction Simulation techniques facilitate the site planning - material storage, delivery path, hoist management and installation logistics. |
| Construction completed | As-built model | Contractor’s architecture, structure, MEP and landscape design, and MEP equipment | Related information to be stored in the model, extra information to future facility management purpose will be added |
|  | COBie | Material, O&M manual, warranty and maintenance information | The information will be export and re-store into model in EcoDomus for facility management purpose |
| Facility management | Model and COBie information re- combine in EcoDomus for facility management | Model: contractor’s design for architecture, structure, MEP and related equipment, landscape design Information: information that facility management required, e.g. material, O&M manual and warranty | Extra time and human resource required for facility management because of the insufficient information, massive information could be centralize and accessible when BIM technology introduced into facility management |

| Marketing purpose | Perspective | Architectural, structural, MEP and landscape design, material, color selection and tree information | Generated directly from BIM model, which is tally with the construction model, minimize the possibility of argument since the inconsistency of market drawing and actual built area |
| --- | --- | --- | --- |
|  | 3D printing | Architectural, structural, MEP and landscape design, material, color selection and tree information | Generated directly from BIM model, which is tally with the construction model, minimize the possibility of argument since the inconsistency of market drawing and actual built area |
|  | Animation for interior and exterior | Architectural, structural, MEP and landscape design, material, color selection and tree information | Generated directly from BIM model, which is tally with the construction model, minimize the possibility of argument since the inconsistency of market drawing and actual built area |
|  | Color plan or isometric perspective | General arrangement plan, interior design including furniture, material and color selection | Generated directly from BIM model, which is tally with the construction model, minimize the possibility of argument since the inconsistency of market drawing and actual built area |
|  | Sales brochure | Leasable area and material selection | Generated directly from BIM model, which is tally with the construction model, minimize the possibility of argument since the inconsistency of market drawing and actual built area |
|  | Area schedule and unit price | Architectural design and unit price | Generated directly from BIM model, which is tally with the construction model, minimize the possibility of argument since the inconsistency of market drawing and actual built area |
|  | Facility | Architectural, structural, MEP and landscape design | Generated directly from BIM model, which is tally with the construction model, minimize the possibility of argument since the inconsistency of market drawing and actual built area |