LOIN SpecIfication

VERSION: 1.1 (Final)

July 2023

© The Government of the Hong Kong Special Administrative Region

The contents of this document remain the property of and may not be reproduced in

whole or in part without the express permission of the Government of the HKSAR

[Blank Page]

| Distribution |  |
| --- | --- |
| Copy No. | Holder |
| 1 | Departmental BIM Steering Committee (DBSC): HQs - AD(T) Survey Division - CLS GEO - CGE/LPM2 CEO - CE/PW EDO - CE/E3 SDO - CE/S3 WDO - CE/W4 NDO - CE/N3 SLO - CE/L1 Survey Division - SLS/BIM |
| 2 | Departmental BIM Working Group (DBWG): Survey Division - CLS GEO - SGE/D2 CEO - SE/P5 & SE/3 EDO - SE/23 SDO - SE/PM WDO - SE/8 NDO - SE/17 SLO - TS Survey Division - E/BIM |

| Amendment History |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Change Number | Revision Description | Pages Affected on Respective Version | Revision / Version Number | Date | Approval Reference |
| 1 | 1st Draft | All | 0.1 | 23.11.2021 |  |
| 2 | 2nd Draft – Circulation to BSWG | All | 0.2 | 30.11.2021 |  |
| 3 | 3rd Draft – Updated according to Feedback from BSWG | All | 0.3 | 24.12.2021 |  |
| 4 | 1st Final – Circulation to BSSG | - | 1.0 | 14.1.2022 |  |
| 5 | Updated according to new DEVB TC(W) requirements | All | 1.1 | xx.07.2023 |  |

- List of Abbreviation

Abbreviation	Full Expression

2D	Two Dimensional

3D	Three Dimensional

4D	Four Dimensional (i.e. Three Dimensional with Time)

5D	Five Dimensional (i.e. Three Dimensional with Time and Cost)

AM	Asset Management

AIR	Asset Information Requirements

AIM	Asset Information Model

BEP	BIM Project Execution Plan

BIM	Building Information Modelling / Building Information Model

BIM Manager	BIM Manager (and his team) appointed in the works

CAD	Computer Aid Design

CAT	Category Code for BIM matrix model elements

CDE	Common Data Environment

BIM CDCP	Common Data Collaboration Platform for BIM

CEDD	Civil Engineering and Development Department, HKSAR Government

CIC	Construction Industry Council, Hong Kong

CMMS	Computerized Maintenance Management System

COBie	Construction Operation Building Information Exchange

CSDI	Common Spatial Data Infrastructure

CSWP	CAD Standard for Works Projects

DEVB	Development Bureau, HKSAR Government

EIR	Exchange Information Requirements

GIS	Geographic Information System

IFC	Industry Foundation Classes

LandsD	Lands Department, HKSAR Government

LOD	Level of Development

LOD-G	Level of Graphics

LOD-I	Level of Information

LOIN	Level of Information Need

Project manager	Agency on behalf of the Employer

PIM	Project Information Model (Project BIM Model)

PIR	Project Information Requirements

Service Manager	Agency on behalf of the Employer

QA	Quality Assurance

QC	Quality Control

TC(W)	Technical Circular (Works)

WIP	Work In Progress

VR	Virtual Reality

TABLE OF CONTENTS

List of Abbreviation	1

1.	Introduction	1

1.1.	Purpose	1

1.2.	Structure	1

2.	LOIN Definition	2

2.1.	General	2

2.2.	LOD-G / LOD-Graphics	2

2.3.	LOD-I / LOD-Information	4

3.	Building Works Model	7

3.1.	Disciplinary Models	7

3.2.	Architectural Model	7

3.3.	Structural Model	32

3.4.	Building Services Model	43

4.	Civil Infrastructure Works Model	1

4.1.	Disciplinary Model	1

4.2.	Fender System Model	2

4.3.	Geotechnical Slope Model	14

4.4.	Geotechnical Structure Model	35

4.5.	Highways Structure Model	54

4.6.	Landscape Model	78

4.7.	Marine Civil Model	91

4.8.	Marine Structure Model	112

4.9.	Roadworks Model	142

4.10.	Sewerage Model	152

4.11.	Specialised Systems Model	158

4.12.	Stormwater Model	161

4.13.	Water Supplies Model	177

4.14.	Gas Supply Model	184

4.15.	Electrical Power Supply Model	188

4.16.	Telecommunication Model	193

5.	Existing Condition Model	196

5.1.	Disciplinary Models	196

5.2.	Existing Structure model	197

5.3.	Existing Site Topography model	221

5.4.	Existing Street Furniture model	233

5.5.	Existing Underground Utilities Models	281

Introduction

Purpose

LOIN is introduced in ISO19650 to communicate the amount of information needed to support the decision-making process of the appointing party. The generic definition of LOD-G and LOD-I may not accurately specify the requirement of each 3D Object elements and introduce miscommunication.

CEDD has identified the common object libraries required in the modelling manual. This Specification further communicate the detail geometrical and informational requirement of each object libraries to support the BIM implementation. The BIM Manager shall propose additional objects and their LOIN according to the particular specification of the works if necessary.

This document should be always read with the BIM Modelling Manual, the particular specifications / design brief, BEP, and the asset information requirements of the maintenance parties. This LOIN definition will not override the specific requirements in the works.

The BIM Manager shall refer to the deliverables and minimum requirement of LOIN in the particular specification of the works to propose the exact LOIN of each objects / additional objects in the BIM Project Execution Plan (BEP) for the approval of the Employer’s Agent / Project Manager. The BIM Manager shall refer to the BEP template of CEDD for further details.

This Specification layout the requirements of the Project Information Model (PIM) in CEDD. The BIM Manager shall review the requirement from maintenance parties in preparation of Asset Information Model (AIM), which may require specific attribute fields or different definition of object unity.

Structure

Section 2 recaps the LOIN definition used in CEDD.

Section 3 -5 specified the LOIN of common 3D Object elements used in CEDD according to CEDD’s matter federation grouping.

LOIN Definition

General

LOIN defines the information required to support each decision-making point throughout the works. LOIN includes geometrical (LOD-G), informational (LOD-I), documentations (DOC), which should be specified in the particular specification of the works and agreed in the BEP.

This generic definition of LOD-G and LOD-I applies to the new 3D Object elements that are not covered in this LOIN specification during the communication of requirement.

LOD-G / LOD-Graphics

LOD-G refers to the graphical representation which deals with geometric representation, symbology, and visualisation.

LOD-G 250 is introduced in CEDD for the purpose of modelling existing 3D Object elements. Some parts of a 3D Object element may not be accurately modelled in 3-Dimensions due to its as-built situation (such as underground portion of a structure). LOD-G is 250 is used when the overall size and physical properties of the entire 3D Object element is unable to be captured with the onsite surveying methodologies.  The definition of LOD-G is listed below:

| LOD-G | Description |
| --- | --- |
| 100 | The model element is graphically represented within the model by a 2D symbol or generic representation or rough 3D shape. It may be a 2D graphical symbol, centre lines, 3D polygons, or simple cuboid to indicate the overall size and approximated location of an object. |
| 200 | The model element is graphically represented within the model as a generic system, object or assembly with approximate quantities, assumed size, shape, location, and orientation. The assumed spaces required for access and maintenance shall be indicated. It is a generic/typical 3D object with most salient geometry being model to indicate the overall size, typical components and approximated location of an object. |
| 250 | The model element is graphically represented within the model as a generic system, object, or assembly. It is modelled with location and orientation derived from site survey information with approximate geometry detail (type, size, shape and dimensions). |
| 300 | The model element is graphically represented within the model as a specific system, object or assembly in terms of quantity, size, shape, location, and orientation. The model shall include details of the spaces required for handling installation, operation and maintenance, and the interface details for checking and coordinating with other models / objects. It is a specific 3D object with most salient geometry being model to indicate the overall size of all visible components and exact location of the object. |
| 400 | The model element is graphically represented within the model as a specific system, object or assembly in terms of quantity, size, shape, location, and orientation with detailing for fabrication, assembly, and installation. It is a specific 3D object with most salient geometry and connection details being model to support the construction and fabrication need of all visible components and exact location of the object. |
| 500 | Not used. Section 2.6.7 and 2.6.8 of CIC BIM Standards - General: Ver. 2.1, 2021 Edition for details. |

LOD-I / LOD-Information

LOD-I refers to the attribute information required to facilitate the modelling, production, analysis, sharing and collaboration of BIM models and deliverables.

In addition to the attribute required to generate deliverables for the works, the BIM Manager shall refer to the below table from the BIM Harmonisation Guidelines as the generic reference to create the attributes of different LOD-I.

| No. | Type | Attribute Name | Description | LOD-Information |  |  |  |  | Input Format |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 100 | 200 | 300 | 400 | 500 |  |
| 1 | Project Information | Organisation Name | Client name (per agreement/ contract) | M | M | M | M | M | Alphanumeric |
|  |  | Project Issue Date | Project Commencement date | M | M | M | M | M | MMM YYYY (eg. Nov 2014) |
|  |  | Project Issue Date | The street address of the project | M | M | M | M | M | Alphanumeric |
|  |  | Project Name | The project name as shown on the drawing sheet’s title block | M | M | M | M | M | Alphanumeric |
|  |  | Project Number | he project number as shown on the drawing sheet’s title block | M | M | M | M | M | Alphanumeric |
| 2 | General Properties | CAT Code | Departmental category | R | R | R | R | R | Alphanumeric |
|  |  | SUBCAT CODE | Sub CAT Code | R | R | R | R | R | Alphanumeric |
|  |  | Locations | Location (e.g. district code for outdoor object) |  | R | R | R | R | Alphanumeric |
|  |  | Departmental Unique ID (e.g. PWD Code in Port and Marine Services) | The unique ID for departmental information management |  | R | R | R | R | Alphanumeric |
|  |  | LOD-G | Definition value | R | R | R | R | R | Alphanumeric |
|  |  | LOD-I | Definition value | R | R | R | R | R | Alphanumeric |
| 3 | Design Properties | Material | Singular material or all materials pertaining to the assembly |  | R | R | R | R | Alphanumeric |
|  |  | Material Grade | Material grade (e.g. concrete grade, steel grade) |  | R | R | R | R | Alphanumeric |
|  |  | Design Capacity | Design capacity |  | R | R | R | R | Alphanumeric |
|  |  | Number | Room Number |  |  |  | R | R | Alphanumeric |
|  |  | Name | Room Name |  |  |  | R | R | Alphanumeric |
| 4 | Classification Properties | OmniClassCode | OmniClass code |  |  | R | R | R | Alphanumeric |
|  |  | OmniClassTitle | OmniClass title |  |  | R | R | R | Alphanumeric |
|  |  | OmniClassVersion | OmniClass version |  |  | R | R | R | Alphanumeric |
| 5 | Manufacturer’s Equipment Properties | Brand Name | Brand name |  |  |  | R | R | Alphanumeric |
|  |  | Manufacturer Name | Manufacturer name |  |  |  | R | R | Alphanumeric |
|  |  | Model Number of element / equipment | Model number |  |  |  | R | R | Alphanumeric |
|  |  | Equipment Capacity | Equipment capacity |  |  |  | R | R | Alphanumeric |
|  |  | Asset ID | Asset ID |  |  |  | R | R | Alphanumeric |
|  |  | Contract Number of the Equipment | The equipment’s contract number |  |  |  | R | R | Alphanumeric |
| 6 | Condition Properties | Certified Completion Date | Certified completion date |  |  |  | R | R | MMM YYYY (eg. Nov 2014) |
|  |  | Handover Date | Handover date |  |  |  | R | R | MMM YYYY (eg. Nov 2014) |
| 7 | Verification Property | Verification | Verification method (input A for "field verified by visual inspection" and B for "field verified by a measured survey") |  |  |  |  | R | Text (e.g. A or B) |

R: Required

The general properties and classification properties are mandatory attributes for all PIM and AIM submitted to CEDD. Yet, some attributes may not be available in a particular object by nature. The BIM Manager shall propose and agree the content and format of the attributes of each disciplinary model in the BEP and agree with the Employer’s Agent / Project Manager.

For all Asset Information Model submitted to Port and Marine Services, the BIM Manager shall refer to the Asset Information Requirement, Modelling Guideline and Standards of PWD (AIR of the PWD) to include the additional attributes required for the PWD’s asset. The disciplinary models of the Ports and Marine Services are listed in the section 3.1 of the BIM Modelling Manual of CEDD.

For all PIM and as-built model submitted to Geotechnical Engineering Services, the BIM Manager shall refer to the BIM project execution plan template of GEO (BEP of GEO) to include the additional attributes required for the GEO’s assets. The disciplinary models of the Geotechnical and Engineering Services are listed in the section 3.1 of the BIM Modelling Manual of CEDD.

The BIM Manager shall refer to the AIR or asset template from the maintenance departments to create the Asset Information Model / As-built Model submission. If an asset template is not available from the maintenance departments, the BIM Manager shall refer to LOD-I 400 to propose the asset template in the BEP.

All attribute fields should be filled in English Language, unless specified by CEDD

The BIM Manager shall refer to the Annex A of the BIM Modelling of CEDD for the technical requirement in the creation of attributes.

LOD-Doc / LOD-Document

LOD-Document refers to the manuals, specification, technical documents, test reports, drawings, etc required to develop a specific deliverable in the works to support decision making. A typical example of LOD-DOC includes the requirement maintenance manual in the as-built/asset information model to support asset maintenance purpose.

The BIM Manager shall refer to the particular specification, BEP and asset information requirement of the project office and maintenance parties in preparing the documents required in the PIM, As-built BIM model and AIM.

For all PIM and as-built model submitted to Geotechnical Engineering Services, the BIM Manager shall refer to the BIM project execution plan template of GEO (BEP of GEO) to include the documents required for the GEO’s assets. The disciplinary models of the Geotechnical and Engineering Services are listed in the section 3.1 of the BIM Modelling Manual of CEDD.

Building Works Model

Disciplinary Models

List of disciplinary models

Building Works Model include the 4 disciplinary models:

| Project Model | 1st Level Federation | 2nd level Federation (Disciplinary Models) | Description |
| --- | --- | --- | --- |
| Project Information Model | Proposed Building Model(s) | Architectural Model | Architectural elements of each building / plant room /etc in the service |
|  |  | Structural Model | Structure elements of each building / plant room /etc in the service |
|  |  | Building Services Model | MEP elements of each building / plant room /etc in the service |

Architectural Model

List of Common Objects Elements

The following object elements are classified under the Architectural Model:

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | Access Ladders and Catwalks | ALA | CAT | 23-17 23 15 13 | Vertical Ladders |
| 2 | Architectural Wall | TWL | Multiple* | 23-15 13 19 | Wall Panels |
| 3 | Blue Colour Paint | MAO | ABP | 23-15 21 11 13 | Textured Paints |
| 4 | Ceiling | CEL | ___ | 23-15 13 21 | Ceiling Panels |
| 5 | Curtain wall/ glass wall | CTP | Multiple* | 23-13 33 19 11 | Cladding and Curtainwall Panels |
| 6 | Door/Entrance | ADO | Multiple* | 23-13 33 21 | Entrances |
| 7 | Elevator / Lift Shaft space | MCO | SHF | 23-19 29 11 11 11 | Lift Shaft Components |
| 8 | Finishes | DTL | FNH | 23-13 00 00 | Structural and Exterior Enclosure Products |
| 9 | Floor, Slab | TLA | Multiple* | 23-15 17 13 | Floor covering strips, Tiles, Blocks and slabs |
| 10 | Furniture | FUR | Multiple* | 23-21 00 00 | Furnishing, Fixture and Equipment Products |
| 11 | Gate | AGT | Multiple* | 23-21 35 21 | Security and Vault Equipment and Furnishings |
| 12 | Louvers | WDW | LVR | 23-17 21 11 15 | Exterior Louvers and Grilles |
| 13 | Mass concrete fill | SFD | OTR | 23-13 15 11 | Concretes |
| 14 | Non-Slip Yellow Nosing | MAO | AYN | 23-15 17 11 15 15 | Slip Resistant floor Treatment |
| 15 | Precast Facade | TWL | TFC | 23-13 33 19 | Precast Concrete Façade |
| 16 | Railing, handrail | FRA | Multiple* | 23-17 25 11 | Guardrails |
| 17 | Ramp | TPD | ROS | 23-17 23 11 | Ramps |
| 18 | Roof / Architectural Roof | Multiple* | ROO | 23-13 39 11 | Exterior Roof Panels |
| 19 | Roof Gutter |  | AGT | 23-13 41 33 | Roof Gutters |
| 20 | Skylight | WDW | SKY | 23-17 17 00 | Skylights |
| 21 | Stairs | TTE | ___ | 23-17 23 17 | Stairs |
| 22 | Tactile Warning strip | FWS | Multiple* | 23-11 21 13 15 | Detectable Warning Surface for Roadways |
| 23 | Window | WDW | Multiple* | 23-17 13 00 | Windows |

Access Ladders and Catwalks

Cat ladder is often used to describe a ladder that is used for working on a sloping roof with a hook at one end and load spreading pads. These kinds of ladder are most commonly designed for use as access steps for maintenance to the roof of a building.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object | Assumed / Typical size of component(s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | Steps railings, posts, supports, post, sign plate, foundation, cat ladder, chord and bracing. | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | Object height must be limited by designed max height |
| 400 | Same as 300 | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Architectural/Non-structural Wall

An architectural wall is 3D Object element used to divide or enclose the building to form the periphery of a room or a building. An architectural wall does not support floor or roof load.

The finish should be separated from the wall 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall thickness (including the finish) | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | Wall thickness of the major components and their finish | Modelled from floor slab to soffit of beam or slab above | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element modelled according to the details of the manufacturer’s information, including wall finish detail including tiling, stone, cladding and screed. Openings modelled according to the requirements of the building services builders | Modelled from Joint to Join from floor slab to soffit of beam or slab above | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Schedules to be dynamically updated from the size of the Openings |

Blue Colour Paint

Blue Colour paint is the special painted finish used in ports and marine facilities for identification and navigation.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | Object with approximated shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located at typical depth of system | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact shape | Exact nominal dimension and approximate thickness | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | Same as 300 | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | 3D Object elements are modelled to support offsite fabrication and installation |

The attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

Ceiling

A Ceiling is the overhead surface or surfaces covering a room, and the underside of a floor or a roof. Ceilings are often used to hide floor and roof construction.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with typical depth of suspended ceiling system | Assumed / Typical size of component (s) | Object insertion point(s) approximately located at typical depth of system | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | Ceiling thickness, framing and major openings | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | Construction details objects in additional to 300 components, such as Framing, tees, hangers, support structures, ceiling tiles, openings and penetrations | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | 3D Object elements are modelled to support offsite fabrication and installation |

Curtain Wall/Glass wall

A curtain wall is defined as thin, usually aluminium-framed wall, containing in-fills of glass, metal panels, or thin stone. The framing is attached to the building structure and does not carry the floor or roof loads of the building.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with typical thickness as a single assembly | Assumed / Typical size of component | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | Mullions, transoms, insulations, air space, interior/exterior skins, shading devices, windows, louvers, doors and major openings on the curtain wall | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | Construction details objects in additional to 300 components, such as Façade brackets, embeds, fixings, cast-ins, secondary sub-frames | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | 3D Object elements are modelled to support offsite fabrication and installation |

Door/Entrance

A door is a movable barrier secured in an opening, known as the doorway, through a building wall or partition for the purpose of providing access to the inside of a building or rooms of a building. A door is held in position by doorframes, the members of which are located at the sides and top of the opening or doorway.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size, dimension in terms of location, count and type | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object elements with all components, including Door and Ironmongery (handles, locks, hinges, etc) | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element modelled according to the details of the manufacturer’s information, | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Elevator/Lift Shaft Space

Lift shaft refers to the enclosed space, reinforced by concrete, to support the movement of lifts/elevators.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | 3D boundary/region with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape | N/A |
| 300 | 3D Object element with exact spacing | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | Overall size and shape of each component | N/A |
| 400 | 3D Object element with specific requirement from the AIR of the maintenance parties | Same as 300 | Same as 300 | N/A | N/A |

Finishes

Finishes is used in the final part of the construction or manufacturing process, forming the final surface of an element. They can protect the element from impact, water, frost, corrosion, abrasion, and so on, and/or they can be decorative.

The finish should be separated from the parent 3D Object elements at LOD-G 300 or above.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | N/A | N/A | N/A | N/A | N/A |
| 250 | N/A | N/A | N/A | N/A | N/A |
| 300 | 3D Object element with exact shape | Exact nominal dimension of each component | Object insertion point(s) located and aligned exactly as parent 3D Object elements | 3D solids blocks rendered by Material texture according to the finish material | N/A |
| 400 | Same as 300 | Same as 300 | Same as 300 | Same as 300 | Patterns and layouts to be updated with the manufacturer’s information |

For the PWD projects, the attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

Floor, Slab

Floor slab refers to a floor that has been formed using concrete and may form part of the structure of a building.

The finish should be separated from the floor/slab at LOD-G 300 or above.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | 2D lines | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact thickness | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with exact thickness and reinforcement | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Furniture

Furniture refers to moveable objects that are used to support human activities in the built environment. It is more widely refer to fitted objects and equipment.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all components and parts | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with all components and parts according to manufacturer’s information | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Gate

Gate refers to a hinged barrier that can be used to secure an opening in a wall, fence and hedge.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object elements with all components, including Door and Ironmongery (handles, locks, hinges, etc) | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element modelled according to the details of the manufacturer’s information to support fabrication | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Louvers

Louver refers to narrow, sloping slats held in a frame and typically used to cover a vertical opening. They provide screening and can also prevent access, shade from the sun and provide protection against wind and rain.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all components and parts | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with all components and parts according to manufacturer’s information | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Mass concrete fill

Mass concrete fill refers to any volume of concrete with dimensions large enough to required that measures be taken to cope with the generation of heat from hydration of cement and attendant volume change to minimize cracking.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/2D polylines | N/A |
| 200 | Generic Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact thickness | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with exact thickness and reinforcement | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Non-slip Yellow Nosing

Non-slip yellow nosing is the anti-slip finish applied in the staircase and decking slab of Marine and port facilities.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with approximated shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located at typical depth of system | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object with all components and parts | Exact nominal dimension and thickness of all components | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with all components and parts according to manufacturer’s information | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

For the PWD projects, the attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

Precast Facade

Precast façade refers to façade that is prepared, cast and cured off-site. It is usually created in a controlled factory environment, using reusable moulds.

The finish should be separated from the parent facade elements at LOD-G 400 or above.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximated size | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all components and parts with all components, including nominal penetrations and Ironmongery (handles, locks, hinges, etc) | Exact nominal dimension and size of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with all components and parts [including penetrations for major openings, brackets, embeds sub-frame Ironmongery (handles, locks, hinges, etc)] according to manufacturer’s information | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Railing, handrail

Railing, handrail or guardrails refers to a fence or barrier made of rails to prevent people from falling off.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximated size | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all components and parts with all components, including railings, posts and support | Exact nominal dimension, size, length, height, and alignment of each component | Object insertion point(s) located and orientated exactly as nominal alignment and levels. | 3D solid block of each component | N/A |
| 400 | 3D Object element with all components and parts with all components, including railings, posts and support according to the details of the manufacturer’s information for fabrication | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Ramp

Ramp refers to sloped pathways used inside and outside buildings to provide access between vertical levels.

The finish should be separated from the parent ramp 3D Object elements at LOD-G 300 or above.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact thickness | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as designed. | 3D solid block of each component | N/A |
| 400 | 3D Object element with exact thickness and reinforcement | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Roof / Architectural Roof

Roof refers to structure forming the upper covering of a building or other shelter.

The finish should be separated from the parent roof 3D Object elements at LOD-G 300 or above.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all components and parts | Exact design dimension of each component | Object insertion point(s) located and orientated exactly as designed. | 3D solid block of each component | N/A |
| 400 | 3D Object element with all components and parts, including the reinforcement components | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

For the PWD projects, the attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

Roof Gutter

A gutter is a pipe or trough along the edge of a roof that carries rainwater away from a building. The purpose of the gutters on a roof is to prevent water damage to a structure or flooding around its foundation. A gutter directs the flow of rainwater away from the house, often into a drain or rain barrel.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | N/A | N/A | N/A | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all components and parts | Exact nominal dimension, alignment, longitudinal fall and invert levels of each section | Object insertion point(s) located and aligned exactly as nominal alignment and locations and invert levels | 3D solid block of each component | N/A |
| 400 | 3D Object element with all components and parts, separated according to the construction sequence | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

For the PWD projects, the attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

Skylight

Skylight refers to opening build into the roof of a building filled with glass.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all components and parts | Exact design dimension of each component | Object insertion point(s) located and orientated exactly as designed. | 3D solid block of each component | N/A |
| 400 | 3D Object element with all components and parts including frame elements and glazing, and Ironmongery according to the manufacturer’s information | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

For the PWD projects, the attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

Stairs

Stairs refer to a set of steps leading from one floor of a building to another, typically inside the building.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with threads, risers and landings | Approximate length, width, dimension and levels | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object elements with threads, risers, and landings to accurately indicate stringers and nosing | Exact nominal length, width, dimension and levels | Object insertion point(s) located and aligned exactly as nominal | 3D solid block of each component | N/A |
| 400 | 3D Object element with threads, risers, and landings to accurately indicate stringers and nosing and support fabrication and installation | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Tactile Warning Strip

“Tactile warning strip” means a standardized pattern applied to or built onto walking surfaces through the use of tactile hazard warning tiles / blocks to warn persons with visual impairment of certain construction features.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D object as a single assemble | Assumed/ Approximate dimension | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D object block | Exact nominal length, width, dimension, levels and alignment | Object insertion point(s) located and aligned exactly as nominal | 3D solid block of each component | N/A |
| 400 | Object block with details to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

For the PWD projects, the attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

Window

Window refers to a vented barrier provided in a wall opening to admit light and air into the structure and to give outside view.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Objects as a single assemble | Assumed / Typical size, dimension in terms of location, count and type | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with ironmongery | Exact design dimension of each component | Object insertion point(s) located and orientated exactly as designed. | 3D solid block of each component | N/A |
| 400 | 3D Object element with ironmongery, window frame elements, brackets, embeds, fixing, cast-in, modelled according to the details of the manufacturer’s information, | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Structural Model

List of Common Objects Elements

The following object elements are classified under the Structural Model:

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | Base Plates, Blots, clip angles, fixing etc. | SFM | BRA | 23-13 35 19 | Rafters, Beams, Joists |
| 2 | Beams | TBS | Multiple* | 23-13 35 11 13 13 | Beams |
| 3 | Columns | SCL | Multiple* | 23-13 35 11 13 11 | Columns |
| 4 | Foundation (Pile, pile cap, ground beams & Footings) | SFD | Multiple* | 23-13 29 00 | Foundations |
| 5 | Mass concrete fill | SFD | OTR | 23-13 15 11 | Concretes |
| 6 | Slabs | TLA | Multiple* | 23-15 17 13 19 | Precast tile and slab flooring |
| 7 | Ramp | TPD | ROS | 23-17 23 11 | Ramps |
| 8 | Stairs | TTE | ___ | 23-17 23 17 | Stairs |
| 9 | Structural Wall | TWL | Multiple* | 23-13 35 21 | Structural Walls |
| 10 | Tank structures | SFD | ___ | 23-27 29 13 | Tank Foundation |

The structure elements of PIU/PWD and GEO are not included in this Model.

Base Plates, Bolts, clip angles, fixing etc.

Base plate refers to a plate attached to the bottom of the column, which rests on a foundation or other support. Bolt refers to parts to anchor plates to a foundation or other support. Clip angle refers to a short piece of angle iron for connecting structural parts at angles. Fixing refers to parts that will hold things together or to attach them to the surfaces such as walls, floors, or roof.

The base plates, bolts, clip angles, fixing should be separately modelled from the parent steel structural element at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | N/A | N/A | N/A | N/A | N/A |
| 250 | N/A | N/A | N/A | N/A | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | 3D Object elements including base plates, blots, clips, angles, welds, coping, washers and nuts modelled according to the details of the manufacturer’s information, | Exact design dimension of each component | Object insertion point(s) located and orientated exactly as designed. | 3D solids of each component rendered by Material texture according to the finish material | N/A |

Beams

Beams refer to structural elements that resist loads applied laterally to their axis.

The finish should be modelled separately from the Beam 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid block | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid block | N/A |
| 300 | 3D Object element | Exact nominal size and dimension | Object insertion point(s) is located, orientated, and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | Responsive with structural grid and reference elevation plane |
| 400 | 3D Object element modelled with reinforcements, camber, chamfer, etc. | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Columns

Column refers to vertical structure intended to transfer a compressive load.

The finish should be separated from the Beam 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid block | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid block | N/A |
| 300 | 3D Object element | Exact nominal size and dimension | Located, orientated, and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | Responsive with structural grid and reference elevation plane |
| 400 | 3D Object element with reinforcements, camber, chamfer, etc. | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Foundation (Pile, pile cap, ground beams & Footings)

Pile foundation refers to a series of columns constructed or inserted into the ground to transmit loads to a lower level of subsoil. Ground beam refers to reinforce concrete beams for supporting walls and joists at or near ground level. Footing refers to foundation unit constructed in brick work, stone masonry or concrete under the base of a wall column for the purpose of distributing the load over a large area.

The finish should be separated from the Beam 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/2D lines | N/A |
| 200 | 3D Object with all foundation components | Assumed / Typical size of component (s) | Located, orientated, and sloped as nominal and/or structural grid and reference elevation plane | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all foundation components | Exact nominal size, dimension, and cut-off depth | Located, orientated, and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | Responsive with structural grid and reference elevation plane |
| 400 | 3D Object element with all foundation components, including reinforcements, chamfer, etc. | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by material texture according to the finish material | N/A |

Mass concrete fill

Mass concrete fill refers to any volume of concrete with dimensions large enough to required that measures be taken to cope with the generation of heat from hydration of cement and attendant volume change to minimize cracking.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/2D lines | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact thickness | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with exact thickness and reinforcement | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Slab

Slab refers concrete structural element. It is used to create flat horizontal surfaces such as floors, roof decks and ceilings.

The finish should be separated from the floor/slab at LOD-G 300 or above.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/2D lines | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact thickness | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with exact thickness and reinforcement | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Ramp

Ramp refers to sloped pathways used inside and outside buildings to provide access between vertical levels.

The finish should be separated from the parent ramp 3D Object elements at LOD-G 300 or above.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/2D lines | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact thickness | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as designed. | 3D solid block of each component | N/A |
| 400 | 3D Object element with exact thickness and reinforcement | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Stairs

Stairs refer to a set of steps leading from one floor of a building to another, typically inside the building.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | Symbols/2D lines | N/A |
| 200 | 3D Object with threads, risers, and landings | Approximate length, width, dimension, and levels | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with threads, risers, and landings to accurately indicate stringers and nosing | Exact nominal length, width, dimension and levels | Object insertion point(s) located and aligned exactly as nominal | 3D solid block of each component | N/A |
| 400 | 3D Object element with threads, risers, and landings to accurately indicate stringers and nosing and support fabrication and installation | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Structural Wall

Structure wall refers to a load bearing wall or a wall that carries load in addition to its own load.

The finish should be separated from the wall 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall thickness (including the finish) | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | Wall thickness of the major components and their finish | Modelled from floor slab to soffit of beam or slab above | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element including reinforcement, wall finish detail including tiling, stone, cladding and screed. Openings modelled according to the requirements of the building services builders | Modelled from Joint to Join from floor slab to soffit of beam or slab above | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Schedules to be dynamically updated from the size of the Openings |

Tank structures

Tank structure refers to create a tank-link seal to protect walls against water penetration.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/2D lines | N/A |
| 200 | 3D Object with all foundation components | Assumed / Typical size of component (s) | Located, orientated, and sloped as nominal and/or structural grid and reference elevation plane | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all foundation components | Exact nominal size, dimension and cut-off depth | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | N/A |
| 400 | 3D Object element modelled with all foundation components, including reinforcements, chamfer, etc. | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Building Services Model

List of Common Objects Elements

The following object elements are classified under the Building Services Model:

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | MVAC System |  |  |  |  |
| 1a | Exhaust (extract) air duct | BDU | ___ | 23-33 49 13 | Ventilation Ducts |
| 1b | Fresh air duct | BDU | ___ | 23-33 49 13 | Ventilation Ducts |
| 1c | Return air duct | BDU | ___ | 23-33 49 13 | Ventilation Ducts |
| 1d | Supply air duct | BDU | ___ | 23-33 49 13 | Ventilation Ducts |
| 1e | Access panel | AAP | ___ | 23-33 49 15 | Duct Access Panels |
| 1f | Air handling unit | BAC | ___ | 23-33 25 00 | Air Handling Units |
| 1g | Chillers | MCQ | CHR | 23-33 21 13 | Chillers |
| 1h | Chilled water supply pipe | MCQ | CHL | 23-33 00 00 | HVAC Specific Products and Equipment |
| 1i | Chilled water return pipe | MCQ | CHL | 23-33 00 00 | HVAC Specific Products and Equipment |
| 1j | Condensate drainpipe | MPI | PVC | 23-33 00 00 | HVAC Specific Products and Equipment |
| 1k | Damper | BDA | Multiple* | 23-33 29 00 | HVAC Dampers |
| 1l | Diffuser, air-boot, air grill, air filter, register | BDI | EXH | 23-33 49 11 | Ventilation Diffusers |
| 1m | Fan | BFA | Multiple* | 23-33 31 19 | Fans |
| 1n | Fan Coil Unit | BFC | ___ | 23-33 33 11 | Fan Coil Unit |
| 1o | Fire damper | BDA | FIR | 23-33 29 23 | Fire Dampers |
| 1p | Insulation | PIS | NA | 23-33 49 17 | Duct Insulation |
| 1q | Primary air unit | BAC | ___ | 23-33 39 00 | Air Conditioning Equipment |
| 1r | Silencer | BSI | ___ | 23-33 00 00 | HVAC Specific Products and Equipment |
| 1s | Smoke extraction system | MCQ | VCS | 23-39 27 11 | Firs Fighting Smoke Extractors |
| 1t | Variable control damper | BDA | VOC | 23-33 29 37 | Volume Control Dampers |
| 1u | Dynamic envelope in MVAC model | DNE | ___ | 23-33 00 00 | HVAC Specific Products and Equipment |
| 2 | Plumbing System |  |  |  |  |
| 2a | Flush water piping | PLM | DRN | 23-31 35 00 | Plumbing Tubing |
| 2b | Fresh water piping (water supplies) | WSD | ___ | 23-31 35 00 | Plumbing Tubing |
| 2c | Tap, faucet | PLM | TAP | 23-31 11 00 | Faucets |
| 2d | Valve | MVA | Multiple* | 23-31 00 00 | Plumbing Specific Products and Equipment |
| 2e | Dynamic envelope in plumbing & water supply model | DNE | ___ | 23-31 00 00 | Plumbing Specific Products and Equipment |
| 3 | Drainage and Sewerage |  |  |  |  |
| 3a | Floor drain | DTH | ODR | 23-31 27 00 | Floor Drains |
| 3b | Gully, sealed trapped gully, clean outs and vent | GUL | Multiple* | 23-31 27 15 | Floor Drains cover |
| 3c | Kitchen waste pipe work including floor drain, open trapped | MPA | Multiple* | 23-31 35 00 | Plumbing Tubing |
| 3d | Rainwater, storm water pipe, storm drain, drainage downpipe | STP | ___ | 23-13 41 39 19 | Siphonic Roof Drains |
| 3e | Rainwater outlet | SNF | ___ | 23-13 41 39 11 | Roof Downspouts |
| 3f | Surface channel, slot channel, external drainage | SUP | DSC | 23-13 41 39 17 | Eavestroughs |
| 3g | Sewerage pipe, foul sewer drains | FWD | CSU | 23-27 39 00 | Piping |
| 3h | Vent pipe | MPI | PVC | 23-27 39 00 | Piping |
| 3i | Dynamic envelope in drainage & sewage model | DNE | ___ | 23-31 00 00 | Plumbing Specific Products and Equipment |
| 4 | Fire Services System |  |  |  |  |
| 4a | Fire Alarm Control Panel | BFP | Multiple* | 23-29 31 13 | Fire Alarm Control Panel |
| 4b | Fire Alarm Audio/ Visual | BFS | Multiple* | 23-29 31 19 | Combination Audible and Visible Fire Notification Devices |
| 4c | Fire Alarm Devices | ALM | Multiple* | 23-29 31 00 | Fire Notification Appliances |
| 4d | Break glass unit | BFB | ___ | 23-29 29 11 | Fire Alarm Pull stations |
| 4e | Fire detection system, heat or smoke detectors | BFD | Multiple* | 23-29 29 13 | Fire Detectors |
| 4f | Fire hydrant/hose reel system | BFH | ___ | 23-29 25 00 | Fire Fighting Equipment |
| 4g | Sprinkler pipe work | SRK | ___ | 23-29 33 00 | Fire suppression System Components |
| 4h | Sprinkler head | SRK | ___ | 23-29 33 00 | Fire suppression System Components |
| 4i | Sprinkler valve & flow switch | SRK | ___ | 23-29 33 00 | Fire suppression System Components |
| 4j | Dynamic envelope fire services model | DNE | ___ | 23-29 33 00 | Fire suppression System Components |
| 5 | Electrical System / MEP System for Marine and Ports |  |  |  |  |
| 5a | Cable Draw pit | ECD | Multiple* | 23-17 11 27 23 | Manhole Accesses |
| 5b | CCTV Camera | BCA | Multiple* | 23-29 11 11 13 | Visual Light wavelength CCTV Camera |
| 5c | Corrosion Monitoring Pit | CSE | Multiple* | 23-17 11 27 23 | Manhole Accesses |
| 5d | Corrosion Monitoring Terminal Box | CSE | Multiple* | 23-17 11 27 23 | Manhole Accesses |
| 5e | Directional Exit Sign | BFX | ___ | 23-35 47 15 | Exit Illuminated Signs |
| 5f | Earthing and Lightning equipment | Multiple* | ___ | 23-35 39 00 | Electric Power protection Devices |
| 5g | Electrical Cable tray, cable containment, power feed, cable ducting | CTF | Multiple* | 23-35 33 19 | Electrical Cable Trays |
| 5h | Emergency lighting | BLF | ___ | 23-35 47 13 | Emergency Lighting |
| 5i | Exit sign | BFX | ___ | 23-35 47 15 | Exit Illuminated Signs |
| 5j | GovWifi equipment | CSE | Multiple* | 23-37 13 17 | Wide Area Network Devices |
| 5k | ICCP Transformer Rectifier | ETR | Multiple* | 23-80 50 00 | Terminals for Power supply |
| 5l | Inspection manhole | UMH | ___ | 23-17 11 27 23 | Manhole Accesses |
| 5m | Light fitting/Lighting | BLF | Multiple* | 23-34 47 11 | Lighting Fixtures |
| 5n | Panel board, motor control center | ECP | ___ | 23-35 31 13 | Distribution Panel Boards |
| 5o | Pillar box (including ICCP AC power supply pillar box and electric pillar box) | ETR | Multiple* | 23-80 50 00 | Terminals for Power supply |
| 5p | Sensors | CSE | Multiple* | 23-27 11 00 | General Instrument |
| 5q | Solar Panel | CSE | SOP | 23-75 10 27 17 | Solar Collector Units |
| 5r | Trucking, bus duct, busbar, busway | CTF | CHN | 23-35 33 21 | Electrical Bus ducts |
| 5s | Dynamic envelope electrical model | DNE | ___ | 23-35 47 00 | Electrical Lighting |

Mechanical Model includes all the 3D Object elements designed for special plants / facilities in proposed in the buildings/facilities. It is applicable to all Environmental and Sustainability services. The BIM Manager should propose the 3D Object elements in the BEP accordingly.

| ITEM | 3D OBJECT ELEMENT | CAT CODE | SUBCAT CODE* | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |

MVAC air ducts

MVAC Air duct includes all air ducts or pipes for conveying air.

This section describes the requirements for exhaust air duct, fresh air duct, return air duct and supply air duct under the Building Services Model.

Dynamic envelope should be always modelled to indicate required occupation/working space.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including insulations with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The ductworks should be modelled as connect systems |
| 400 | 3D Object element with components modelled, including ducts, insulations, flanges, hangers according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

MVAC Access panel

Access panel refers to an opening that can be easily removed to provide access to MVAC system.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | 3D Object element, including maintenance space envelop with exact size and dimension | Size and shape requirement according to support the fabrication requirement. | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |

MVAC Air handling unit

Air handling unit refers to encased assembly consist of a fan or fans and other necessary equipment.

The BIM shall refer to the AIR of the maintenance parties in developing the LOD-G 400 model.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3DObject with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with outer form and shape. Dynamic envelope to indicate approximated maintenance spaces | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The unit should be connected to the duct system |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

MVAC Chillers

Chiller refers to machine that remove heat from a liquid coolant via vapour-compression, adsorption refrigeration or absorption refrigeration cycles.

The BIM shall refer to the AIR of the maintenance parties in developing the LOD-G 400 model.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with outer form and shape, including water inlet/outlet, condense water inlet/outlet, anti-vibration footing, power provision, compressor, evaporator; and dynamic envelope to indicate approximated maintenance spaces | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The unit should be connected to the duct system |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

MVAC Chilled water supply pipe

Chilled water supply pipe refers to pipes carry chilled water around a building.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including insulations with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The pipes should be modelled as connect systems |
| 400 | 3D Object elements with components modelled, including pipes and insulations according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

MVAC Chilled water return pipe

Chilled water return pipe refers to pipes carry water return to the chiller.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including insulations with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The pipes should be modelled as connect systems |
| 400 | 3D Objects with components modelled, including pipes and insulations according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

MVAC Condensate drainpipe

Condensate drainpipes refer to pipes carrying moisture as to the exit through the evaporator coil.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including insulations with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The pipes should be modelled as connect systems |
| 400 | 3D Object element with components modelled, including pipes and insulations according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

MVAC Damper

A damper is a valve or plate that stop or regulates the flow of air inside the air handling unit.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including insulations with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The damper should be connected with the parent systems |
| 400 | 3D Object element with components modelled, including dampers, insulations and hangers according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

MVAC Diffuser, air-boot, air grill, air filter, register

Diffuser refers to air distribution outlet, usually located in the ceiling and consisting of deflecting vanes discharging supply air in various directions and planes and arranged to promote mixing of the air which is supplied to the room with the air already in the room.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with size and dimension of each component | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The object should be modelled as connect systems |
| 400 | 3D Object element with components modelled according to the details of the manufacturer’s information to support fabrication and installation. | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

MVAC Fan

Fan refers to a mechanical device for moving air or other gases in a direction at an angle to the incoming fluid.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with outer form and shape, housing, impellers, and casing. Dynamic envelope to indicate approximated maintenance spaces | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The unit should be connected to the duct system |
| 400 | 3D object elements with components modelled to support the fabrication requirement. | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

MVAC Fan Coil Unit

Fan Coil Unit refers to a device that uses a coil and a fan heat or cool a room without connectin to ductwork.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with outer form and shape, water inlet/outlet, power provision, dip tray, connections for condensation drain. Dynamic envelope to indicate approximated maintenance spaces | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The unit should be connected to the duct system |
| 400 | 3D object elements with components modelled to support the fabrication requirement. | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

MVAC Fire Damper

Fire damper refers to device designed to impede the spread of fire through walls, floors and partitions.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including insulations with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The damper should be connected with the parent systems |
| 400 | 3D Object element with components modelled, including dampers, insulations and hangers according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

MVAC Insulation

Insulation refers to a thick layer of substance that keeps the temperature constant.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | Aligned and updated with the parent pipes/ducts |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with overall size and dimension. | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | Aligned and updated with the parent pipes/ducts |
| 400 | N/A | N/A | N/A | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

MVAC Primary air unit

Primary air unit refers to machine used to transfer and modify the air in a structure as part of HVAC system.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with outer form and shape. Dynamic envelope to indicate approximated maintenance spaces | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The unit should be connected to the duct/pipe system |
| 400 | 3D object elements with components modelled to support the fabrication requirement. | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

MVAC Silencer

Mechanism capable of reducing the sound pressure level between its inlet and its outlet.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with outer form and shape. Dynamic envelope to indicate approximated maintenance spaces | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The unit should be connected to the duct/pipe system |
| 400 | 3D object elements with components modelled to support the fabrication requirement. | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

MVAC Smoke extraction system

Smoke extraction system refers to exhaust ducts, which remove heat and smoke through grilles located on the ventilation ducts.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including insulations with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The ductworks should be modelled as connect systems |
| 400 | 3D Object element with components modelled, including ducts, insulations, flanges, hangers according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

MVAC Variable control damper

Variable control damper refers to system enable energy-efficient system distribution by optimising the amount and temperature of distributed air.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with outer form and shape. Dynamic envelope to indicate approximated maintenance spaces | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The unit should be connected to the duct/pipe system |
| 400 | 3D object elements with components modelled to support the fabrication requirement. | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Dynamic envelope in MVAC model

Dynamic envelope refers to an area near the MVAC model to prevent access.

The geometry of the envelope shall always align with the LOD-G requirement of the parent object.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic layout | Assumed size | Symbol insertion point(s) approximately located | 2D polygons | N/A |
| 200 | 3D Object with approximate size/dimension aligned with the parent object | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | Aligned with the parent object |
| 250 | N/A | N/A | N/A | N/A | N/A |
| 300 | 3D Object element with exact size/dimension aligned with the parent object | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block for each parent component | Aligned with the parent object |
| 400 | N/A | N/A | N/A | N/A | N/A |

Plumbing Flush water piping

Plumbing flush water piping refers to pipe that carries flush water in a system.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including pipes, all fittings and accessory with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal location, invert levels, and longitudinal fall. | 3D solid block of each component | The pipes should be modelled as connect systems |
| 400 | 3D Object element with components modelled, including pipes, all fittings and accessory according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces if applicable | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Plumbing Fresh water piping

Plumbing flush water piping refers to pipe that carries fresh water in a system.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including pipes, all fittings and accessory with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal location, invert levels, and longitudinal fall. | 3D solid block of each component | The pipes should be modelled as connect systems |
| 400 | 3D Object element with components modelled, including pipes, all fittings and accessory according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces if applicable | Size and shape requirement according to support the fabrication requirement. | Same as 300 | Rendered by Material texture according to the finish material | Same as 300 |

Plumbing Tab, faucet

A faucet is a device for delivering water from a plumbing system. It can consist of the following components: spout, handle(s), lift rod, cartridge, aerator, mixing chamber, and water inlets. When the handle is turned on, the valve opens and controls the water flow adjustment under any water or temperature condition.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including spout, handle(s), lift rod, cartridge, aerator, mixing chamber, and water inlets with outer form and shape. Dynamic envelope to indicate approximated maintenance spaces | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The unit should be connected to the pipe system |
| 400 | 3D object elements with components modelled to support the fabrication requirement. | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Dynamic envelope in plumbing & water supply model

Dynamic envelope refers to an area near the plumbing and water supply model to prevent access.

The geometry of the envelope shall always align with the LOD-G requirement of the parent object.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic layout | Assumed size | Symbol insertion point(s) approximately located | 2D polygons | N/A |
| 200 | 3D Object with approximate size/dimension aligned with the parent object | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | Aligned with the parent object |
| 250 | N/A | N/A | N/A | N/A | N/A |
| 300 | 3D Object element with exact size/dimension aligned with the parent object | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block for each parent component | Aligned with the parent object |
| 400 | N/A | N/A | N/A | N/A | N/A |

Floor drain

Floor drain refers to plumbing fixture that is installed in the floor of the structure, mainly designed to remove any standing water nearby.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including grate, flashing, outlet and trap with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The floor drain should be connected with the parent waste water systems |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Gully, sealed trapped gully, clean outs and vent

Gully refers to a drainage fitting which has an open top, a definite base, and an outlet to one or more side of the body of the gully.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including grate, hopper, plot, water trap and outlet with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The object should be connected with the parent waste water systems |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Kitchen waste pipe work including floor drain, open trapped

Kitchen waste pipe work refers to piping system connects pipes in kitchen to gully.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including sink, fittings, trap and outlet with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The object should be connected with the parent wastewater systems |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Rainwater, storm water pipe, storm drain

Rainwater pipe refers to pipes and drains that is used to direct rainwater away from a building.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including pipes, all fittings and accessory with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal location, invert levels, and longitudinal fall. | 3D solid block of each component | The pipes should be modelled as connect systems |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Rainwater outlet

Rainwater outlets are designed to provide effective drainage of rainwater on flat roofs.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including grate and downspouts with outer form and shape. Dynamic envelope to indicate approximated maintenance spaces | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The unit should be connected to the rainwater pipe system |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Surface channel, slot channel, external drainage

A rain gutter, eavestrough, eaves-shoot or surface water collection channel is a component of a water discharge system for a building.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including trough, endcap, and downspout with outer form and shape. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The object should be connected as a system |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Sewerage pipe, foul sewer drains

Sewage pipe and foul sewer drain refer pipes used to carry wastewater to a sewage works for treatment in a building/facility.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including waste pipes, soil pipe, trap, all fittings and accessory with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal location, invert levels, and longitudinal fall. | 3D solid block of each component | The pipes should be modelled as connected systems |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Vent pipe

Vent pipe refers to pipes above a waste pipe or soil pipe that allows gas to escape from the system.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including pipes, all fittings and accessory with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal location, invert levels, and longitudinal fall. | 3D solid block of each component | The pipes should be modelled as connected systems |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Dynamic envelope in drainage & sewage model

Dynamic envelope refers to an area near the drainage and sewage model to prevent access.

The geometry of the envelope shall always align with the LOD-G requirement of the parent object.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic layout | Assumed size | Symbol insertion point(s) approximately located | 2D polygons | N/A |
| 200 | 3D Object with approximate size/dimension aligned with the parent object | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | Aligned with the parent object |
| 250 | N/A | N/A | N/A | N/A | N/A |
| 300 | 3D Object element with exact size/dimension aligned with the parent object | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block for each parent component | Aligned with the parent object |
| 400 | N/A | N/A | N/A | N/A | N/A |

Fire Alarm Control Panel

Fire alarm control panel refers to the device which all other devices of the fire alarm system is connected.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with outer form and shape. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The unit should be connected to the fire system |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Fire Alarm Audio/Visual

Fire alarm audio/visual refers to sound or visual by the fire alarm device during an emergency.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with outer form and shape. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The unit should be connected to the fire system |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Fire Alarm Devices

Fire alarm device refers to the device which will give a sound/visual warning.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with outer form and shape. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The unit should be connected to the fire system |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Break glass unit

Break glass unit refers to device with a breakable glass to set off the fire alarm in an emergency.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with outer form and shape. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The unit should be connected to the fire system |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Fire detection system, heat or smoke detectors

Fire detection system and detectors refers to device that will detect fire, heat or smoke.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with outer form and shape. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The unit should be connected to the fire system |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Fire hydrant/hose reel system

Fire hydrant/hose reel system refers to a fitting in public place of the building with a nozzle by which a fire hose may be attached to a water main.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including water inlet/outlet, wheel size, front/back panel, stop cock valve, fire hose and pipe with outer form and shape. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The unit should be connected as connected system |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Sprinkler pipe work

A fire sprinkler system is an active fire protection method, consisting of a water supply system, providing adequate pressure and flowrate to a water distribution piping system, onto which fire sprinklers are connected.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including pipes, all fittings and accessory with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal location, invert levels, and longitudinal fall. | 3D solid block of each component | The pipes should be modelled as connect systems |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Sprinkler head

A fire sprinkler or sprinkler head is the component of a fire sprinkler system that discharges water when the effects of a fire have been detected, such as when a predetermined temperature has been exceeded.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including the thread, plug, frame, glass blub and detector with outer form and shape. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The unit should be connected to the fire system |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Sprinkler valve & flow switch

Control valves control the flow of water in an automatic fire sprinkler system.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including the all gauges, water inlet/outlet, all flow switch/control, monitor valve, test drain, panel, retard chamber, alarm with outer form and shape. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The unit should be connected to the fire system |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Dynamic envelope fire services model

Dynamic envelope refers to an area near the fire services model to prevent access.

The geometry of the envelope shall always align with the LOD-G requirement of the parent object.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic layout | Assumed size | Symbol insertion point(s) approximately located | 2D polygons | N/A |
| 200 | 3D Object with approximate size/dimension aligned with the parent object | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | Aligned with the parent object |
| 250 | N/A | N/A | N/A | N/A | N/A |
| 300 | 3D Object element with exact size/dimension aligned with the parent object | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block for each parent component | Aligned with the parent object |
| 400 | N/A | N/A | N/A | N/A | N/A |

Cable Draw Pit

A Drawpit is the manhole of the ducting world. They are chambers situated along a line of underground ducting and facilitate access to the ducting to allow for initial installation, maintenance and monitoring.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including wall, slab, frame and cover, location of equipment, routing and size of cable trucking with exact size and dimension | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element including wall, slab, frame and cover, location of equipment, containments, risers, switch board, routing and size of cable trucking. | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | Rendered by Material texture according to the finish material | N/A |

For PWD projects, the attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

CCTV Camera

Closed-circuit television, also known as video surveillance, is the use of video cameras to transmit a signal to a specific place, on a limited set of monitors.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including the housing and installation support of with exact size and dimension | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

The attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

Corrosion Monitoring pit

A Corrosion Monitoring pit is the manhole/chambers situated in marine facilitate access to allow for initial installation, maintenance and monitoring of corrosion monitoring equipment.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including wall, slab, frame and cover, location of equipment, with exact size and dimension | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

The attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

Directional Exit Sign

An exit sign is a pictogram or short text in a public facility (such as a building, aircraft, or boat) denoting the location of the closest emergency exit to be used in case of fire or other emergency that requires rapid evacuation.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object outline with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including housing, supports, text and symbol with exact size and dimension | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

The attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

Electrical cable tray, cable containment, power feed, cable duct

Cable trays are mechanical support systems that provide a rigid structural system for electrical cables, raceways, and insulated conductors used for electric power distribution, control, signal instrumentation, and communication.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including cable channel/tray/ladder, supports, fittings and connecters. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object elements with components modelled, including cable channel/tray/ladder, supports, fittings and connecters according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Earthing and Lightning equipment

A system including the metal rod mounted on a structure and earthing system used to protect the structure from a lightning strike.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object outline with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including lightning rod, mounting base, cable strap, ground rod and earthing equipment with exact size and dimension | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

The attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

Emergency lighting

An emergency light is a battery-backed lighting device that switches on automatically when a building experiences a power outage.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object outline with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including housing, supports, text and symbol with exact size and dimension | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

The attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

Exit Sign

An exit sign is a pictogram or short text in a public facility (such as a building, aircraft, or boat) denoting the location of the closest emergency exit to be used in case of fire or other emergency that requires rapid evacuation.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object outline with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including housing, supports, text and symbol with exact size and dimension | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

The attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

GovWiFi equipment

The GovWifi equipment is commonly used for local area networking of devices and Internet access supplied by the HK Government, allowing nearby digital devices to exchange data by radio waves.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object outline with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including switches and power supplies with outer size and dimension | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

The attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

ICCP Transformer Rectifier

Cathodic protection transformer rectifier units (also known as TR or TRU) are used as power supply component of Impressed Current Cathodic Protection (ICCP) systems. This unit is purpose-built and generally featured remote control monitoring capabilities and current interrupters.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object outline with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including rectifier, test meter, electrolyte with outer size and dimension | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

The attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

Inspection Manhole

A manhole is an opening to a confined space such as a shaft, utility vault, or large vessel. Manholes are often used as an access point for an underground public utility, allowing inspection, maintenance, and system upgrades.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including wall, slab, frame and cover with exact size and dimension | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

The attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

Lighting fitting/lighting

A light fixture, light fitting, or luminaire is an electrical device containing an electric lamp that provides illumination.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including housing, lights/lamp, cover with exact size and dimension | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

The attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

Panel board, motor control center

A panel board is a component of an electrical distribution system which divides an electrical power feed into branch circuits, while providing a protective circuit breaker or fuse for each circuit, in a common enclosure.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including the housing and switches with exact size and dimension | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

The attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

Pillar box

Cathodic protection transformer rectifier units (also known as TR or TRU) are used as power supply component of Impressed Current Cathodic Protection (ICCP) systems. This unit is purpose-built and generally featured remote control monitoring capabilities and current interrupters.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object outline with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including the housing with outer size and dimension | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

The attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

Sensors

A sensor is a device, module, machine, or subsystem whose purpose is to detect events or changes in its environment and send the information to other electronics, frequently a computer processor.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with outer size and dimension | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

The attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

Solar Panel

A solar cell panel, solar electric panel, photo-voltaic module or just solar panel is an assembly of photo-voltaic cells mounted in a framework for installation. Solar panels use sunlight as a source of energy to generate direct current electricity.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with outer size and dimension | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

The attributes specified in the AIR of the PWD should be included as LOD-I 300, 400 & 500 requirement

Trucking, bus duct, busbar, busway

Compact power distribution systems that rely on copper or aluminium busbars to channel electricity, while being enclosed in an aluminium housing for protection.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including bus duct, cable tray, riser, end cover, elbow, switch board, hanger and supports. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object elements with components modelled, including bus duct, cable tray, riser, end cover, elbow, switch board, hanger and supports according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Dynamic envelope electrical model

Dynamic envelope refers to an area near the electrical model to prevent access.

The geometry of the envelope shall always align with the LOD-G requirement of the parent object.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic layout | Assumed size | Symbol insertion point(s) approximately located | 2D polygons | N/A |
| 200 | 3D Object with approximate size/dimension aligned with the parent object | Assumed size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | Aligned with the parent object |
| 250 | N/A | N/A | N/A | N/A | N/A |
| 300 | 3D Object element with exact size/dimension aligned with the parent object | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block for each parent component | Aligned with the parent object |
| 400 | N/A | N/A | N/A | N/A | N/A |

Civil Infrastructure Works Model

Disciplinary Model

List of disciplinary models

Proposed Works Model include the 15 disciplinary models:

| Project Model | 1st Level Federation | 2nd level Federation (Disciplinary Models) | Description |
| --- | --- | --- | --- |
| Project Information Model | Proposed Works Model | Fender System Model | Fender systems and related object elements in port and marine facilities |
|  |  | Geotechnical Model | Site boundary, Geological elements, Slope works and related stabilisation system |
|  |  | Geotechnical Structure Model | Concrete structures, foundations and access elements in slope works |
|  |  | Highway Structure Model | Underground structure for foundation supports |
|  |  |  | Above ground structure of highways |
|  |  |  | Other structure such as Tunnel linings, portals, etc |
|  |  | Landscape Model | Outdoor landscape systems including planting, parks, recreation facilities, etc |
|  |  | Marine Civil Model | Furnitures within marine facilities, including site boundary, proposed topologies and bathymetry |
|  |  | Marine Structure Model | Structural elements in port and marine facilities |
|  |  | Roadworks Model | Road works systems including roadworks and related dwarf wall, retaining structure, slope works and site boundary |
|  |  | Sewerage Model | Proposed/Rediverted Sewerage network of the Service |
|  |  | Specialised Systems Model | Tunnel Ventilation system |
|  |  |  | Tunnel Fire Services System |
|  |  |  | Traffic Control and Surveillance System |
|  |  | Stormwater Model | Proposed/Rediverted Stormwater network of the Service |
|  |  | Water Supplies Model | Proposed/Rediverted freshwater supplies network of the Service |
|  |  | Gas Supply Model | Proposed/Rediverted Gas Supply system of the Services |
|  |  | Electrical Power Supply Model | Proposed/Rediverted Power supply system of the Services |
|  |  | Telecommunications Model | Proposed/Rediverted Telecommunication system of the Services |

Fender System Model

Summary of elements

The Fender Model includes all the object elements in the fendering system proposed in the works for the Port and Marine services. The following object elements should be included:

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | Capping (Rubber or Timber) | FEN | FCA (PWD Code) | 23-39 21 11 11 13 | Boat Fenders |
| 2 | Chain for Fender System | FEN | FCH | 23-39 21 11 11 13 | Boat Fenders |
| 3 | Eye blot for Fender System | FEN | FEB | 23-39 21 11 11 13 | Boat Fenders |
| 4 | Frontal Pad | FEN | FFP | 23-39 21 11 11 13 | Boat Fenders |
| 5 | Horizontal Fender (plastic, timber, rubber) | FEN | FHF | 23-39 21 11 11 13 | Boat Fenders |
| 6 | Horizontal/Inclined fender wailing | FEN | FVF/FIW | 23-39 21 11 11 13 | Boat Fenders |
| 7 | Rubber Buffer | FEN | FRB | 23-39 21 11 11 13 | Boat Fenders |
| 8 | Step Block (Rubber or Timber) | FEN | FSB | 23-39 21 11 11 13 | Boat Fenders |
| 9 | Steel Bracket | FEN | FBK | 23-39 21 11 11 13 | Boat Fenders |
| 10 | Steel Plate for Wailing system | FEN | FSP | 23-39 21 11 11 13 | Boat Fenders |
| 11 | Vertical Fender (plastic, timber, rubber) | FEN | FVF | 23-39 21 11 11 13 | Boat Fenders |

LOD-I Requirement for Fender Model

All the attributes specified in the AIR of the PWD should be included in the Fender Model for LOD-I 200, 300, 400 and 500 requirements. During the preparation of the PIM, the LOD-I should align with the details in the AIR of PWD as far as practicable

Capping (Rubber or Timber)

The longitudinal piece of timber fitted over the heads of the frames or timbers of the port and marine facilities to form the top of the bulwark.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object elements | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object with components to be modelled according to AIR of the PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Chain for Fender System

A chain to fasten the fenders and the rubber buffer

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and length | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | Same as 200 | Exact nominal size and length | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object with components to be modelled according to AIR of the PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Eye blot for Fender System

Eyebolts is some form of anchor bolt to enable fastening into fenders.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with openings | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | Same as 200 | Exact nominal dimension of openings | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object with components to be modelled according to AIR of the PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Frontal Pad

Rubber protection elements at fender panels.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object elements | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object with components to be modelled according to AIR of the PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Horizontal Fender (plastic, timber, rubber)

A fender is a bumper used to absorb the kinetic energy of a boat or vessel berthing against the port and marine facility.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object elements | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Horizontal/Inclined fender wailing

A horizontal/inclined beam used to provide support to the side of the fender system.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object elements | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Rubber Buffer

Rubber buffers are elastic buffers to limit the spring travel by moving masses to absorb the kinetic energy of objects as vibration dampers to achieve a damping effect and thus effectively protect objects from vibrations.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | Same as 200 | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Step Block (Rubber or Timber)

...

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with approximate size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object elements | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Steel Bracket

A steel bracket is a general-purpose connector where fender members cross each other at angles.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with approximate location and typical size of the openings and screw | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | Same as 200 | Exact nominal dimension of object and openings | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Steel Plate for Wailing system

A steel plate attached to both sides at each joint of the fenders.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with approximate location and typical size of the openings and screws | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | Same as 200 | Exact nominal dimension of object and openings | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Vertical Fender (plastic, timber, rubber)

A fender is a bumper used to absorb the kinetic energy of a boat or vessel berthing against the port and marine facility.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with approximate size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object elements | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Geotechnical Slope Model

Summary of elements

The Geological slope Model include the geological object elements proposed in the works for Geotechnical and Engineering services. The following object elements should be included:

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | Baffle | GSM | BAF | 23-11 17 00 | Retention Structures |
| 2 | Earthwork (cut/fill) (for slopeworks) | SUS | Multiple* | 23-11 13 11 17 | Soil Stabilization Fill |
| 3 | Flexible barrier net, shackle for net, round clip | GSM | FBR | 23-11 19 00 | Slide and Avalanche Protection |
| 4 | Flexible Barrier Post, Base plate, Footing, shackle on post, running wheel | GSM | FBR | 23-11 19 00 | Slide and Avalanche Protection |
| 5 | Flexible barrier cable rope, rope clip, braking element | GSM | FBR | 23-11 19 00 | Slide and Avalanche Protection |
| 6 | Man-made slope (Registered) | GSM | MMS | 14-34 11 00 | Slopped Topographical spaces |
| 7 | Natural slope | GSM | NSL | 14-34 11 00 | Slopped Topographical spaces |
| 8 | Raking Drains | GSM | RKD | 23-11 13 11 23 | Field Drainage Blocks |
| 9 | Rigid barrier | GSM | RBR | 23-11 17 00 | Retention Structures |
| 10 | Soil Nail | GSM | SON | 23-11 11 13 11 | Earth Reinforcement Soil nails |
| 11 | Site/Slope Boundary Polygon | LOT | SIT | 14-37 11 14 | Site |
| 12 | Terrain (Site formation) | DTM | TSF | 14-37 11 14 | Site |
| 13 | Geological model |  |  |  |  |
| 13a | Borehole | GEO | BOH | N/A | N/A |
| 13b | Fill | GEO | FIL | N/A | N/A |
| 13c | Compacted Fill | GEO | COF | N/A | N/A |
| 13d | Design Groundwater profile | GEO | GRW | N/A | N/A |
| 14 | Ground anchors | GSM | GAR | 23-11 11 11 | Retaining Stabilizing Ground Anchors |
| 15 | Gabion for Rigid Barrier | GSM | GAB | 23-11 17 15 | Gabions |
| 16 | Cushioning Material for Rigid Barrier | GSM | CUM | 23-11 00 00 | Site Products |
| 17 | Steel Grating for Rigid Barrier | GSM | GRA | 23-39 29 15 | Waste Water Channels, Gullies, Gratings, Covers |
| 18 | Tree Ring | GSM | TRR | 23-11 27 13 | Planting Accessories |
| 19 | Erosion Control Mat | GSM | ECM | 23-11 15 11 21 | Sheeting Synthetic Erosion Controls |
| 20 | Wire Mesh | GSM | WRM | 23-13 19 11 15 | Mesh for General Use |

LOD-I Requirement for Geotechnical Model

All the attributes specified in the LOD specification of GEO should be included in the Geotechnical Model for LOD-I 300, 400 and 500 requirements. During the preparation of the PIM, the LOD-I should align with the details in the LOD specification of GEO as far as practicable.

LOD-Doc / LOD-Document for Geological model

The BIM Manager shall refer to the BEP of the GEO in preparing the LOD-DOC of the PIM and as-built information model.

Baffle

An element for natural terrain hazard mitigation measure constructed at the uphill of the proposed rigid/flexible barrier to serve as prescribed measures for enhancing robustness of the mitigation scheme.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object including post, footings and foundations | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object including post, footings and foundations | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid of each component | N/A |
| 400 | 3D Object including post, footings, foundations, steel base with stiffeners, bolts with washers and nuts, cement grout (if any) | Exact construction size of each component parts | Same as 300 | 3D solid of each component rendered by Material texture according to the finish material | N/A |

Earthwork for slope works (Cut/Fill)

Earthwork consists of excavations (cuts) and embankments (fills) for slope formation. Earthwork includes all types of materials excavated and placed in embankment, including soil, granular material, rock, shale, and random material.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Cut and fill area boundary | Assumed boundary | object insertion point(s) approximately located | 2D hatches | N/A |
| 200 | Cut and fill area boundary, 2D Contour lines and spot levels | Approximate boundary | Spots and contour insertion point(s) approximately located | 2D polygons, contour lines and symbols | N/A |
| 250 | N/A | N/A | N/A | N/A | N/A |
| 300 | 3D Digital terrain models of each calculated cut and fill surfaces | Exact nominal locations and levels | Object insertion point(s) located and orientated exactly as nominal | 3D Boundary, Contour lines, spot levels and symbols | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 400 | 3D Digital terrain models of each calculated rock and soil cut and fill solids | Exact calculated locations and volumes | Object insertion point(s) located and orientated exactly as calculated | 3D solids of each component | N/A |

Flexible barrier net, Base plate, shackle for net, round clip, Post, Footing, shackle on post, running wheel, cable rope, rope clip, braking element

A flexible barriers net is used to trap stream loads from a traversing a stream course.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including post and barrier with approximated size | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including standing post, foundation concrete pad, steel wire ropes, ground anchor | Exact nominal location, dimension, and orientation | Objects insertion point(s) located, orientated and aligned exactly as nominal locations, levels and alignment. | 3D solid block of each component; barrier as a semi-transparent sheet | Steel post location and orientation updated with the alignment of the barrier |
| 400 | 3D Object element including standing post, barrier, foundation concrete pad, steel wire ropes, ground anchor, breaking elements | Exact setting out location, dimension and orientation | Objects insertion point(s) located, orientated, and aligned exactly as setting out locations, levels and alignment; approximate location of breaking elements | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Man-made slope (Registered Slope)

A slope formed by unnatural process and usually registered under a maintenance department.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | Slope boundary, 2D Contour lines and spot levels | Approximate dimension | Object insertion point(s) approximately located | 2D polygons, contour lines and symbols | N/A |
| 250 | 3D Digital terrain models including locations of exposed rock head, toe-line, soil nails, settlement markers | Exact Surveyed locations, extend and size | Object insertion point(s) is located at / derived from the onsite survey point(s) | 3D Boundary, Contour lines, spot levels and symbols | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 300 | 3D Digital terrain models including locations of exposed rock head, toe-line, soil nails, settlement markers, berm in 3D polylines / symbols | Exact nominal locations, extend and size | Object insertion point(s) located and orientated exactly as nominal | 3D Boundary, Contour lines (5m major, 1m minor), spot levels, 3D polylines and symbols | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 400 | 3D Digital terrain models including soil/rock interfaces, benching and waterproofing layer, as well as locations of exposed rock head, toe-line, soil nails, settlement markers, berm in 3D polylines /symbols | Exact setting out locations, extend and size | Same as 300 | Rendered by Material texture according to the finish material | N/A |

Natural slope

A slope formed by natural process.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | Slope boundary, 2D Contour lines and spot levels | Approximate dimension | Object insertion point(s) approximately located | 2D polygons, contour lines and symbols | N/A |
| 250 | 3D Digital terrain models including locations of man-made features, large boulders, rock outcrop, tree locations | Exact Surveyed locations, extend and size | Object insertion point(s) is located at / derived from the onsite survey point(s) | 3D Boundary, Contour lines, spot levels and symbols | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Raking Drains

Raking drains are used in lowering the groundwater level and relieving the groundwater pressure at depth or used as contingency measures to cater for uncertainties in the groundwater conditions and possible adverse effects of subsurface seepage (e.g. from leaking services) on slope stability.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact size/dimension | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal locations, levels and alignment. | 3D solid block of each component | N/A |
| 400 | 3D Object element including the drill hole, and end cap | Exact nominal dimension of each component | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Rigid barrier

Rigid debris-resisting barriers are a key example of risk mitigation measures, which catch landslide debris and prevent them from reaching populated or developed areas.

Information Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including barrier structure, slabs, wall, large openings, cushioning material, foundations | Exact nominal dimension and size | Object insertion point(s) located and orientated exactly as nominal locations, levels and alignment. | 3D solid block of each component | N/A |
| 400 | 3D Object element including barrier structure, slabs, wall, large openings, cushioning material, foundations, movement joints, foundation piles and caps, steel furniture (e.g. trash grating) | Exact nominal dimension of each component | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Soil Nail

Soil nailing is a remedial construction measure to treat unstable natural soil slopes or as a construction technique that allows the safe over-steepening of new or existing soil slopes.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension/orientation | Assumed / Typical size of component (s) | Object insertion point(s) approximately located and orientated | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Assumed size | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including steel bars, soil nail head, tie beams, grillage beams, grout envelope | Exact nominal dimension and size | Object insertion point(s) located and orientated exactly as nominal locations and levels | 3D solid block of each component | N/A |
| 400 | Same as 300 | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Site/Slope Boundary Polygon

Site boundary means the outermost perimeter of the slope.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 2D polylines | Assumed / Typical size of component(s) | Object insertion point(s) approximately located | N/A | N/A |
| 250 | 3D polylines | Exact Surveyed locations and levels | Object insertion point(s) is located at / derived from the onsite survey point(s) | Closed polygons | N/A |
| 300 | 3D polygons | Exact nominal locations and levels | Object insertion point(s) located and orientated exactly as nominal | Closed polygons | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Terrain  (Site formation)

Proposed topography is the mesh representation of entire finished site surfaces.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | Site boundary, 2D Contour lines and spot levels | Approximate dimension | Object insertion point(s) approximately located | 2D polygons, contour lines and symbols | N/A |
| 250 | 3D Digital terrain models including slope and platforms | Exact Surveyed locations and levels | Object insertion point(s) is located at / derived from the onsite survey point(s) | 3D Boundary, Contour lines, spot levels and symbols | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 300 | 3D Digital terrain models, including slope, platforms and breaklines of proposed man-made features | Exact nominal locations and levels | Object insertion point(s) located and orientated exactly as nominal | 3D Boundary, Contour lines, spot levels and symbols | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 400 | N/A | N/A | N/A | N/A | N/A |

Borehole

A borehole is a narrow shaft bored in the ground, either vertically or horizontally to support site investigation.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size | Assumed / Typical size of component (s) | Object insertion point(s) approximately located and orientated | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Assumed size | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including concrete chamber, covers, outer casing, inner casing | Exact nominal dimension and size | Object insertion point(s) located and orientated exactly as nominal locations and levels | 3D solid block of each component | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Geology – Fill / Compact fill / Design Ground water profile

Various layer of material that covers the earth's surface.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Material’s extend boundary, 2D Contour lines and spot levels | Approximate location | Object insertion point(s) approximately located | 2D polygons, contour lines and symbols | N/A |
| 200 | 3D object including stratigraphy, with layers of fill, transported soils, location of the boreholes | Approximate location, depth and thickness | Object insertion point(s) approximately located | 3D Boundary, Contour lines, spot levels and symbols | N/A |
| 250 | Same as 200 | Same as 200 | Object modelled according to the borehole logs results or AGS from ground investigation works | Same as 200 | Same as 200 |
| 300 | 3D Digital terrain models, including inferred surfaces boundary of soils and rocks, location of boreholes, as well as ground water profile for design purpose | Exact inferred location, depth, and thickness of all components | 3D surfaces modelled according to the borehole logs results or AGS from ground investigation works | 3D Boundary, Contour lines, spot levels and 3D objects of surfaces | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 400 | N/A | N/A | N/A | N/A | N/A |

Ground anchors

.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size | Assumed / Typical size of component (s) | Object insertion point(s) approximately located and orientated | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Assumed size | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including steel bar, wire rope, concrete pad, grout envelope | Exact nominal location, dimension, and orientation | Object insertion point(s) located and orientated exactly as nominal locations and levels | 3D solid block of each component | N/A |
| 400 | Same as 300 | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Gabion for Rigid Barrier

.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size | Assumed / Typical size of component (s) | Object insertion point(s) approximately located and orientated | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact size/dimension | Exact nominal location, dimension, and orientation | Object insertion point(s) located and orientated exactly as nominal locations, levels and alignment. | 3D solid block of each component | N/A |
| 400 | Same as 300 | Exact setting out location, dimension and orientation | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Cushioning Material for Rigid Barrier

.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size | Assumed / Typical size of component (s) | Object insertion point(s) approximately located and orientated | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact size/dimension | Exact nominal location, dimension, and orientation | Object insertion point(s) located and orientated exactly as nominal locations, levels and alignment. | 3D solid block of each component | N/A |
| 400 | Same as 300 | Exact setting out location, dimension and orientation | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Steel Grating for Rigid Barrier

.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size | Assumed / Typical size of component (s) | Object insertion point(s) approximately located and orientated | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact size/dimension | Exact nominal location, dimension, and orientation | Object insertion point(s) located and orientated exactly as nominal locations, levels and alignment. | 3D solid block of each component | N/A |
| 400 | Same as 300 | Exact setting out location, dimension and orientation | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Tree Ring

.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size | Assumed / Typical size of component (s) | Object insertion point(s) approximately located and orientated | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Assumed size | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact size/dimension | Exact nominal dimension and size | Object insertion point(s) located and orientated exactly as nominal locations and levels | 3D solid block of each component | N/A |
| 400 | Same as 300 | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Erosion Control Mat

.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with extend and thickness | Assumed / Typical size of component (s) | Object insertion point(s) approximately located and orientated | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | Same as 250 | Exact nominal thickness of each layer | Object insertion point(s) located and orientated exactly as nominal locations and levels | Overall shape 3D solid block, rendered by Material texture according to the finish material | N/A |
| 400 | Same as 300 | Same as 300 | Same as 300 | Same as 300 | N/A |

Wire Mesh

.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size | Assumed / Typical size of component (s) | Object insertion point(s) approximately located and orientated | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Assumed size | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with extend and thickness | Exact nominal thickness of each layer | Object insertion point(s) located and orientated exactly as nominal locations and levels | Overall shape 3D solid block, rendered by Material texture according to the finish material | N/A |
| 400 | Same as 300 | Same as 300 | Same as 300 | Same as 300 | N/A |

Geotechnical Structure Model

Summary of elements

The Geotechnical Structure Model includes the structural object elements proposed in the works in Geotechnical and Engineering services. The following object elements should be included:

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | Foundation (pile) | SFD | FPL | 23-13 29 11 | Foundation piles |
| 2 | Foundation (pile cap) | SFD | FCA | 23-13 29 11 11 21 | Pile Caps |
| 3 | Foundation (ground beam) | SFD | FBM | 23-13 29 15 13 | Grade Beams (shallow foundations) |
| 4 | Footing | SFD | OTR | 23-13 29 15 15 | Strip Foundation Blocks (shallow foundations) |
| 5 | Mass Concrete Infill (including No-fines Concrete) | GSM | MAC | 23-13 15 11 | Concretes |
| 6 | Retaining Wall on Slope | SUS | RET | 23-11 17 13 | Retaining Walls |
| 7 | Structural concrete beam | GSM | BEM | 23-13 35 11 | Structural Frames |
| 8 | Structural concrete wall | GSM | SWL | 23-13 31 11 | Structural Concrete |
| 9 | Structural concrete column | GSM | SCL | 23-13 35 11 | Structural Frames |
| 10 | Structural concrete slab | GSM | SLA | 23-13 31 11 | Structural Concrete |
| 11 | Structural steel column, post | GSM | SCL | 23-13 35 11 | Structural Frames |
| 12 | Steel access ladder | GSM | SAL | 23-17 23 15 | Ladders |
| 13 | Temporary work, temporary structure, platform | GSM | TMW | 23-23 25 00 | Scaffolding |
| 14 | Steel Handrailing (to Maintenance Stairway on Slope) | GSM | SRL | 23-17 25 13 | Handrails |
| 15 | Steel Staircase | GSM | SST | 23-17 23 17 11 | Stair component products |
| 16 | Steel Gate | GSM | SGA | 23-17 23 17 11 21 | Stair Barrier Gates |
| 17 | Maintenance Access (incl. Concrete Stairway/Berm) | GSM | CMS | 23-13 31 17 | Concrete Formwork |
| 18 | Foundation (Other) | SFD | Multiple* | 23-13 29 00 | Chain Link Metal Fences |
| 19 | Chain Link Fence on Slope | GSM | CLF | 23-11 25 19 19 | Foundations |
| 20 | Skin Wall | GSM | SWL | 23-13 00 00 | Structural and Exterior Enclosure Products |

LOD-I Requirement for Geotechnical Structure Model

All the attributes specified in the LOD specification of GEO should be included in the Geotechnical Structural Model for LOD-I 300, 400 and 500 requirements. During the preparation of the PIM, the LOD-I should align with the details in the LOD specification of GEO as far as practicable.

LOD-Doc / LOD-Document for Geotechnical Structure model

The BIM Manager shall refer to the BEP of the GEO in preparing the LOD-DOC of the PIM and as-built information model.

Structural Foundation – Pile / Pile Cap / Foundation / Footings

Foundation refers to a series of columns constructed or inserted into the ground to transmit loads to a lower level of subsoil. Foundation/Ground beam refers to reinforce concrete beams for supporting walls and joists at or near ground level. Footing refers to foundation unit constructed in brick work, stone masonry or concrete under the base of a wall column for the purpose of distributing the load over a large area.

The finish should be separated from the Beam 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | 3D Object with all foundation components | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all foundation components | Exact nominal size, dimension and cut-off depth | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | Move with structural grid and reference elevation plane |
| 400 | 3D Object element modelled with all foundation components, including reinforcements, chamfer, etc. | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Mass concrete fill

Mass concrete fill refers to any volume of concrete with dimensions large enough to required that measures be taken to cope with the generation of heat from hydration of cement and attendant volume change to minimize cracking.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact thickness | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with exact thickness and reinforcement | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Retaining Wall on Slope

Retaining walls are relatively rigid walls used for supporting soil laterally so that it can be retained at different levels on the two sides. Retaining walls are structures designed to restrain soil to a slope that it would not naturally keep to.

The finish / skin wall should be separated from the wall 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object, including footing with size and shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including foundation and retaining wall elements. | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid of each component | N/A |
| 400 | 3D Object element including bearing depth, foundation, reinforcement, pile cut-off depths (if applicable) Openings modelled according to the requirements of the building services builders | Exact construction size of each component parts according to joints | Same as 300 | 3D solid of each component rendered by Material texture according to the finish material | N/A |

Structural concrete beam

Beams refer to structural elements that resist loads applied laterally to their axis.

The finish should be separated from the Beam 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element | Exact nominal size and dimension | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | Responsive with structural grid and reference elevation plane |
| 400 | 3D Object element modelled with reinforcements, camber, chamfer, etc. | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Structural concrete wall

Structure wall refers to a load bearing wall or a wall that carries load in addition to its own load.

The finish should be separated from the wall 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall thickness (including the finish) | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | Wall thickness of the major components and their finish | Modelled from floor slab to soffit of beam or slab above | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element modelled according to the details of the manufacturer’s information, including wall finish detail including tiling, stone, cladding and screed. Openings modelled according to the requirements of the building services builders | Modelled from Joint to Join from floor slab to soffit of beam or slab above | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Schedules to be dynamically updated from the size of the Openings |

Structural concrete column

Column refers to vertical structure intended to transfer a compressive load.

The finish should be separated from the Beam 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element | Exact nominal size and dimension | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | Responsive with structural grid and reference elevation plane |
| 400 | 3D Object element modelled with reinforcements, camber, chamfer, etc. | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Structural concrete slab

Slab refers concrete structural element used to create horizontal surfaces such as berms and decks.

The finish should be separated from the slab at LOD-G 300 or above.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact thickness | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with exact thickness and reinforcement | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Structural steel column, post

Steel column refers to vertical structure intended to transfer a compressive load.

The finish should be separated from the steel beam 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element | Exact nominal size and dimension | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | N/A |
| 400 | 3D Object element modelled with base plates, blots, clips, angles, welds, coping, washers, and nuts modelled according to the details of the manufacturer’s information, | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Steel access ladder

Access ladder is often used to describe the maintenance access ladder on the slope.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object | Assumed / Typical size of component(s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | Steps railings, posts, supports, post, sign plate, foundation, ladder, chord and bracing. | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | Same as 300 | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Temporary work, temporary structure, platform

Temporary structure used to support a work crew and materials to aid in the construction, maintenance and repair of the slope.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object including post, platform, guard rail | Assumed / Typical size of component(s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including post, platform, guard rail, base plates, bracings, toe board, access ladder | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | Same as 300 | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Steel Handrailing (to Maintenance Stairway on Slope)

Steel handrailing are railings designed to minimize fall risk at a work site where there is high fall risk.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object including post, rail, vertical members | Assumed / Typical size of component(s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including post, rail, vertical members, supports, baseplates and concrete footing | Exact nominal height, length, dimension of each component | Object insertion point(s) located and orientated exactly as nominal locations, levels and alignment | 3D solid block of each component | N/A |
| 400 | Same as 300 | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Steel Staircase

Access ladder is often used to describe the maintenance access ladder on the slope.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object | Assumed / Typical size of component(s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including Steps railings, posts, supports, post, sign plate, foundation, ladder, chord and bracing. | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal locations, levels and alignment. | 3D solid block of each component | N/A |
| 400 | Same as 300 | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Steel Gate

Steel Gate refers to a hinged barrier that can be used to secure an opening in a wall, fence and hedge.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all components, including post, foundation, and flats | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | Same as 300 | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Maintenance Access (including Concrete stairway / Berms)

Maintenance access include the concrete stairway and berms constructed to allow the inspection and maintenance of the man-made slope features.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object including stairway and berms with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including stairway and berms | Exact nominal dimension and thickness of each component; approximate size of steps | 3D solid block of each component rendered by Material texture according to the finish material | N/A | N/A |
| 400 | Same as 300 | Same as 300 | Same as 300 |  |  |

Foundation (Other)

.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all components, including post, foundation, and flats | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | Same as 300 | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Chain Link Fence on Slope

.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all components, including post, tube bracing, foundation, barbed wire, wire mesh and flats | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal locations, levels and alignment. | 3D solid block of each component rendered by Material texture according to the finish material | N/A |
| 400 | Same as 300 | Same as 300 | Same as 300 | Same as 300 | N/A |

Skin Wall

Skin wall is a type of wall treatment that can be a useful way of adding detail or changing the overall aesthetic of a retaining structure.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D hatches | Assumed size | Symbol insertion point(s) approximately located | 2D hatches | N/A |
| 200 | N/A | N/A | N/A | N/A | N/A |
| 250 | N/A | N/A | N/A | N/A | N/A |
| 300 | 3D Object element with exact shape and thickness | Exact nominal dimension, thickness of each component | Object insertion point(s) located and aligned exactly as parent 3D Object elements | Rendered by Material texture according to the finish material | N/A |
| 400 | Same as 300 | Same as 300 | Same as 300 | Same as 300 | Patterns and layouts to be updated with the manufacturer’s information |

For the GEO projects, the attributes specified in the BEP of the GEO should be included as LOD-I 300, 400 & 500 requirement

Highways Structure Model

Summary of elements

Highways Structural Model includes all the structural object elements proposed in the works for road, tunnels, and highways construction. It is applicable to all services. The following object elements should be included:

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Super structure model: Bridge |  |  |  |  |  |
| 1a | Abutment | TBR | AOR | 23-13 31 11 | Structural Concrete |  |
| 1b | Bearings | TBR | BEA | 23-39 13 13 19 | Bridge Bearings |  |
| 1c | Deck/Segment | TBR | DEC | 23-39 13 13 23 | Bridge Decking |  |
| 1d | Girder/main beams/webs | TBR | GOM | 23-39 13 13 13 | Bridge beams |  |
| 1e | Pier/Column/Soffit | TBR | COP/SOF | 23-13 31 11 | Structural Concrete |  |
| 2 | Noise Enclosure System |  |  |  |  |  |
| 2a | Noise Enclosure | TNE | Multiple* | 23-39 11 11 13 |  | Noise Barriers |
| 2b | Noise Barrier | TNB | Multiple* | 23-39 11 11 13 |  | Noise Barriers |
| 3 | Super structure model: Footbridge |  |  |  |  |  |
| 3a | Abutment | TFB | AOR | 23-13 31 11 | Structural Concrete |  |
| 3b | Bearings | TFB | BEA | 23-39 13 13 19 | Bridge Bearings |  |
| 3c | Deck/Segment | TFB | DEC | 23-39 13 13 23 | Bridge Decking |  |
| 3d | Girder/main beams/webs | TFB | GOM | 23-39 13 13 13 | Bridge beams |  |
| 3e | Pier/Column/Soffit | TFB | COP | 23-13 31 11 | Structural Concrete |  |
| 3f | Staircases (bridge) | TFB | ROS | 23-17 23 17 11 | Stair components products |  |
| 3g | Roof and skylights (bridge) | TFB | ROO | 23-13 39 11 | Exterior Roof Panels |  |
| 3h | Lift Tower (Bridge) | TLT | Multiple* | 23-13 31 11 | Structural Concrete |  |
| 4 | Sub- Structure Model |  |  |  |  |  |
| 4a | Pile | SFD | FPL | 23-13 29 11 | Foundation piles |  |
| 4b | Pile Cap | SFD | PCA | 23-13 29 11 11 21 | Pile Caps |  |
| 4c | Cable Trench | CTF | CHN | 23-13 31 11 | Structural Concrete |  |
| 4d | Excavation and refill for sub-structure | EXL | BAF | 23-11 13 11 17 | Soil Stabilization Fill |  |
| 4e | Foundation | Multiple* | Multiple* | 23-13 29 00 | Foundation |  |
| 5 | Other Structural elements |  |  |  |  |  |
| 5a | Diaphragm Wall | EXL | DIW | 23-11 17 00 | Retaining Structures |  |
| 5b | Lining | UCL | Multiple* | 23-39 13 11 15 | Tunnel Lining |  |
| 5c | Tunnel Structure Segments | TIS | Multiple* | 23-39 13 11 | Tunnels |  |
| 5d | Panel Wall | UPL | PW_ | 23-13 35 21 19 | Structural Panels |  |
| 5e | Temp. Support Structure | EXL | Multiple* | 23-11 11 00 | Ground Anchorages |  |

LOD-I Requirement for Highway Structure Model

All the attributes specified in the AIR of Highways Department should be included in the Highways Structural Model for LOD-I 300, 400 and 500 requirements. During the preparation of the PIM, the LOD-I should align with the details in the AIR of Highways Department as far as practicable.

LOD-Doc / LOD-Document for Highway Structure model

The BIM Manager shall refer to the LOD specification/AIR of Highways Department in preparing the LOD-DOC of the PIM and as-built information model.

Bridge Abutment

A bridge abutment is a structure which connects the deck of a bridge to the ground, at the ends of a bridge span, helping support its weight both horizontally and vertically.

The finish of the abutment should be modelled as separated object at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object, including wingwall, abutment wall, foundation, ballast wall, cantilever, bearing seat, foundations | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including wingwall, abutment wall, foundation, ballast wall, cantilever, bearing seat and plinth, foundations | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as designed. | 3D solid block of each component | N/A |
| 400 | 3D Object element including wingwall, abutment wall, foundation, ballast wall, cantilever, bearing seat and plinth, foundations and reinforcement | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Bridge Bearings

A bridge bearing is a component of a bridge which typically provides a resting surface between bridge piers and the bridge deck. The purpose of a bearing is to allow controlled movement and thereby reduce the stresses involved.

The finish of the abutment should be modelled as separated object at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of object | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with major components | Exact nominal dimension | Object insertion point(s) located, levelled and orientated exactly as designed. | 3D solid block of each component | N/A |
| 400 | 3D Object element including upper plate, top bearing plate, urethane disc, limiting ring and threaded shear pin | Exact nominal dimension and size of each component | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Bridge Deck/Segment

A deck is the surface of a bridge. A structural element of its superstructure, it may be constructed of concrete or steel.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including the outline of slab and deck | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | Dynamically updated according to the changes in the parent alignment and sections |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including top slab, bottom slab, parapet, profile barrier, boxing out | Exact nominal dimension of each component | Object continuously aligned and orientated exactly as designed 3D alignment and cross fall of the parent road/object | 3D solid block of each component | Dynamically updated according to the changes in the parent alignment, cross fall, and sections |
| 400 | 3D Object element including top slab, bottom slab, parapet, profile barrier, boxing out, catchpits, gully, downpipes, blister for prestress tendon, openings, construction joints and expansion joints | Exact construction size of each component parts according to joints | Object location and orientated exactly as designed setting out. | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Bridge Girder/main beams/webs

A girder is a support beam used in construction used to build bridges. It is the main horizontal support of a structure which supports smaller beams. Girders often have an I-beam cross section composed of two load-bearing flanges separated by a stabilizing web, but may also have a box shape, Z shape, or other forms.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including concrete beam, top and bottom flange, bar, interior, and exterior girder | Exact nominal size and dimension | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | N/A |
| 400 | 3D Object element including concrete beam, top and bottom flange, bar, interior and exterior girder, wedge, web, plate, bracket, reinforcement | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Bridge Pier/Column/Soffit

A pier short column used as a foundation member in construction. Also, a large column or wall type bridge support. A column is a Vertical structural member supporting a floor beam, girder, or other member, and supporting primarily vertical loads.

The finish of the abutment should be modelled as separated object at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including foundation, pier, pier head, portal | Exact nominal size and dimension | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | Responsive with structural grid and reference elevation plane |
| 400 | 3D Object element including foundation, pier, pier head, portal with reinforcement | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Noise Enclosure System / Noise Barrier

A noise enclosure system is an exterior structure designed to protect inhabitants of sensitive land use areas from noise pollution

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Lines and Symbols | N/A |
| 200 | 3D Object with size and shape of the steel structure, foundation, panels | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including steel structure, barrier, foundation, panels | Exact nominal size and dimension | Located, orientated and sloped as nominal | Overall shape 3D solid block of each component | N/A |
| 400 | 3D Object element including steel structure, barrier, foundation, panels, blots, nuts, angles, welds, coping, bases plates, bracing to support fabrication | Exact construction size and shape according to construction joints | Same as 300 | Overall 3D shape solid block of each component rendered by Material texture according to the finish material | N/A |

Footbridge Abutment

A footbridge abutment is a structure which connects the deck of a bridge to the ground, at the ends of a bridge span, helping support its weight both horizontally and vertically.

The finish of the abutment should be modelled as separated object at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object, including wingwall, abutment wall, foundation, ballast wall, cantilever, bearing seat, foundations | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including wingwall, abutment wall, foundation, ballast wall, cantilever, bearing seat and plinth, foundations | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as designed. | 3D solid block of each component | N/A |
| 400 | 3D Object element including wingwall, abutment wall, foundation, ballast wall, cantilever, bearing seat and plinth, foundations, and reinforcement | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Footbridge Bearings

A bridge bearing is a component of a bridge which typically provides a resting surface between bridge piers and the bridge deck. The purpose of a bearing is to allow controlled movement and thereby reduce the stresses involved.

The finish of the abutment should be modelled as separated object at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of object | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with major components | Exact nominal dimension | Object insertion point(s) located, levelled and orientated exactly as designed. | 3D solid block of each component | N/A |
| 400 | 3D Object element including upper plate, top bearing plate, urethane disc, limiting ring and threaded shear pin | Exact nominal dimension and size of each component | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Footbridge Deck/Segment

A deck is the surface of a bridge. A structural element of its superstructure, it may be constructed of concrete or steel.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including the outline of slab and deck | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | Dynamically updated according to the changes in the alignment and sections |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including top slab, bottom slab, parapet, profile barrier, boxing out | Exact nominal dimension of each component | Object aligned and orientated exactly as designed 3D alignment and cross fall | 3D solid block of each component | Dynamically updated according to the changes in the alignment, cross fall, and sections |
| 400 | 3D Object element including top slab, bottom slab, parapet, profile barrier, boxing out, catchpits, gully, downpipes, blister for prestress tendon, openings, construction joints and expansion joints | Exact construction size of each component parts according to joints | Object location and orientated exactly as designed setting out. | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Footbridge Girder/main beams/webs

A girder is a support beam used in construction used to build bridges. It is the main horizontal support of a structure which supports smaller beams. Girders often have an I-beam cross section composed of two load-bearing flanges separated by a stabilizing web, but may also have a box shape, Z shape, or other forms.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including concrete beam, top and bottom flange, bar, interior and exterior girder | Exact nominal size and dimension | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | N/A |
| 400 | 3D Object element including concrete beam, top and bottom flange, bar, interior and exterior girder, wedge, web, plate, bracket, reinforcement | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Footbridge Pier/Column/Soffit

A pier short column used as a foundation member in construction. Also, a large column or wall type bridge support. A column is a Vertical structural member supporting a floor beam, girder, or other member, and supporting primarily vertical loads.

The finish of the abutment should be modelled as separated object at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including foundation, pier, pier head, portal | Exact nominal size and dimension | Located, orientated, and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | Responsive with structural grid and reference elevation plane |
| 400 | 3D Object element including foundation, pier, pier head, portal with reinforcement | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Footbridge Staircases (Bridge)

Staircases refer to a set of steps leading from one floor of a footbridge to another.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with threads, risers, landings, and foundations | Approximate length, width, dimension, and levels | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with threads, risers, landings, and foundation to accurately indicate stringers and nosing | Exact nominal length, width, dimension, and levels | Object insertion point(s) located and aligned exactly as nominal | 3D solid block of each component | N/A |
| 400 | 3D Object element with threads, risers, landings and foundation to accurately indicate stringers and nosing with reinforcement | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Footbridge Roof and skylights (bridge)

Roof refers to structure forming the upper covering of a footbridge. Skylight refers to opening build into the roof of a footbridge filled with glass

The finish should be separated from the parent roof 3D Object elements at LOD-G 300 or above.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all components and parts | Exact design dimension of each component | Object insertion point(s) located and orientated exactly as designed. | 3D solid block of each component | N/A |
| 400 | 3D Object element with all components and parts, including the reinforcement components, frame elements and glazing, and Ironmongery according to the manufacturer’s information | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Footbridge Lift Tower (Bridge)

The structural element to provide the travel space of the escalator for footbridge.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with columns, beams, curtain walls | Exact nominal size and dimension | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | Responsive with structural grid and reference elevation plane |
| 400 | 3D Object element Elements with columns, beams, curtain walls with reinforcement | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Sub-Structure/foundation Pile and Pile cap

Foundation (pile and pile cap) refers to a series of columns constructed or inserted into the ground to transmit loads to a lower level of subsoil.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | 3D Object with all foundation components | Assumed / Typical size of component (s) | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all foundation components | Exact nominal size, dimension and cut-off depth | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | N/A |
| 400 | 3D Object element modelled with all foundation components, including reinforcements, chamfer, etc. | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Sub-Structure Cable Trench

Cable trenching is a method of laying cables into the ground by digging trenches. The cables are submerged underground via the cable trench structure and are then covered by a layer of earth, bitumen or concrete.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | 3D Object with all foundation components | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all foundation components | Exact nominal size, dimension and depth | Located, orientated, and sloped as nominal alignments, offsets and levels | 3D solid block of each component | N/A |
| 400 | 3D Object element modelled with all foundation components, including reinforcements and chambers | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Excavation and refill for sub-structure

Excavation and refill refer to the soil or rock removed / fill from/to a site to form/fill an open face, hole or cavity.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | 3D Object with extend and depth | Assumed / approximate size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | N/A | N/A | N/A | N/A | N/A |
| 300 | 3D Object element with extend and depth | Exact nominal location and elevation of each terrain mesh | Generated from 3D digital terrain meshes located and orientated exactly as nominal. | 3D solid block of each soil/rock components | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Diaphragm Wall

Diaphragm wall is a continuous wall constructed in ground in to facilitate certain construction activities, such as: As a retaining wall. As a cut-off provision to support deep excavation. As the final wall for basement or other underground structure (e.g. tunnel and shaft).

Geometrical Requirement\

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall thickness | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including wall thickness of the major components and their finish | Modelled from floor slab to soffit of beam or slab above | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element modelled according to the details of the manufacturer’s information, including reinforcement and wall finish detail Openings modelled according to the requirements of the building services builders | Modelled from Joint to Joint from slab to soffit of slab | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Lining

Tunnel lining is used in both trenchless construction and rehabilitation to maintain the shape of the tunnel and to shield whatever might pass through it.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall thickness | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | Dynamically updated according to the changes in the parent alignment, cross fall, and sections |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including base slab, ventilation tunnel, lining with exact thickness | Exact nominal alignment and thickness | Object continuously aligned and orientated exactly as designed 3D alignment and cross fall of the parent road/object. | 3D solid block of each component | Dynamically updated according to the changes in the parent alignment, cross fall, and sections |
| 400 | 3D Object element including reinforcement. Openings modelled according to the requirements of the building services builders | Modelled from Joint to Joint | Object insertion point(s) located, and aligned exactly as nominal alignments, offsets and level at Joints | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Tunnel Structure segments

Segmental concrete tunnel liners are prefabricated structural support pieces that are combined to form a complete tube along tunnel passageways. The segments are created in manufacturing plants for tunnel projects

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall thickness | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | Dynamically updated according to the changes in the alignment, cross fall, and sections |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact thickness | Exact nominal alignment and thickness | Object insertion point(s) located, and aligned exactly as nominal alignments, offsets and cross-fall. | 3D solid block of each component | Dynamically updated according to the changes in the alignment, cross fall, and sections |
| 400 | 3D Object element, including reinforcement according to the manufacturer’s information to support fabrication (if applicable) | Modelled from Segment to Segment | Object insertion point(s) located, and aligned exactly as nominal alignments, offsets and cross-fall/level at Joints | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Panel Wall

Panel wall means a non-bearing wall in skeleton construction, built between columns or piers, and wholly supported at each story.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall thickness | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including wall thickness of the major components and their finish | Modelled from floor slab to soffit of beam or slab above | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element modelled according to the details of the manufacturer’s information, including reinforcement and wall finish detail Openings modelled according to the requirements of the building services builders | Modelled from Joint to Joint from slab to soffit of slab | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Temp. Support Structure / ELS

Excavation and lateral support works (ELS works) is normally called for in the construction of pile caps, basement, underground drainage and tunnel works.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | 3D Object with size and shape of the supports | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | N/A | N/A | N/A | N/A | N/A |
| 300 | 3D Object element with size and shape of the supports and foundation elements | Exact nominal size and dimension | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Landscape Model

Summary of elements

Landscape Model includes all the object elements for the landscaping works proposed in the works. It is applicable to all services. The following object elements should be included:

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | Barrier | FBF | Multiple* | 23-39 11 11 15 | Traffic Barrier |
| 2 | Bollard | FBL | Multiple* | 23-39 11 19 | Bollards |
| 3 | Hard Surface Cover (e.g. shotcrete) | GSM | HSC | 23-11 13 11 | Soil stabilization products |
| 4 | Lighting | LSL | Multiple* | 23-35 47 11 21 23 | Street and Roadway Lighting Fixture |
| 5 | Planter (including planter for slope) | LTW | SHS | 22-11 27 13 | Planting Accessories |
| 6 | Railing / Handrail | FRA | Multiple* | 23-17 25 11 | Guardrails |
| 7 | Recreation area/facilities | LOT | REA | 23-11 31 00 | Athletic and Recreational Surfaces |
| 8 | Retaining Wall Finishes | DTL | FIN | 23-13 00 00 | Structural and Exterior Enclosure Products |
| 9 | Signage / Traffic Sign | FTW | Multiple* | 23-39 11 13 | Roadway signage |
| 10 | Sign Gantry | VRS | ___ | 23-39 11 13 | Roadway signage |
| 11 | Tree (New plant) | LTP | Multiple* | 23-11 27 19 13 | Trees |
| 12 | Vegetation Surface Cover | GSM | VSC | 23-11 13 11 | Soil stabilization products |

Barrier

Barrier refers to linear 3D Object elements installed in the works area to control road traffic to prevent automotive vehicles from colliding or crashing into pedestrians and structures.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Lines and Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including the post, barrier, footings, and foundation | Exact nominal dimension of each component | Object continuously aligned and orientated exactly as designed 3D alignment and cross fall of the parent pavement/object | 3D solid block of each component as a 3D continuous span | Barrier dynamically updated according to the changes in the parent alignment, cross fall, and sections |
| 400 | 3D Object element including the post, barrier, footings, and foundation | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Bollards

Bollards refers to posts installed in the works area to control road traffic prevent automotive vehicles from colliding or crashing into pedestrians and structures.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including the post, barrier, footings and foundations | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | Specific components with details to be modelled according to AIR of the maintenance parties | Size and shape requirement according to the AIR of the maintenance parties | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Hard surface cover

Hand surface cover refers to material (e.g. shotcrete) applies to the slope surface to protect it from landslip.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D hatches | Assumed size | Symbol insertion point(s) approximately located | 2D hatches | N/A |
| 200 | 3D Object with extend and thickness | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with extend and thickness | Exact nominal thickness of each layer | Object insertion point(s) located exactly as nominal locations and elevations | 3D solid of each layer, rendered by Material texture according to the finish material | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Lighting

Lighting refers to the light mounted on a pole and constituting one of a series spaced at intervals along a public street or highway.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including the lights, post, footings and foundations | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | Specific components with details to be modelled according to AIR of the maintenance parties | Size and shape requirement according to the AIR of the maintenance parties | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Planter (including planter for slope)

Planter refers to the concrete structure for the planting of vegetations.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including footings | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid of each component | N/A |
| 400 | Specific components with details to be modelled according to AIR of the maintenance parties | Size and shape requirement according to the AIR of the maintenance parties | Same as 300 | 3D solid of each component rendered by Material texture according to the finish material | N/A |

Railing/Handrail

Railing, handrail, or guardrails refers to a fence or barrier made of rails to prevent people from falling off.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximated size | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all components and parts with all components, including railings, bracing, supports, posts and footing | Exact nominal dimension, size, length, height and alignment of each component | Object insertion point(s) located and aligned exactly as nominal alignment and levels. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Recreation area/facilities

Recreation area refers to surface/area designated to the recreation area or facilities.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D hatches | Assumed size | Symbol insertion point(s) approximately located | 2D hatches | N/A |
| 200 | 3D Object with extend and thickness | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with extend and thickness | Exact nominal thickness of each layer | Object insertion point(s) located exactly as nominal locations and elevations | 3D solid of each component | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Retaining Wall Finishes

Retaining Wall Finishes are used in the final part of the construction or manufacturing process, forming the final surface of the retaining wall. They can protect the retaining wall from impact, water, frost, corrosion, abrasion, and so on, and/or they can be decorative.

The finish should be separated from the parent 3D Object elements at LOD-G 300 or above.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D hatches | Assumed size | Symbol insertion point(s) approximately located | 2D hatches | N/A |
| 200 | N/A | N/A | N/A | N/A | N/A |
| 250 | N/A | N/A | N/A | N/A | N/A |
| 300 | 3D Object element with exact shape and thickness | Exact nominal dimension, thickness of each component | Object insertion point(s) located and aligned exactly as parent 3D Object elements | Rendered by Material texture according to the finish material | N/A |
| 400 | Same as 300 | Same as 300 | Same as 300 | Same as 300 | Patterns and layouts to be updated with the manufacturer’s information |

For the GEO projects, the attributes specified in the BEP of the GEO should be included as LOD-I 300, 400 & 500 requirement

Signage / Traffic Sign

Traffic signs or road signs are signs erected at the side of or above roads or highways to give instructions or provide information to road users.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including the post, sign plate, foundation, draw pit (if applicable) | Exact nominal dimension of each component according to the requirement of the Transport department | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | Same as 300 | Size and shape requirement according to the AIR of the maintenance parties | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Sign Gantry

Traffic signs or road signs are signs erected at the side of or above roads or highways to give instructions or provide information to users.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including the post, sign plate, TCSS sign-plates, foundation, draw pit (if applicable), bracing, cat ladder | Exact nominal dimension of each component according to the requirement of the Transport department | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element including the post, sign plate, foundation, draw pit (if applicable), bracing, cat ladder, TCSS accessories | Size and shape requirement according to the AIR of the maintenance parties | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Tree (New plant)

New plant tree is a perennial plant with an elongated stem, or trunk, supporting branches and leaves planted under the works’ requirement.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with diameter, height and spread | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | Object with specified diameter, height and spread | Exact nominal diameter, height and spread | Object insertion point(s) located exactly as nominal location(s) and level(s). | 3D solid of each component | N/A |
| 400 | Specific components with details to be modelled according to AIR of the maintenance parties | Size and shape requirement according to the AIR of the maintenance parties | Same as 300 | 3D solid of each component | N/A |

For the GEO projects, the attributes specified in the BEP of the GEO should be included as LOD-I 300, 400 & 500 requirement

Vegetation surface cover

Vegetation surface cover refers to vegetation material applies to the slope surface to protect it from landslip.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D hatches | Assumed size | Symbol insertion point(s) approximately located | 2D hatches | N/A |
| 200 | 3D Object with extend and thickness | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid block | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid block | N/A |
| 300 | 3D Object element with extend and thickness | Exact nominal thickness of each layer | Object insertion point(s) located exactly as nominal locations and elevations | Overall shape 3D solid block of each layer, rendered by Material texture according to the finish material | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Marine Civil Model

List of Common Objects Elements

The Civil Model mainly used in the Ports and marine services; the following object elements are classified under Civil Model:

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | Access/Cat Ladder | ALA | CAL | 23-17 23 15 11 | Vertical Ladders |
| 2 | Terrain (Proposed Profile or Tunnel Seabed Levels) | DTM | BAP | 14-34 17 24 | Shoreline |
| 3 | Barrier Bollard | SIT | CBB | 23-39 11 19 | Bollards |
| 4 | Bench | SIT | CBN | 23-11 29 13 11 | Exterior Benches |
| 5 | Concrete Plinth | SFD | CCP | 23-13 31 15 | Precast Structural Concrete |
| 6 | Fence | SIT | CFC | 23-11 25 19 | Fences |
| 7 | Gate | SIT | CGT | 23-11 25 11 | Perimeter Entry Devices |
| 8 | Lifebuoy | SIT | CLB | 23-29 00 00 | Facility and Occupant Protection products |
| 9 | Marine Notice Board | SIT | CMN | 23-19 11 17 | Notice Board |
| 10 | Mooring eye | FEN | CME | 23-39 21 11 11 11 | Mooring Post |
| 11 | Mooring Bollard | FEN | CBO | 23-39 21 11 11 11 | Mooring Post |
| 12 | Navigation Light Post | SIT | CNL | 23-39 21 17 13 | Navigation Lights |
| 13 | Pier Notice board | SIT | CNB | 23-19 11 17 | Notice Board |
| 14 | Terrain (Site formation) | DTM | TSF | 14-34 17 99 | Other Land and Water Topographical Transitions |
| 15 | Railing/Handrail | FRA | Multiple* | 23-17 25 11 | Guardrails |
| 16 | Signage/information plate (include landing /structural no. plate, pier) | SIT | CSG | 23-19 11 11 | Information Signs |
| 17 | Step iron | FEN | CSI | 23-39 21 11 11 11 | Mooring Post |
| 18 | Site Boundary Polygon | LOT | SIT | 14-37 11 14 | Site |
| 19 | Tidal Gauge House | SIT | CGH | 23-13 31 15 | Precast Structural Concrete |
| 20 | Tidal Gauge Tubes | SIT | CTG | 23-27 11 21 23 | Level Sensors |

LOD-I Requirement for Marine Civil Model

All the attributes specified in the AIR of the PWD should be included in the Marine Civil Model for LOD-I 200, 300, 400 and 500 requirements. During the preparation of the PIM, the LOD-I should align with the details in the AIR of PWD as far as practicable

Access/Cat Ladder

Cat ladder is often used to describe a ladder that is used for working on a sloping roof with a hook at one end and load spreading pads. These kinds of ladder are most commonly designed for use as access steps for maintenance to the roof of a building.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object | Assumed / Typical size of component(s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including steps railings, posts, supports, post, sign plate, foundation, cat ladder, chord and bracing. | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | Object height must be limited by designed max height |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Terrain (Proposed Profile or Tunnel Seabed Levels)

Bathymetry is the underwater topography to show the proposed water profile.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | Boundary, 2D Contour lines and spot levels | Assumed / Typical size of component(s) | Object insertion point(s) approximately located | 2D lines and symbols | N/A |
| 250 | 3D Digital terrain models | Exact Surveyed locations and depths | Object insertion point(s) is located at / derived from the onsite survey point(s) | Boundary, Contour lines and spot levels | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 300 | 3D Digital terrain models | Exact nominal locations and depths | Object insertion point(s) located and orientated exactly as nominal | Boundary, Contour lines and spot levels | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | Object rendered by Material texture according to the finish material | Same as 300 |

Barrier / Bollard

Bollards refers to posts and structure installed in the ports and marine facilities to control road traffic prevent automotive vehicles from colliding or crashing into pedestrians and structures.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including the post, bollard, and the footings | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Bench

A long seat for several persons in marine facilities.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all components and parts | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Concrete Plinth

A concrete plinth is a reinforced concrete beam that is constructed between the wall and its foundation.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact thickness | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Fence

A fence is a structure that encloses an area, typically outdoors, and is usually constructed from posts that are connected by boards, wire, rails, or netting. A fence differs from a wall in not having a solid foundation along its whole length.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks; | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including post, strut, horizontal railing, steel tie, hoist, foundation, mesh and barbed wire | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component; barrier fence as a semi-transparent sheet | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Gate

Gate refers to a hinged barrier that can be used to secure an opening in a wall, fence, and hedge.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object elements with all components, including post, foundation, and flats | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Lifebuoy

A lifebuoy is a life-saving buoy designed to be thrown to a person in water, to provide buoyancy and prevent drowning.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object elements with hanger | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Marine Notice Board

A display board in the Port and Marine facilities.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object elements with text and labels | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Mooring eye

A forged or cast metal loop to which mooring ropes may be tied.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object elements with size and width | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Mooring Bollard

A mooring bollard refers to a strong vertical post of timber or iron, fixed to the ground structure of ports and marine facilities, to which the ship's mooring lines etc are secured.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object elements with foundation | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Navigation Light Post

A marine navigation light post is the concrete/steel structure for the navigation lights.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object elements with foundation | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Pier Notice board

A bulletin board in the Port and Marine facilities for public.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object elements with size and width | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Terrain  (Site formation)

Proposed topography is the mesh representation of entire finished site surfaces.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | Site boundary, 2D Contour lines and spot levels | Approximate dimension | Object insertion point(s) approximately located | 2D polygons, contour lines and symbols | N/A |
| 250 | 3D Digital terrain models including slope and platforms | Exact Surveyed locations and depths | Object insertion point(s) is located at / derived from the onsite survey point(s) | 3D Boundary, Contour lines, spot levels and symbols | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 300 | 3D Digital terrain models, including layout and size of man-made features in 3D polylines / symbols | Exact nominal locations and depths | Object insertion point(s) located and orientated exactly as nominal | 3D Boundary, Contour lines, 3D polylines, spot levels and symbols | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | Rendered by Material texture according to the finish material | Same as 300 |

Railing/Handrail

Railing, handrail or guardrails refers to a fence or barrier made of rails to prevent people from falling off from the Port and Marine facilities.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximated size | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all components and parts with all components, including railings, bracing, supports, posts and footing | Exact nominal dimension, size, length, height and alignment of each component | Object insertion point(s) located and aligned exactly as nominal alignment and levels. | 3D solid of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid of each component rendered by Material texture according to the finish material | N/A |

Signage/information plate (including landing/structural no. plate, pier)

A bulletin board in the Port and Marine facilities to show the pier/structural number.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object elements include the plate and the text | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Step iron

A U-shaped heavy metal loop which is set into masonry work for ladders.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object with size and width | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Site Boundary Polygon

Site boundary means the outermost perimeter of the works/site.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | Site extend boundary | Assumed / Typical size of component(s) | Object insertion point(s) approximately located | 2D polygon | N/A |
| 250 | Site extend boundary | Exact Surveyed locations and levels | Object insertion point(s) is located at / derived from the onsite survey point(s) | Closed polygon | N/A |
| 300 | Site extend boundary | Exact nominal locations and levels | Object insertion point(s) located and orientated exactly as nominal | Closed polygon | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Tidal Gauge House

A tide gauges house is used to house the tidal gauge tube.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including wall, slab, frame, and cover | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Tidal Gauge Tubes

Tide gauges tubes are used to measure tides.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including the stilling well, float wheel, and the measurement equipment | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Marine Structure Model

Summary of elements

The Marine Structure Model includes all the object elements in the structural elements in the Port and Marine services proposed in the works. The following object elements should be included:

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | Terrain (Shoreline / Beach Finish Level) | DTM | BFL | 12-34 17 24 | Shoreline |
| 2 | Pile Cap | MSF | SPC | 23-13 29 11 11 21 | Pile Caps |
| 3 | Pile Foundation | MSF | SPI | 23-13 29 11 11 | Foundation Piles |
| 4 | Concrete Foundation for Beacon | MSF | SCF | 23-13 29 15 15 | Strip Foundation Blocks |
| 5 | Anchor Blot/Post | MSF | SAB | 23-13 35 19 | Rafters, Beams and Joists |
| 6 | Structural Beam (Concrete Beam, Steel Beam and Tie Beam) | MSF | SBM | 23-13 35 11 13 13 | Beams |
| 7 | Structural Column (Concrete Column, Steel Column and Post) | MSO | SCL | 23-13 35 11 13 11 | Column |
| 8 | Bracing (Horizontal/Vertical) | MSF | SBH /SBV | 23-13 35 19 | Rafters, Beams and Joists |
| 9 | Slab/Pier Deck | MSO | SLA | 23-15 17 13 19 | Precast Tile and Slab Flooring |
| 10 | Precast Beam Slab Panel | MSO | SPP | 23-13 31 15 | Precast Structural Concrete |
| 11 | Ramp | MSO | SRP | 23-17 23 11 | Ramps |
| 12 | Corbel/Concrete Bracket for Pier | MSO | SCO | 23-13 29 00 | Structural Foundation |
| 13 | Structural wall / retaining wall | MSO | SWL | 23-13 35 21 11 | Concrete structural wall |
| 14 | Concrete structure for Beacon, Dolphin, vertical seawall, and a solid pier | MSF | SCS | 23-13 31 11 | Structural Concrete |
| 15 | Steel structure for Beacon | MSO | SSS | 23-39 21 11 11 | Navigation components |
| 16 | Access Structure | MSO | SAC | 23-13 35 23 11 | Structural Concrete floor decks |
| 17 | Landing Platform | MSO | SLP | 23-13 35 23 11 | Structural Concrete floor decks |
| 18 | Landing Staircase Structure | MSO | SLS | 23-13 31 11 | Structural Concrete |
| 19 | Landing Step | MSO | SLT | 23-13 31 11 | Structural Concrete |
| 20 | Concrete Coping | MSF | SCC | 23-13 31 11 | Structural Concrete |
| 21 | Gabion Wall | MSO | SGW | 23-11 17 15 | Gabions |
| 22 | Wave Absorption Chamber | MSO | SWA | 23-13 31 11 | Structural Concrete |
| 23 | Precast Concrete Block (include seawall block, solid pier concrete block, wave wall/barrier) | MSO | SBK | 23-13 31 15 | Precast Structural Concrete |
| 24 | Rock Armour for seawall and breakwater | MSO | SRA | 23-39 21 15 13 | Seawalls |
| 25 | Rock Fill | MSO | SRF | 23-13 11 13 | Aggregates |
| 26 | Berm Stone | MSO | SBS | 23-13 21 11 | Concrete Masonry Units |
| 27 | Bagged Concrete | MSO | SBC | 23-13 21 11 | Concrete Masonry Units |
| 28 | Levelling Stone | MSO | SLV | 23-13 11 13 | Aggregates |
| 29 | Pell Mell Rubble | MSO | SPM | 23-13 11 13 | Aggregates |

LOD-I Requirement for Marine Structure Model

All the attributes specified in the AIR of the PWD should be included in the Fender Model for LOD-I 200, 300, 400 and 500 requirements. During the preparation of the PIM, the LOD-I should align with the details in the AIR of PWD as far as practicable

Terrain (Shoreline / Beach Finish Level)

Beaches are dynamic environments which for the buffer between land and sea. The Beach finish level refers to the terrain surface from the foreshore (surf zone) to the backshore (usually above the high tide mark).

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | Boundary, 2D Contour lines and spot levels | Assumed / Typical size of component(s) | Object insertion point(s) approximately located | 2D lines and symbols | N/A |
| 250 | 3D Digital terrain models | Exact Surveyed locations and levels | Object insertion point(s) is located at / derived from the onsite survey point(s) | Boundary, Contour lines and spot levels | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 300 | 3D Digital terrain models | Exact nominal locations and levels | Object insertion point(s) located and orientated exactly as nominal | Boundary, Contour lines and spot levels | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | Object rendered by Material texture according to the finish material | Same as 300 |

Pile and Pile cap

Foundation (pile and pile cap) refers to a series of columns constructed or inserted into the seabed to transmit loads to a lower level of subsoil.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | 3D Object with all foundation components | Assumed / Typical size of component (s) | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all foundation components | Exact nominal size, dimension and cut-off depth | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | N/A |
| 400 | 3D Object element modelled with all foundation components, including reinforcements, chamfer, etc. | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Concrete Foundation for Beacon

Foundation refers to the concrete structures constructed or inserted into the seabed to transmit loads to a lower level of subsoil.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | 3D Object with all foundation components | Assumed / Typical size of component (s) | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all foundation components | Exact nominal size, dimension and cut-off depth | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | N/A |
| 400 | 3D Object element modelled with all foundation components, including reinforcements, chamfer, etc. | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Anchor Blot/Post

Anchor bolts are used to connect structural and non-structural elements to concrete.

The base plates, bolts, clip angles, fixing should be separately modelled from the parent element at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | N/A | N/A | N/A | N/A | N/A |
| 250 | N/A | N/A | N/A | N/A | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | 3D Object element, such as base plates, blots, clips, angles, welds, coping, washers and nuts modelled according to the details of the manufacturer’s information, | Exact design dimension of each component | Object insertion point(s) located and orientated exactly as designed. | Overall shape, rendered by Material texture according to the finish material | N/A |

Structural Beam (Concrete Beam, Steel Bean and Tie Beam)

Beams refer to structural elements that resist loads applied laterally to their axis of the port and marine facilities.

The finish should be modelled separately from the Beam 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element | Exact nominal size and dimension | Located, orientated, and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | Responsive with structural grid and reference elevation plane |
| 400 | 3D Object element modelled with reinforcements, camber, chamfer, etc. | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Structural Column (Concrete Column, Steel Column and Post)

Column refers to vertical structure intended to transfer a compressive load of the port and marine facilities.

The finish should be modelled separately from the Column 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element | Exact nominal size and dimension | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | Responsive with structural grid and reference elevation plane |
| 400 | 3D Object element modelled with reinforcements, camber, chamfer, etc. | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Bracing (Horizontal/Vertical)

A bracing system serves to stabilize the main girders during construction, to contribute to the distribution of load effects and to provide restraint to compression flanges or chords where they would otherwise be free to buckle laterally.

The finish should be modelled separately from the Beam 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element | Exact nominal size and dimension | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | Responsive with structural grid and reference elevation plane |
| 400 | 3D Object element modelled with reinforcements, camber, chamfer, etc. | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Slab/Pier Deck

Slab refers concrete structural element. It is used to create horizontal deck surfaces of the port and marine facility.

The finish, surfacing material and floor paving should be modelled separately from the slab/deck at LOD-G 300 or above.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/2D lines | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact thickness | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with exact thickness and reinforcement | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Precast Beam Slab Panel

Precast Beams slab refer to precast structural elements that resist loads applied laterally to their axis of the port and marine facilities.

The finish should be modelled separately from the beam 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element | Exact nominal size and dimension | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | Responsive with structural grid and reference elevation plane |
| 400 | 3D Object element modelled with reinforcements, camber, chamfer, etc to support prefabrication | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Ramp

Ramp refers to sloped pathways used inside ports and marine facilities to provide access between vertical levels.

The finish, surfacing material and floor paving should be modelled separately from the parent Ramp object at LOD-G 300 or above.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/2D lines | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact thickness | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as designed. | 3D solid block of each component | N/A |
| 400 | 3D Object element with exact thickness and reinforcement | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Corbel/Concrete Bracket for Pier

A corbel is a structural piece of stone, wood or metal jutting from a wall to carry a super incumbent weight, a type of bracket. A corbel is a solid piece of material in the wall, whereas a console is a piece applied to the structure.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element Elements | Exact nominal size and dimension | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | Responsive with structural grid and reference elevation plane |
| 400 | 3D Object element modelled with reinforcements | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Structural wall / retaining wall

Structure wall refers to a load bearing wall or a wall that carries load in addition to its own load.

The finish and surfacing material should be modelled separately from the wall  at LOD-G 300 or above.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall thickness (including the finish) | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object including wall thickness of the major components and their finish | Modelled from floor slab to soffit of beam or slab above | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element including reinforcement, wall finish detail including tiling, stone, cladding and screed. Openings modelled according to the requirements of the building services builders | Modelled from Joint to Join from floor slab to soffit of beam or slab above | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Schedules to be dynamically updated from the size of the Openings |

Concrete structure for Beacon, Dolphin, vertical seawall, and a solid pier

Structural concrete elements/block(s) to support the port and marine facilities

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element | Exact nominal size and dimension of each block or component parts | Object insertion point(s) Located, levelled, and orientated and as nominal | 3D solid block of each component | N/A |
| 400 | 3D Object element modelled with reinforcements | Exact construction size of each block, component part according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Steel structure for Beacon

Steel structure above the structural foundation in Beacon facilities

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | 3D Object with size and shape of the supports | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | N/A | N/A | N/A | N/A | N/A |
| 300 | 3D Object element including post, strut, bracing | Exact nominal size and dimension | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | Overall shape 3D solid blocks | N/A |
| 400 | 3D Object element modelled with post, strut, bracing base plates, blots, clips, angles, welds, coping, washers, and nuts modelled according to the details of the manufacturer’s information, | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Access Structure

Structural concrete slabs or elements to allow the access of the port and marine facilities from the boarding boat.

The finish, surfacing material and floor paving should be modelled separately from the parent structure at LOD-G 300 or above.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element | Exact nominal size and dimension of each block or component parts | Object insertion point(s) Located, levelled, and orientated and as nominal | 3D solid block of each component | N/A |
| 400 | 3D Object element modelled with reinforcements | Exact construction size of each block, component part according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Landing Platform

A landing platform is a concrete platform sticking out into water, usually the sea, which people walk along or use when getting onto or off boats.

The finish, surfacing material and floor paving should be modelled separately from the slab at LOD-G 300 or above.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element | Exact nominal size and dimension of each block or component parts | Object insertion point(s) Located, levelled, and orientated and as nominal | 3D solid block of each component | N/A |
| 400 | 3D Object element modelled with reinforcements | Exact construction size of each block, component part according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Landing Staircase Structure

A landing is the area of a structural concrete floor slab near the top or bottom step of a stair.

The finish, surfacing material and floor paving should be modelled separately from the slab at LOD-G 300 or above.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/2D lines | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact thickness | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with exact thickness and reinforcement | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Landing Step

Landing steps are the concrete blocks between staircase landings

The finish, surfacing material and floor paving should be modelled separately from the slab at LOD-G 300 or above.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | Symbols/2D lines | N/A |
| 200 | 3D Object with threads, risers, and landings | Approximate length, width, dimension and levels | Object insertion point(s) approximately located | Overall shape of staircase | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape of staircase | N/A |
| 300 | 3D Object element with threads, risers, and the structural blocks | Exact nominal height, width and levels of each component | Object insertion point(s) located and aligned exactly as nominal | Overall shape of each step and structural blocks | N/A |
| 400 | 3D Object element with threads, risers, structural blocks and reinforcement | Same as 300 | Same as 300 | Overall shape rendered by Material texture according to the finish material | N/A |

Concrete Coping

Copings are used to cap the tops of masonry parapets and freestanding walls to prevent rainwater from penetrating into the construction below

The finish should be modelled separately from the concrete coping 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element | Exact nominal size and dimension | Located, orientated and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | Responsive with structural grid and reference elevation plane |
| 400 | 3D Object element modelled with reinforcements, camber, chamfer, etc. | Same as 300 | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Gabion Wall

A gabion wall is a retaining wall made of stacked stone-filled gabions tied together with wire

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | Generic Linear object with all foundation components | Assumed / Typical size of component (s) | Located, orientated and aligned as nominal alignment and level | Overall shape of a Linear object | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape of a Linear object | N/A |
| 300 | 3D Object element blocks with all foundation components | Exact nominal size, and dimension | Object insertion point(s) is located, aligned and levelled as nominal locations | Overall shape of a Linear object | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Object insertion point(s) is located, orientated and as nominal locations | Shape of each object block | N/A |

Wave Absorption Chamber

Wave absorption chamber is a structure chamber constructed to reduce wave reflection

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame, and cover with approximate size/dimension | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including wall, slab, frame, and cover | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element including wall, slab, frame, cover, and reinforcement to support prefabrication | Size and shape requirement according to the AIR of PWD | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Precast Concrete Block (include seawall block, solid pier concrete block, wave wall/barrier)

Precast concrete solid blocks to support the port and marine structure.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | 3D Object | Assumed / Typical size of component (s) | Located, orientated and aligned as nominal alignment and level | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element blocks, settlement marker | Exact nominal size, and dimension of each block | Object insertion point(s) is located, aligned, and levelled as nominal locations | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Object insertion point(s) is located, orientated and as nominal locations | N/A | N/A |

Rock Armour for seawall and breakwater

A human-placed rock or other material used to protect shoreline structures against water and wave.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | Generic extend of the object | Assumed / Typical extend and thickness | Located, orientated and aligned as nominal alignment and level | Overall shape of an aera coverage | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape of an aera coverage | N/A |
| 300 | 3D Object element | Exact nominal extend and thickness | Object insertion point(s) is located, orientated and levelled as nominal locations | Overall shape of a single solid block | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | Same as 300 | Same as 300 |

Rock Fill

Rock in armour layers and underlayers in rubble mound construction should normally be placed from the bottom to the top of a section, in such a manner and sequence that individual rock pieces interlock and do not segregate and the interstices are kept free of small rock fragments

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | Site boundary, 2D Contour lines and spot levels | Approximate dimension | Object insertion point(s) approximately located | 2D polygons, contour lines and symbols | N/A |
| 250 | 3D Digital terrain models including slope and platforms | Exact Surveyed locations and depths | Object insertion point(s) is located at / derived from the onsite survey point(s) | 3D Boundary, Contour lines, spot levels and symbols | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 300 | 3D Digital terrain models, including slope and platforms | Exact nominal locations and depths | Object insertion point(s) located and orientated exactly as nominal | 3D Boundary, Contour lines, spot levels and symbols | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 400 | N/A | N/A | N/A | N/A | N/A |

Berm Stone

Berm stone are used protect the toe of the port and marine structure against scouring due to waves and currents

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | Generic extend of the object | Assumed / Typical extend and thickness | Located, orientated and aligned as nominal alignment and level | Overall shape of an aera coverage | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape of an aera coverage | N/A |
| 300 | 3D Object element | Exact nominal extend and thickness | Object insertion point(s) is located, orientated and levelled as nominal locations | Overall shape of an single solid block | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | Same as 300 | Same as 300 |

Bagged Concrete

Bagged concrete is used to create seawall structure.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | Generic Linear object with all foundation components | Assumed / Typical size of component (s) | Located, orientated and aligned as nominal alignment and level | Overall shape of a Linear object | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape of a Linear object | N/A |
| 300 | 3D Object element blocks with all foundation components | Exact nominal size, and dimension | Object insertion point(s) is located, aligned and levelled as nominal locations | Overall shape of a Linear object | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Object insertion point(s) is located, orientated and as nominal locations | Shape of each object block | N/A |

Levelling Stone

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | Generic extend of the object | Assumed / Typical extend and thickness | Located, orientated and aligned as nominal alignment and level | Overall shape of a aera coverage | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape of a aera coverage | N/A |
| 300 | 3D Object element | Exact nominal extend and thickness | Object insertion point(s) is located, orientated and levelled as nominal locations | Overall shape of a single solid block | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | Same as 300 | Same as 300 |

Pell Mell Rubble

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | Generic extend of the object | Assumed / Typical extend and thickness | Located, orientated and aligned as nominal alignment and level | Overall shape of a aera coverage | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape of a aera coverage | N/A |
| 300 | 3D Object element | Exact nominal extend and thickness | Object insertion point(s) is located, orientated and levelled as nominal locations | Overall shape of a single solid block | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of PWD | Size and shape requirement according to the AIR of PWD | Same as 300 | Same as 300 | Same as 300 |

Roadworks Model

Summary of elements

The Roadworks Model includes all object elements of the road works proposed in the works. It is applicable to all services. The following object elements should be included:

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | Earthwork (Cut/Fill) | SUS | Multiple* | 23-11 13 11 17 | Soil Stabilization Fill |
| 2 | Kerb | RKB | Multiple* | 23-39 11 11 19 | Roadway Curbs |
| 3 | Man-made slope (for road construction) | SRS/SUS | Multiple* | 23-11 13 11 17 | Soil Stabilization Fill |
| 4 | Pavement |  |  |  |  |
| 4a | Carriage way | RCW | Multiple* | 23-11 21 13 | Roadways |
| 4b | Cycle Track | RCT | Multiple* | 23-11 21 00 | Pavements |
| 4c | Foot path | RFW | Multiple* | 23-11 21 00 | Pavements |
| 5 | Retaining Structure | SFD | DWL | 23-11 17 00 | Retaining Structures |
| 6 | Road Marking | RMK | Multiple* | 23-39 11 15 11 | Roadway Surface Markings |
| 7 | Site Boundary Polygon | LOT | SIT | 14-37 11 14 | Site |
| 8 | Traffic Island- Other | RIO | Multiple* | 23-39 11 11 19 | Roadway Curbs |
| 9 | Traffic Island- Refuge Island | RIR | Multiple* | 23-39 11 11 19 | Roadway Curbs |
| 10 | Terrain (Roadworks) | DTM | SIT | 14-37 11 14 | Site |

Earthwork associated with roadworks (Cut/Fill)

Earthwork consists of roadway excavations (cuts) and roadway embankments (fills) for highways and associated items of work. Earthwork includes all types of materials excavated and placed in embankment, including soil, granular material, rock, shale, and random material.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Cut and fill area boundary | Assumed boundary | object insertion point(s) approximately located | 2D hatches | N/A |
| 200 | Cut and fill area boundary, 2D Contour lines and spot levels | Approximate boundary | Spots and contour insertion point(s) approximately located | 2D polygons, contour lines and symbols | N/A |
| 250 | N/A | N/A | N/A | N/A | N/A |
| 300 | 3D Digital terrain models of each calculated cut and fill surfaces | Exact nominal locations and levels | Object insertion point(s) located and orientated exactly as nominal | 3D Boundary, Contour lines, spot levels and symbols | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 400 | 3D Digital terrain models of each calculated rock and soil cut and fill solids | Exact calculated locations and volumes | Object insertion point(s) located and orientated exactly as calculated | 3D solids of each rock and soil region | N/A |

Kerb

A kerb or curb is a vertical or sloping member provided along the edge of a pavement or shoulder to give strength and protect the edge of the pavement

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including the foundation and backing | Exact nominal dimension of each component | Object continuously aligned and orientated exactly as designed 3D alignment and cross fall of the parent pavement/object | 3D solid each component as a 3D continuous span | Dynamically updated according to the changes in the parent alignment, cross fall, and sections |
| 400 | 3D Object element including kerb, foundation, backing, ramping, and blinding | Exact construction size of each component parts according to joints | Object location and orientated exactly as designed setting out. | 3D solid of each segment, rendered by material texture according to the finish material | N/A |

Man-made slope

A slope (associated with the roadworks) formed by unnatural process and usually registered under a maintenance department.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | Slope boundary, 2D Contour lines and spot levels | Approximate dimension | Object insertion point(s) approximately located | 2D polygons, contour lines and symbols | N/A |
| 250 | 3D Digital terrain models including locations of exposed rock head, toe-line, soil nails, settlement markers | Exact Surveyed locations, extend and size | Object insertion point(s) is located at / derived from the onsite survey point(s) | 3D Boundary, Contour lines, spot levels and symbols | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 300 | 3D Digital terrain models including locations of exposed rock head and toe-line, settlement markers and berms in symbols / 3D polylines | Exact nominal locations, extend and size | Object insertion point(s) located and orientated exactly as nominal | 3D Boundary, Contour lines (5m major, 1m minor), spot levels, symbols and 3D polylines | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 400 | 3D Digital terrain models including benching and waterproofing layer and locations of exposed rock head, toe-line, soil nails, settlement markers, berm, soil/rock interfaces in symbols / 3D polylines | Exact setting out locations, extend and size | Same as 300 | 3D terrain model rendered by Material texture according to the finish material | N/A |

Pavement - Carriage way / Cycle Track / Footpath

A pavement is the durable surface material laid down on an area intended to sustain vehicular or foot traffic, such as a carriageway, cycle track or footpath.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including friction course, wearing course, base-course, road-base, sub-base, concrete slab, paving block | Exact nominal dimension of each component | Object continuously aligned and orientated exactly as designed 3D alignment and cross fall of the parent pavement/object | 3D solid of each component as a 3D continuous span | Dynamically updated according to the changes in the parent alignment, cross fall, and sections |
| 400 | 3D Object element type including friction course, wearing course, base-course, road-base, sub-base, concrete slab, paving block, openings (boxing out, man-holes), and joints | Exact construction size of each component parts according to joints | Object location and orientated exactly as designed setting out. | 3D solid of each segment, rendered by material texture according to the finish material | N/A |

Retaining Structure (Roadside retaining structures)

Retaining structures (associated with the roadworks) are relatively rigid walls used for supporting soil laterally so that it can be retained at different levels on the two sides next to the pavement (Carriageway, foot walk, cycle track)

The finish / skin wall should be separated from the wall 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object, including footing with size and shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including foundation and retaining wall elements. | Exact nominal dimension of each component | Object continuously aligned and orientated exactly as designed 3D alignment and cross fall of the parent pavement/object | 3D Solid of each component as a 3D continuous span | Dynamically updated according to the changes in the parent alignment, cross fall, and sections |
| 400 | 3D Object element including bearing depth, foundation, reinforcement, pile cut-off depths (if applicable) Openings modelled according to the requirements of the building services builders | Exact construction size of each component parts according to joints | Same as 300 | 3D Solid of each component rendered by Material texture according to the finish material | N/A |

Road Marking

Road marking is any kind of device or material that is used on a road surface in order to convey official information.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape | N/A |
| 300 | 3D Object element with size, pattern, and width; approximate thickness | Exact nominal dimension, size and pattern and approximate thickness of each mark / line according to the requirement of the Transport Department | Object insertion point(s) located, aligned, and orientated exactly as nominal parent alignments and road levels. | Overall size, shape and pattern of each mark / line as a 3D continuous span | N/A |
| 400 | Same as 300 | Size and shape requirement according to the AIR of the maintenance parties | Same as 300 | Rendered by Material texture according to the finish material required by the Transport Department | N/A |

Traffic Island - Other

A traffic island is a solid or painted object in a road that channels traffic.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape | N/A |
| 300 | 3D Object element with size, pattern and width; approximate thickness | Exact nominal dimension, size and pattern and approximate thickness of each mark / line according to the requirement of the Transport Department | Object insertion point(s) located, aligned, and orientated exactly as nominal parent alignments and road levels. | Overall size, shape and pattern of each mark / line as a 3D continuous span | N/A |
| 400 | Same as 300 | Size and shape requirement according to the AIR of the maintenance parties | Same as 300 | Rendered by Material texture according to the finish material required by the Transport Department | N/A |

Traffic Island - Refuge Island

A refuge island, also known as a pedestrian refuge or pedestrian island, is a small section of pavement or sidewalk, completely surrounded by asphalt or other road materials, where pedestrians can stop before finishing crossing a road.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including Pavement (friction course, wearing course, base-course, road-base, sub-base, concrete slab, paving block), Kerb (kerb, foundation and backing), and Signage (if applicable) | Exact nominal dimension of each component | Object continuously aligned and orientated exactly as designed 3D alignment and cross fall of the parent pavement / object | 3D solid of each component as a 3D continuous span | N/A |
| 400 | 3D Object element type including Pavement (friction course, wearing course, base-course, road-base, sub-base, concrete slab, paving block), Kerb (kerb, foundation and backing), and Signage (if applicable), openings (boxing out, man-holes), draw pits and joints | Exact construction size of each component parts according to joints | Object location and orientated exactly as designed setting out. | 3D solid of each segment, rendered by material texture according to the finish material | N/A |

Terrain (Roadworks)

Proposed topography is the mesh representation of entire finished road surfaces.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | Site boundary, 2D Contour lines and spot levels | Approximate dimension | Object insertion point(s) approximately located | 2D polygons, contour lines and symbols | N/A |
| 250 | 3D Digital terrain models including low points | Exact Surveyed locations and levels | Object insertion point(s) is located at / derived from the onsite survey point(s) | 3D Boundary, Contour lines, spot levels and symbols | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 300 | 3D Digital terrain models, including low points | Exact nominal locations and levels | Object insertion point(s) located and orientated exactly as nominal | 3D Boundary, Contour lines, spot levels and symbols | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 400 | N/A | N/A | N/A | N/A | N/A |

Sewerage Model

Summary of elements

The Sewerage Model include all the object elements of the Sewerage works proposed in to works. It is applicable to all services. The following object elements should be included:

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | Box Culvert (Sewerage) | FBP | ___ | 23-11 21 21 11 13 15 | Concrete Box Culverts |
| 2 | Chamber | FCH | ___ | 23-39 29 11 15 | Wastewater drainage pipes, couplings, collectors |
| 3 | Rising Main | FRM | ___ | 23-39 29 11 15 | Wastewater drainage pipes, couplings, collectors |
| 4 | Sewerage Manhole | FMH | Multiple* | 23-39 29 11 13 11 | Wastewater storm drain manhole |
| 5 | Sewerage Manhole Cover | FMC | Multiple* | 23-39 29 11 13 13 | Wastewater storm drain manhole cover |
| 6 | Sewerage Gravity Sewer | FWD | ___ | 23-39 29 11 15 | Wastewater drainage pipes, couplings, collectors |
| 7 | Special Manhole | FSH | Multiple* | 23-39 29 11 13 11 | Wastewater storm drain manhole |
| 8 | Terminal Manhole | FLH | Multiple* | 23-39 29 11 13 11 | Wastewater storm drain manhole |
| 9 | Tunnel (Sewerage) | FTP | ___ | 23-11 21 21 11 13 15 | Concrete Box Culverts |

Box Culvert (Sewerage) / Tunnel (Sewerage)

A culvert is a structure that channels sewerage water past an obstacle or to channel a subterranean waterway. Typically embedded to be surrounded by soil, a culvert may be made from a pipe, reinforced concrete, or other material

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including the outline of slab and box | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including top slab, bottom slab, wall, wing wall of inlet and outlet, foundation | Exact nominal dimension of each component | Object continuously aligned and orientated exactly as designed 3D alignment and cross fall of the parent object | 3D solid blocks of each component in a continuous span | Dynamically updated according to the changes in the parent alignment, cross fall, and sections |
| 400 | 3D Object element including top slab, bottom slab, wall, wing wall of inlet and outlet, foundation, openings, construction joints and expansion joints | Exact construction size of each component parts according to joints | Object location and orientated exactly as designed setting out. | Overall size and shape of each segment, rendered by material texture according to the finish material | N/A |

Chamber

A manhole (alternatively person hole, utility hole, maintenance hole, or sewer hole) is an opening to a confined space such as a shaft, utility vault, or large vessel. Manholes are often used as an access point for an underground public utility, allowing inspection, maintenance, and system upgrades.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid block | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid block | N/A |
| 300 | 3D Object element including wall, slab, frame and cover | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element including wall, slab, frame and cover and reinforcement | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Rising Main

Rising Main means a sewer through which foul sewage and/or surface water is pumped.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including pipes, all fittings and accessory with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal location, invert levels, and longitudinal fall. | 3D solid block of each component | The pipes should be modelled as connect systems |
| 400 | Specific components modelled, including pipes, all fittings and accessory according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces if applicable | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Sewerage Gravity Sewer

A gravity sewer is a conduit utilizing the energy resulting from a difference in elevation to remove unwanted sewage.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including pipes, all fittings and accessory with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal location, invert levels, and longitudinal fall. | 3D solid block of each component | The pipes should be modelled as connect systems |
| 400 | Specific components modelled, including pipes, all fittings and accessory according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces if applicable | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Sewerage Manhole and cover / Special Manhole / Terminal Manhole

A manhole (alternatively person hole, utility hole, maintenance hole, or sewer hole) is an opening to a confined space such as a shaft, utility vault, or large vessel. Manholes are often used as an access point for an underground public utility, allowing inspection, maintenance, and system upgrades.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including wall, slab, frame, and cover | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element including wall, slab, frame, cover, benching arm, step iron, stainless steel hook, chain, handrailing and reinforcement | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Specialised Systems Model

Summary of elements

The Specialised Systems Model includes all the object elements of the electrical, mechanical and control system required in the roads, tunnels and highways proposed in to works. It is applicable to all services. The following object elements should be included:

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | TCSS | SPQ | ___ | 23-39 11 15 17 | Traffic Signals |
| 2 | Tunnel Ventilation System | UFC | Multiple* | 23-29 27 00 | Fire Ventilation Equipment |

TCSS

Traffic Control & Surveillance Systems (TCSS) are a monitoring and control system for tunnel and highway operations. It usually includes LED signs, CCTV, backbone networking, emergency telephone services, public address systems and trunked mobile radio.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including LED signs, CCTV, networking facilities, emergency telephone booth, radio stations | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including LED signs, CCTV, networking facilities, emergency telephone booth, radio stations | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | Same as 400 | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | Rendered by Material texture according to the finish material | N/A |

Tunnel Ventilation System

Tunnel ventilation is a system where exhaust fans are located at one end of the house and two large openings are installed at the opposite end.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including exhaust fan and ventilation tunnel | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including exhaust fan, ventilation tunnel, major power supplies, status, and control panels | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | Same as 400 | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | Rendered by Material texture according to the finish material | N/A |

Stormwater Model

Summary of elements

The Stormwater Model includes all the object elements of storm water drains proposed in the works. It is applicable to all services. The following object elements should be included

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | Box Culvert | SBP | ___ | 23-11 21 21 11 13 15 | Concrete Box Culverts |
| 2 | Catchpit | SCH | ___ | 23-39 29 13 19 11 | Surface Water Catch Basins, Grates and Frames |
| 3 | Chamber | SBH | Multiple* | 23-39 29 13 19 17 | Surface water retention chamber |
| 4 | Decked Nullah | SDP | ___ | 23-11 21 21 15 | Channels |
| 5 | Inlet | SIH | ___ | 23-39 29 13 19 13 | Combination storm drain and underdrain inlets |
| 6 | Nullah | SNP | ___ | 23-11 21 21 15 | Channels |
| 7 | Outlet | SNF | ___ | 23-39 29 13 | Waste water subdrainage |
| 8 | Gully | GUL | Multiple* | 23-39 29 15 | Waste water channels, gullies, gratings, covers |
| 9 | Gully Pipe | SWD | ___ | 23-39 29 13 17 | Subgrade drains |
| 10 | Sand Trap | SPH | ___ | 23-39 29 11 17 | Wastewater Pipework Access Fittings |
| 11 | Stepped Channel | SSP | Multiple* | 23-11 21 21 15 | Channels |
| 12 | Stormwater manhole cover | SMC | Multiple* | 23-39 29 13 19 15 | Storm drainage manholes, frames and covers |
| 13 | Stormwater manhole | SMH | Multiple* | 23-39 29 13 19 15 | Storm drainage manholes, frames and covers |
| 14 | Stormwater pipe | SWD | ___ | 23-39 29 13 17 | Subgrade drains |
| 15 | Terminal manhole | SLH | Multiple* | 23-39 29 13 19 15 | Storm drainage manholes, frames and covers |
| 16 | U-Channel / Covered U-Channel | SUP | ___ | 23-11 21 21 15 | Channels |
| 17 | Concrete Cover for Channels | SUP | SUC | 23-39 29 15 | Waste Water Channels, Gullies, Gratings, Covers |
| 18 | Cast Iron Grating for Channels | SUP | SUG | 23-39 29 15 | Waste Water Channels, Gullies, Gratings, Covers |

Box Culvert

A culvert is a structure that channels stormwater past an obstacle or to channel a subterranean waterway. Typically embedded to be surrounded by soil, a culvert may be made from a pipe, reinforced concrete, or other material

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including the outline of slab and box | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including top slab, bottom slab, wall, wing wall of inlet and outlet, foundation | Exact nominal dimension of each component | Object continuously aligned and orientated exactly as designed 3D alignment and cross fall of the parent object | 3D solid blocks of each component in a continuous span | Dynamically updated according to the changes in the parent alignment, cross fall, and sections |
| 400 | 3D Object element including top slab, bottom slab, wall, wing wall of inlet and outlet, foundation, openings, construction joints and expansion joints | Exact construction size of each component parts according to joints | Object location and orientated exactly as designed setting out. | 3D solid blocks of each segment, rendered by material texture according to the finish material | N/A |

Catchpit

Catchpits are usually constructed to receive stormwater from stream courses or from slopes.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximated size of outline | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including slab, wall, pipe connector, cover and frame (if applicable), trap | Exact nominal dimension and size | Object insertion point(s) located and orientated exactly as nominal location, cover level and invert level. | 3D solid blocks of each component | Depth of the catchpit controlled by parameters related to the cover level and/or the pipe invert(s), specified in the applicable standard drawings |
| 400 | 3D Object element including slab, wall, pipe connector, cover and frame (if applicable), trap, reinforcement details, step iron, benching arm, stainless steel hook/chain, handrailing | Exact setting out location, dimension and orientation | Objects insertion point(s) located, orientated, and aligned exactly as setting out locations, cover levels and invert levels | 3D solid blocks of each component rendered by material texture according to the finish material | N/A |

Chamber

A manhole (alternatively person hole, utility hole, maintenance hole, or sewer hole) is an opening to a confined space such as a shaft, utility vault, or large vessel. Manholes are often used as an access point for an underground public utility, allowing inspection, maintenance, and system upgrades.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid block | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid block | N/A |
| 300 | 3D Object element including wall, slab, frame and cover | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element including wall, slab, frame and cover and reinforcement | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Decked Nullah / Nullah

Nullah refers to an open, usually concrete-lined channel designed to allow rapid drainage of storm precipitation or industrial wastewater from high ground, to prevent flooding of urbanised coastal areas.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including the outline of slab and box | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including top slab, bottom slab, wall, wing wall of inlet and outlet, foundation | Exact nominal dimension of each component | Object continuously aligned and orientated exactly as designed 3D alignment and cross fall of the parent object | 3D solid blocks of each component in a continuous span | Dynamically updated according to the changes in the parent alignment, cross fall, and sections |
| 400 | 3D Object element including top slab, bottom slab, wall, wing wall of inlet and outlet, foundation, openings, construction joints and expansion joints | Exact construction size of each component parts according to joints | Object location and orientated exactly as designed setting out. | 3D solid blocks of each segment, rendered by material texture according to the finish material | N/A |

Inlet

Inlet is an opening structure to allow incoming of stormwater to a stormwater system from a larger body of water.

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including the outline of slab and box | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including headwall, slab, orifice plate, trash rack, safety grate and foundation | Exact nominal dimension of each component | Object continuously aligned and orientated exactly as designed 3D alignment and cross fall of the parent object | 3D solid blocks of each component | N/A |
| 400 | 3D Object element including headwall, slab, orifice plate, trash rack, safety grate, foundation, construction joints and expansion joints | Exact construction size of each component parts according to joints | Object location and orientated exactly as designed setting out. | 3D solid blocks of each segment, rendered by material texture according to the finish material | N/A |

Outlet

Outlet is an opening structure to allow outgoing of stormwater from a stormwater system to a larger body of water.

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including the outline of slab and box | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including headwall, slab, orifice plate, trash rack, safety grate and foundation | Exact nominal dimension of each component | Object continuously aligned and orientated exactly as designed 3D alignment and cross fall of the parent object | 3D solid blocks of each component | N/A |
| 400 | 3D Object element including headwall, slab, orifice plate, trash rack, safety grate, foundation, construction joints and expansion joints | Exact construction size of each component parts according to joints | Object location and orientated exactly as designed setting out. | 3D solid blocks of each segment, rendered by material texture according to the finish material | N/A |

Gully

A gully is a fitting with a chamber which is designed to collect rainwater, wastewater, and groundwater, conveying it to an underground surface-water sewer.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including wall, slab, grating, weir, gully former and cover | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element including wall, slab, grating, weir, gully former, cover and reinforcement | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Gully Pipe

Drainage pipe that connects gully and stormwater manholes

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including pipes, all fittings and accessory with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal location, invert levels, and longitudinal fall. | 3D solid block of each component | The pipes should be modelled as connect systems |
| 400 | Specific components modelled, including pipes, all fittings and accessory according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces if applicable | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Sand Trap

Sand trap is a structure that is constructed to exclude the quantity of sand that is carried by water flowing in the channels or tunnels for power generation or irrigation or some other purposes. Sand trap is provided in the form of chambers that depends upon the discharge that is to be carried by the channel or tunnel.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including wall, slab, frame and cover | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element including wall, slab, frame, cover, benching arm, step iron, stainless steel hook, chain, handrailing and reinforcement | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Stepped Channel

Stepped surface channel used for draining rainwater away from driveways, patios and forecourt areas.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with bottom slab, channel wall, steps | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal locations, levels and alignment. | 3D solid block of each component | N/A |
| 400 | 3D Object element with bottom slab, channel wall, steps, location of joints | Exact nominal dimension of each component | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Stormwater Manhole and cover / Terminal Manhole Terminal manhole

A manhole (alternatively person hole, utility hole, maintenance hole, or sewer hole) is an opening to a confined space such as a shaft, utility vault, or large vessel. Manholes are often used as an access point for an underground public utility, allowing inspection, maintenance, and system upgrades.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including wall, slab, frame, and cover | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element including wall, slab, frame, cover, benching arm, step iron, stainless steel hook, chain, handrailing and reinforcement | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Stormwater pipe

Conduits used to carry off rainwater.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including pipes, all fittings and accessory with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal location, invert levels, and longitudinal fall. | 3D solid block of each component | The pipes should be modelled as connect systems |
| 400 | Specific components modelled, including pipes, all fittings and accessory according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces if applicable | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

U-Channel / Covered U-Channel

U-shape surface channel used for draining rainwater away from driveways, patios, and forecourt areas.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with bottom slab, channel wall, steps | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal locations, levels and alignment. | 3D solid block of each segment | N/A |
| 400 | 3D Object element with bottom slab, channel wall, steps, location of joints | Exact nominal dimension of each component | Same as 300 | 3D solid block of each segment rendered by Material texture according to the finish material | N/A |

Concrete Cover for Channels

.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact size/dimension | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal locations, levels and alignment. | 3D solid block of each component | N/A |
| 400 | Same as 300 | Exact nominal dimension of each component | Same as 300 | 3D solid block of each segment rendered by Material texture according to the finish material | N/A |

Cast Iron Grating for Channels

.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with exact size/dimension | Exact nominal dimension | Object insertion point(s) located and orientated exactly as nominal locations, levels and alignment. | 3D solid block of each component | N/A |
| 400 | Same as 300 | Exact nominal dimension of each component | Same as 300 | 3D solid block of each segment rendered by Material texture according to the finish material | N/A |

Water Supplies Model

Summary of elements

The Water Supplies Model includes all the object elements of the water supplies network proposed in the works. It is applicable to all services. The following object elements should be included

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | Fresh watermain | WSD | ___ | 23-39 27 11 | Water Utility pipeline equipment |
| 2 | Salt Watermain | SSD | ___ | 23-39 27 11 | Water Utility pipeline equipment |
| 3 | Chamber / Pump Pit | SBH | Multiple* | 23-39 27 11 | Water Utility pipeline equipment |
| 4 | Fittings | PPF | Multiple* | 23-39 27 11 | Water Utility pipeline equipment |
| 5 | Valve | PPA | VLV | 23-27 31 00 | Valves |
| 6 | Thrust Block | THB | ___ | 23-39 27 11 | Water Utility pipeline equipment |
| 7 | Fire Hydrant | FS_ | SFH | 23-29 25 13 | Fire Hydrants |

Fresh watermain / Salt water main

Pipe used to supply water services.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including pipes, all fittings and valve with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal location, invert levels, and longitudinal fall. | 3D solid block of each component | The pipes should be modelled as connect systems as far as practicable |
| 400 | Specific components modelled, including pipes, all fittings, valve and concrete blocks according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces if applicable | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Chamber / Sump Pit

A manhole (alternatively person hole, utility hole, maintenance hole, or sewer hole) is an opening to a confined space such as a shaft, utility vault, or large vessel. Manholes are often used as an access point for an underground public utility, allowing inspection, maintenance, and system upgrades.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid block | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid block | N/A |
| 300 | 3D Object element including wall, slab, frame and cover | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element including wall, slab, frame and cover and reinforcement | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Fittings

A fitting or adapter is used in pipe systems to connect straight sections of pipe or tube and adapt to different sizes or shapes. Fittings mainly include elbow, coupling, union, reducers, tee, cap, flange, flare, and plug

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal location, invert levels, and longitudinal fall. | 3D solid block of each component | The pipes should be modelled as connect systems as far as practicable |
| 400 | Specific components modelled, including fasteners, clean-out, and gasket according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces if applicable | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Valve

Valves stop (or regulate) the flow of supply water

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | The object should be modelled as connect systems as far as practicable |
| 400 | Specific components modelled, including fasteners, clean-out, and gasket according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces if applicable | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Thrust Block

A thrust block can be defined as a structural member that is installed at the rear of the launching shaft for the purpose of transmitting the horizontal force transmitted from the pipe jacks to the surrounding soil.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | 3D Object | Assumed / Typical size of component (s) | Located, orientated, and sloped as nominal and/or structural grid and reference elevation plane | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with all components | Exact nominal size, and dimension | Located, orientated, and sloped as nominal and/or structural grid and reference elevation plane | 3D solid block of each component | N/A |
| 400 | 3D Object element modelled with all components, including reinforcements, chamfer, etc. | Exact construction size of each component parts according to joints | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Fire Hydrant

A fire hydrant is a connection point by which firefighters can tap into a water supply. It is a component of active fire protection

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal location. | 3D solid block of each component | The object should be modelled as connect systems as far as practicable |
| 400 | Specific components modelled, according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces if applicable | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Gas Supply Model

Summary of elements

The Gas Supply Model includes all the object elements of the gas supply utilities proposed in the works. It is applicable to all services. The following object elements should be included

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | Gas Pipe | GAP | ___ | 23-39 25 11 | Gas Utility Pipeline Equipment |
| 2 | Gas Inspection pit | GIP | ___ | 23-17 11 27 23 | Man Hole Accesses |
| 3 | Gas Value | PPA | VLV | 23-27 31 00 | Valves |

Gas pipe

Pipe used to supply gas.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element, including pipes, all fittings and valve with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal location and levels | 3D solid block of each component | The pipes should be modelled as connect systems as far as practicable |
| 400 | Specific components modelled, including pipes, all fittings, valve and concrete blocks according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces if applicable | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Gas Inspection Pit

A manhole is an opening to a confined space such as a shaft, utility vault, or large vessel. Manholes are often used as an access point for an underground public utility, allowing inspection, maintenance, and system upgrades.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including wall, slab, frame and cover with exact size and dimension | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Gas Valve

Valves stop (or regulate) the flow of supply gas

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element with overall size and dimension. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal location, invert levels, and longitudinal fall. | 3D solid block of each component | The pipes should be modelled as connect systems as far as practicable |
| 400 | Specific components modelled, including fasteners and gasket according to the details of the manufacturer’s information to support fabrication and installation. Dynamic envelope to indicate the maintenance spaces if applicable | Size and shape requirement according to support the fabrication requirement. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | Same as 300 |

Electrical Power Supply Model

Summary of elements

The Electrical Power Supply Model includes all the object elements of the electrical power supply utilities proposed in the works. It is applicable to all services. The following object elements should be included

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | Electric Box | ELQ | Multiple* | 23-39 23 00 | Electrical Utility Equipment |
| 2 | Electric Inspection pit | EIP | ___ | 23-17 11 27 23 | Man Hole Accesses |
| 3 | Electric Manholes | EMH | ___ | 23-17 11 27 23 | Man Hole Accesses |
| 4 | Power Cables / lines | EPD | ___ | 23-35 33 25 | Electrical Wireway |
| 5 | Transformers | ETR | ___ | 23-39 23 11 | Electrical Transmission Equipment |

Electric Box

An electrical box is an enclosure housing electrical connection. It protects the electrical connections from the weather, as well as prevent people from accidental electric shocks

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object outline of the housing with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including housing, control panel and footing with outer size and dimension | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Electric Inspection pit / Electric Manhole

A manhole is an opening to a confined space such as a shaft, utility vault, or large vessel. Manholes are often used as an access point for an underground public utility, allowing inspection, maintenance, and system upgrades.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including wall, slab, frame and cover with exact size and dimension | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Power Cables / lines

Power distribution systems that rely on copper or aluminium busbars to channel electricity, while being enclosed in an housing for protection.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of cable containment | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including cable duct, and concrete surroundings. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal depths and size of each cable containment | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Transformers

A transformer is a passive component that transfers electrical energy from one electrical circuit to another circuit, or multiple circuits

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object outline of the housing and footings with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including housing, control panel and footing with outer size and dimension | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Telecommunication Model

Summary of elements

The Telecommunication Model includes all the object elements of the telecommunication utilities proposed in the works. It is applicable to all services. The following object elements should be included

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | Telecommunication lines | TED | ___ | 23-37 11 17 13 | Telecommunication Wireways |
| 2 | Tel Inspection pit | TIP | ___ | 23-17 11 27 23 | Man Hole Accesses |
| 3 | Tel Manholes / Drawpits | TMN | ___ | 23-17 11 27 23 | Man Hole Accesses |

Telecommunication lines

Telecommunication/networking systems that rely on copper or aluminium busbars to channel signals, while being enclosed in a housing for protection.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of cable containment | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including cable duct, and concrete surroundings. Dynamic envelope to indicate approximated maintenance spaces if applicable | Exact nominal depths and size of each cable containment | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Tel Inspection pit / Tel Manhole / Drawpits

A manhole is an opening to a confined space such as a shaft, utility vault, or large vessel. Manholes are often used as an access point for an underground public utility, allowing inspection, maintenance, and system upgrades.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | 3D Object element including wall, slab, frame and cover with exact size and dimension | Exact nominal dimension of each component | Object insertion point(s) located and orientated exactly as nominal. | 3D solid block of each component | N/A |
| 400 | 3D Object element with components to be modelled according to AIR of the maintenance party | Size and shape requirement according to the AIR of maintenance party. | Same as 300 | 3D solid block of each component rendered by Material texture according to the finish material | N/A |

Existing Condition Model

Disciplinary Models

List of disciplinary models

Existing Condition Model include the 4 disciplinary models:

| Project Model | 1st Level Federation | 2nd level Federation (Disciplinary Models) | Description |
| --- | --- | --- | --- |
| Project Information Model | Existing Condition Model | Existing Structure | All existing above ground and underground structures of the Service area |
|  |  | Existing Site Topography | Topology (Digital Terrain Model) of the site area, include GI data and bore hole data |
|  |  | Existing Street Furniture | Relevant existing planters, traffic sign, road markings, railings, street lighting, barriers, kerbs etc |
|  |  | Existing Underground Utilities | All existing underground utilities systems of the Service area |

Existing Structure model

List of Common Objects Elements

The following object elements are classified under the Existing Structure model:

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | Culverts | TCU | Multiple* | 23-11 21 21 11 13 | Concrete Culverts |
| 2 | Existing Building Structure Mass Model | MAS | MAS | 23-19 29 00 | Complete Buildings |
| 3 | Existing Bridge and Tunnel Structure Mass Model | MAS | MAS | 23-39 13 00 | Tunnels and Bridges |
| 4 | Existing Retaining Structure | SRW | ___ | 23-11 17 00 | Retention Structure |
| 5 | Foundation (pile) | SFD | FPL | 23-13 29 11 | Foundation piles |
| 6 | Foundation (pile cap) | SFD | FCA | 23-13 29 11 11 21 | Pile Caps |
| 7 | Foundation (ground beam) | SFD | FBM | 23-13 29 15 13 | Grade Beams (shallow foundations) |
| 8 | Foundation (Other) | SFD | Multiple* | 23-13 29 00 | Foundations |
| 9 | Footing | SFD | OTR | 23-13 29 15 15 | Strip Foundation Blocks (shallow foundations) |
| 10 | Mass Concrete Infill (including No-fines Concrete) | GSM | MAC | 23-13 15 11 | Concretes |
| 11 | Retaining Wall on Slope | SUS | RET | 23-11 17 13 | Retaining Walls |
| 12 | Concrete Beam,Steel Beam, Hanger | BEM | ___ | 23-13 35 11 | Structural Frames |
| 13 | Structural Wall | SWL | ___ | 23-13 31 11 | Structural Concrete |
| 14 | Structural Columns | SCL | Multiple* | 23-13 35 11 | Structural Frames |
| 15 | Floor, Slab | SLA | ___ | 23-13 31 11 | Structural Concrete |
| 16 | Steel access ladder | GSM | SAL | 23-17 23 15 | Ladders |
| 17 | Steel Handrailing (to Maintenance Stairway on Slope) | GSM | SRL | 23-17 25 13 | Handrails |
| 18 | Steel Staircase (Maintenance Stairway on Slope) | GSM | SST | 23-17 23 17 11 | Stair component products |
| 19 | Steel Gate | GSM | SGA | 23-17 23 17 11 21 | Stair Barrier Gates |
| 20 | Maintenance Access (incl. Concrete Stairway/Berm) | GSM | CMS | 23-13 31 17 | Concrete Formwork |
| 21 | Diaphragm Wall | EXL | DIW | 23-11 17 00 | Retention Structures |
| 22 | Lining | UCL | Multiple* | 23-39 13 11 15 | Tunnel Lining |
| 23 | Tunnel Structure Segments | TIS | Multiple* | 23-39 13 11 | Tunnels |
| 24 | Temp. Support Structure | EXL | Multiple* | 23-11 11 00 | Ground Anchorages |
| 25 | Chain Link Fence on Slope | GSM | CLF | 23-11 25 19 19 | Chain Link Metal Fences |

Culverts

A culvert is a structure that channels stormwater past an obstacle or to channel a subterranean waterway. Typically embedded to be surrounded by soil, a culvert may be made from a pipe, reinforced concrete, or other material

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including the outline of slab and box | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Existing Building Structure Mass Model

Exterior of the existing structure in 3-dimension solid blocks

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object, including outline mass with size and shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | 3D Solid of each component rendered by Material texture | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Existing Bridge and Tunnel Structure Mass Model

Exterior of the existing bridge and tunnel in 3-dimension solid blocks

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object, including outline mass with size and shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | 3D Solid of each component rendered by Material texture | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Existing Retaining Structure

Retaining structure are relatively rigid walls used for supporting soil laterally so that it can be retained at different levels on the two sides.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object, including footing with size and shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | 3D Solid of each component rendered by Material texture | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Structural Foundation – Pile / Pile Cap / Foundation / Footings

Foundation refers to a series of columns constructed or inserted into the ground to transmit loads to a lower level of subsoil. Foundation/Ground beam refers to reinforce concrete beams for supporting walls and joists at or near ground level. Footing refers to foundation unit constructed in brick work, stone masonry or concrete under the base of a wall column for the purpose of distributing the load over a large area.

The finish should be separated from the Beam 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | 3D Object with all foundation components | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Foundation (Other)

Foundation (Other) is classified as other foundation which is not categorised as pile/pile cap/footings.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Mass concrete fill

Mass concrete fill refers to any volume of concrete with dimensions large enough to required that measures be taken to cope with the generation of heat from hydration of cement and attendant volume change to minimize cracking.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Retaining Wall on Slope

Retaining walls are relatively rigid walls used for supporting soil laterally so that it can be retained at different levels on the two sides. Retaining walls are structures designed to restrain soil to a slope that it would not naturally keep to.

The finish / skin wall should be separated from the wall 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object, including footing with size and shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Concrete Beam,Steel Beam, Hanger

Beams refer to structural elements that resist loads applied laterally to their axis.

The finish should be separated from the Beam 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Structural Wall

Structure wall refers to a load bearing wall or a wall that carries load in addition to its own load.

The finish should be separated from the wall 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall thickness (including the finish) | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Structural Columns

Column refers to vertical structure intended to transfer a compressive load.

The finish should be separated from the Beam 3D Object elements at LOD-G 400.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall shape | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Floor, Slab

Slab refers concrete structural element used to create horizontal surfaces such as berms and decks.

The finish should be separated from the slab at LOD-G 300 or above.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Steel access ladder

Access ladder is often used to describe the maintenance access ladder on the slope.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object | Assumed / Typical size of component(s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Steel Handrailing (to Maintenance Stairway on Slope)

Steel handrailing are railings designed to minimize fall risk at a work site where there is high fall risk.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object including post, rail, vertical members | Assumed / Typical size of component(s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Steel Staircase

Access ladder is often used to describe the maintenance access ladder on the slope.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object | Assumed / Typical size of component(s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Steel Gate

Steel Gate refers to a hinged barrier that can be used to secure an opening in a wall, fence and hedge.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Maintenance Access (including Concrete stairway / Berms)

Maintenance access include the concrete stairway and berms constructed to allow the inspection and maintenance of the man-made slope features.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object including stairway and berms with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Diaphragm Wall

Diaphragm wall is a continuous wall constructed in ground in to facilitate certain construction activities, such as: As a retaining wall. As a cut-off provision to support deep excavation. As the final wall for basement or other underground structure (e.g. tunnel and shaft).

Geometrical Requirement\

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall thickness | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Lining

Tunnel lining is used in both trenchless construction and rehabilitation to maintain the shape of the tunnel and to shield whatever might pass through it.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall thickness | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | Dynamically updated according to the changes in the parent alignment, cross fall, and sections |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Tunnel Structure segments

Segmental concrete tunnel liners are prefabricated structural support pieces that are combined to form a complete tube along tunnel passageways. The segments are created in manufacturing plants for tunnel projects

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object with overall thickness | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | Dynamically updated according to the changes in the alignment, cross fall, and sections |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Temp. Support Structure / ELS

Excavation and lateral support works (ELS works) is normally called for in the construction of pile caps, basement, underground drainage and tunnel works.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbol | N/A |
| 200 | 3D Object with size and shape of the supports | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | N/A | N/A | N/A | N/A | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Chain Link Fence on Slope

      - A Chain Link Fence On Slope is a structure that encloses a slope, and is usually constructed from posts that are connected by netting.

- 5.2.23.2   Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Existing Site Topography model

List of Common Objects Elements

The following object elements are classified under the Existing Site Topography model:

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | Terrain (Bathymetry) | DTM | BAE | 14-34 17 99 | Other Land and Water Topographical Transitions |
| 2 | Land Lot Boundary | LOT | LAN | 14-37 11 11 | Parcel |
| 3 | Terrain (Rock head) | DTM | ROK | 14-31 99 00 | Other Space Designations to facilitate design and construction |
| 4 | Terrain (Shoreline / Beach Finish Level) | DTM | BFL | 12-34 17 24 | Shoreline |
| 5 | Terrain (Site Topology) | DTM | TOP | 14-37 11 14 | Site |
| 6 | Tree / Vegetation | LTP | Multiple* | 23-11 27 19 13 | Trees |
| 7 | Man-made slope (Registered) | GSM | MMS | 14-34 11 00 | Slopped Topographical spaces |
| 8 | Natural slope | GSM | NSL | 14-34 11 00 | Slopped Topographical spaces |
| 9 | Site/Slope Boundary Polygon | LOT | SIT | 14-37 11 14 | Site |
| 10 | Borehole | GEO | BOH | N/A | N/A |
| 11 | Fill | GEO | FIL | N/A | N/A |
| 12 | Marine deposits | GEO | MAD | N/A | N/A |
| 13 | Alluvium | GEO | ALL | N/A | N/A |
| 14 | Colluvium | GEO | COL | N/A | N/A |
| 15 | Residual Soil | GEO | RES | N/A | N/A |
| 16 | Completely decomposed rock | GEO | CDR | N/A | N/A |
| 17 | Highly decomposed rock | GEO | HDR | N/A | N/A |
| 18 | Bedrock | GEO | BED | N/A | N/A |
| 19 | Compacted Fill | GEO | COF | N/A | N/A |

Terrain (Bathymetry)

Existing Bathymetry is the underwater topography to show the proposed water profile before the works.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | Boundary, 2D Contour lines and spot levels | Assumed / Typical size of component(s) | Object insertion point(s) approximately located | 2D lines and symbols | N/A |
| 250 | 3D Digital terrain models | Exact Surveyed locations and depths | Object insertion point(s) is located at / derived from the onsite survey point(s) | Boundary, Contour lines and spot levels | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Land Lot Boundary

Land lot boundary refers to the land lot ownership information created according to the record provided by the Lands Department.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 2D polylines | Assumed / Typical size of component(s) | Object insertion point(s) approximately located | N/A | N/A |
| 250 | 3D polylines | Exact Surveyed locations and levels | Object insertion point(s) is located at / derived from the onsite survey point(s) | Closed polygons | N/A |
| 300 | 3D polygons | Exact determinated locations and levels | Object insertion point(s) located and orientated exactly as determined by the surveyor | Closed polygons | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Terrain (Rock head)

Rock head surface determined from the site investigation records

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Material’s extend boundary, 2D Contour lines and spot levels | Approximate location | Object insertion point(s) approximately located | 2D polygons, contour lines and symbols | N/A |
| 200 | Object including stratigraphy with decomposed rock and bed rock, location of the boreholes | Approximate location, depth, and thickness | Object insertion point(s) approximately located | 3D Boundary, Contour lines, spot levels and symbols | N/A |
| 250 | Same as 200 | Same as 200 | Object modelled according to the borehole logs results or AGS from ground investigation works | Same as 200 | Same as 200 |
| 300 | 3D Digital terrain models, including inferred surfaces boundary of decomposed rocks and bed rock, location of boreholes, as well as ground water profile for design purpose | Exact inferred location, depth, and thickness of all components | 3D surfaces modelled according to the borehole logs results or AGS from ground investigation works | 3D Boundary, Contour lines, spot levels and 3D objects | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 400 | N/A | N/A | N/A | N/A | N/A |

Terrain (Shoreline / Beach Finish Level)

Beaches are dynamic environments which for the existing buffer between land and sea. The Beach finish level refers to the terrain surface from the foreshore (surf zone) to the backshore (usually above the high tide mark).

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | Boundary, 2D Contour lines and spot levels | Assumed / Typical size of component(s) | Object insertion point(s) approximately located | 2D lines and symbols | N/A |
| 250 | 3D Digital terrain models | Exact Surveyed locations and levels | Object insertion point(s) is located at / derived from the onsite survey point(s) | Boundary, Contour lines and spot levels | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 300 | 3D Digital terrain models | Exact nominal locations and levels | Object insertion point(s) located and orientated exactly as nominal | Boundary, Contour lines and spot levels | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 400 | N/A | N/A | N/A | N/A | N/A |

Terrain (Site Topology)

Proposed topography is the mesh representation of entire existing site surfaces.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | Site boundary, 2D Contour lines and spot levels | Approximate dimension | Object insertion point(s) approximately located | 2D polygons, contour lines and symbols | N/A |
| 250 | 3D Digital terrain models including slope and platforms | Exact Surveyed locations and levels | Object insertion point(s) is located at / derived from the onsite survey point(s) | 3D Boundary, Contour lines, spot levels and symbols | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 300 | 3D Digital terrain models, including slope, platforms and breaklines of man-made features | Exact nominal locations and levels | Object insertion point(s) located and orientated exactly as nominal | 3D Boundary, Contour lines, spot levels, symbols and 3D breaklines | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 400 | N/A | N/A | N/A | N/A | N/A |

Tree / Vegetation

Plant tree is a perennial plant with an elongated stem, or trunk, supporting branches and leaves in the existing site area.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with diameter, height and spread | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Man-made slope (Registered Slope)

A slope formed by unnatural process and usually registered under a maintenance department.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | Slope boundary, 2D Contour lines and spot levels | Approximate dimension | Object insertion point(s) approximately located | 2D polygons, contour lines and symbols | N/A |
| 250 | 3D Digital terrain models including locations of exposed rock head, toe-line, soil nails, settlement markers | Exact Surveyed locations, extend and size | Object insertion point(s) is located at / derived from the onsite survey point(s) | 3D Boundary, Contour lines, spot levels and symbols | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 300 | 3D Digital terrain models including locations of exposed rock head, toe-line, soil nails, settlement markers, berm in 3D polylines / symbols | Exact nominal locations, extend and size | Object insertion point(s) located and orientated exactly as nominal | 3D Boundary, Contour lines (5m major, 1m minor), spot levels, 3D polylines and symbols | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 400 | N/A | N/A | N/A | N/A | N/A |

Natural slope

A slope formed by natural process.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | Slope boundary, 2D Contour lines and spot levels | Approximate dimension | Object insertion point(s) approximately located | 2D polygons, contour lines and symbols | N/A |
| 250 | 3D Digital terrain models including locations of man-made features, large boulders, rock outcrop, tree locations | Exact Surveyed locations, extend and size | Object insertion point(s) is located at / derived from the onsite survey point(s) | 3D Boundary, Contour lines, spot levels and symbols | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Site/Slope Boundary Polygon

Site boundary means the outermost perimeter of the slope.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 2D polylines | Assumed / Typical size of component(s) | Object insertion point(s) approximately located | N/A | N/A |
| 250 | 3D polylines | Exact Surveyed locations and levels | Object insertion point(s) is located at / derived from the onsite survey point(s) | Closed polygons | N/A |
| 300 | 3D polygons | Exact nominal locations and levels | Object insertion point(s) located and orientated exactly as nominal | Closed polygons | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Borehole

A borehole is a narrow shaft bored in the ground, either vertically or horizontally to support site investigation.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size | Assumed / Typical size of component (s) | Object insertion point(s) approximately located and orientated | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Assumed size and location | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | Same as 200 | Exact nominal size and location | Object insertion point(s) located and orientated exactly as nominal locations and levels | 3D solid block of each component | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Geology – Fill / Marine Deposit / Alluvium / Colluvium / Residual Soil / Completely decomposed rock / Highly decomposed rock / Bedrock / Compact fill

Various layer of material that covers the earth's surface.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Material’s extend boundary, 2D Contour lines and spot levels | Approximate location | Object insertion point(s) approximately located | 2D polygons, contour lines and symbols | N/A |
| 200 | 3D object including stratigraphy, with layers of fill, transported soils, decomposed rock and bed rock, location of the boreholes | Approximate location, depth and thickness | Object insertion point(s) approximately located | 3D Boundary, Contour lines, spot levels and symbols | N/A |
| 250 | Same as 200 | Same as 200 | Object modelled according to the borehole logs results or AGS from ground investigation works | Same as 200 | Same as 200 |
| 300 | 3D Digital terrain models, including inferred surfaces boundary of soils and rocks, location of boreholes, as well as ground water profile for design purpose | Exact inferred location, depth, and thickness of all components | 3D surfaces modelled according to the borehole logs results or AGS from ground investigation works | 3D Boundary, Contour lines, spot levels and 3D objects of surfaces | Support earthwork calculation. Allow users to switch object appearance dynamically by configuration |
| 400 | N/A | N/A | N/A | N/A | N/A |

Existing Street Furniture model

List of Common Objects Elements

The following object elements are classified under the Existing Street Furniture model:

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | Barrier | FBF | Multiple* | 23-39 11 11 15 | Traffic Barrier |
| 2 | Bollard at Road | FBL | Multiple* | 23-39 11 19 | Bollards |
| 3 | Catchpit | SCH | ___ | 23-39 29 13 19 11 | Surface Water Catch Basins, Grates and Frames |
| 4 | Pavement | RCT | PAV | 23-11 21 17 | Paving blocks |
| 4a | Carriage way | RCW | Multiple* | 23-11 21 13 | Roadways |
| 4b | Cycle Track | RCT | Multiple* | 23-11 21 00 | Pavements |
| 4c | Foot path | RFW | Multiple* | 23-11 21 00 | Pavements |
| 5 | E&M Pit / Cable Draw Pit | ECD | ___ | 23-17 11 27 23 | Manhole Accesses |
| 6 | Bus Shelter / Terminus | FST | Multiple* | 23-19 29 13 | Covers and Shelters |
| 7 | Electric Pole | SIT | ETP | 23-39 23 11 11 | Electrical Utility Poles |
| 8 | Electric Transformer | ETR | Multiple* | 23-35 13 11 | Current Transformers |
| 9 | Exterior Benches / Seating | SIT | SIT | 23-11 29 13 | Exterior Seating |
| 10 | Fence | FBF | Multiple* | 23-11 25 19 | Fences |
| 11 | Fire Hydrant | FS_ | SFH | 23-29 25 13 | Fire Hydrants |
| 12 | Fire Valve | PPA | VLV | 23-29 25 13 | Fire Hydrants |
| 13 | Gate | FEG | Multiple* | 23-11 25 15 | Perimeter Gates |
| 14 | Grating | GRA | Multiple* | 23-39 29 15 | Waste Water Channels, Gullies, Gratings, Covers |
| 15 | Guard Rails/ Railings | FRA | Multiple* | 23-17 25 11 | Guardrails |
| 16 | Gully | GUL | Multiple* | 23-39 29 15 | Waste Water Channels, Gullies, Gratings, Covers |
| 17 | Hoarding | EHO | Multiple* | 23-11 25 13 | Perimeter Walls |
| 18 | Inspection Chamber | ECD | ___ | 23-17 11 27 23 | Manhole Accesses |
| 19 | Lamp Post / Lighting column/ Lighting pole | LLP | Multiple* | 23-35 47 11 21 23 | Street and Roadway Lighting Fixture |
| 20 | Planter | LTW | SHS | 23-11 27 13 | Planting Accessories |
| 21 | Road Kerbs | RKB | Multiple* | 23-39 11 11 19 | Roadways Curbs |
| 22 | Road Marking | RMK | Multiple* | 23-39 11 15 11 | Roadway Surface Markings |  |
| 23 | Sign Pole / Traffic sign | FTS | Multiple* | 23-39 11 13 | Roadway Signage |
| 24 | Staircase | TTE | ___ | 23-17 23 17 11 | Stair Component Products |
| 25 | Street Name Plate | FNP | Multiple* | 23-11 29 33 | Exterior Directional Signs |
| 26 | Telephone Pole | SIT | TEP | 23-37 11 17 | Communication Wireways |
| 27 | Traffic Light | FTL | Multiple* | 23-39 11 15 17 | Traffic Signals |
| 28 | U-Channel/ Covered U-Channels | SUP | ___ | 23-11 21 21 15 | Channels |
| 29 | Sand Trap | SPH | ___ | 23-39 29 11 17 | Wastewater Pipework Access Fittings |
| 30 | Storm Water Manhole cover | SMC | Multiple* | 23-39 29 13 19 15 | Storm Drainage Manholes, Frames, and Covers |
| 31 | Baffle | GSM | BAF | 23-11 17 00 | Retention Structures |
| 32 | Raking Drains | GSM | RKD | 23-11 13 11 23 | Field Drainage Blocks |
| 33 | Soil Nail | GSM | SON | 23-11 11 13 11 | Earth Reinforcement Soil nails |
| 34 | Stepped Channel | SSP | Multiple* | 23-11 21 21 15 | Channels |
| 35 | Flexible Barrier | GSM | FBR | 23-11 19 00 | Slide and Avalanche Protection |
| 36 | Rigid barrier | GSM | RBR | 23-11 17 00 | Retention Structures |
| 37 | Hard Surface Cover (e.g. shotcrete) | GSM | HSC | 23-11 13 11 | Soil stabilization products |
| 38 | Recreation area/facilities | LOT | REA | 23-11 31 00 | Athletic and Recreational Surfaces |
| 39 | Vegetation Surface Cover | GSM | VSC | 23-11 13 11 | Soil stabilization products |
| 40 | Tree Ring | GSM | TRR | 23-11 27 13 | Planting Accessories |
| 41 | Erosion Control Mat | GSM | ECM | 23-11 15 11 21 | Sheeting Synthetic Erosion Controls |
| 42 | Wire Mesh | GSM | WRM | 23-13 19 11 15 | Mesh for General Use |
| 43 | Ground Anchors | GSM | GAR | 23-11 11 11 | Retaining Stabilizing Ground Anchors |
| 44 | Concrete Cover for Channels | SUP | SUC | 23-39 29 15 | Waste Water Channels, Gullies, Gratings, Covers |
| 45 | Cast Iron Grating for Channels | SUP | SUG | 23-39 29 15 | Waste Water Channels, Gullies, Gratings, Covers |

Barrier

Barrier refers to linear object elements installed in the works area to control road traffic to prevent automotive vehicles from colliding or crashing into pedestrians and structures.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Lines and Symbols | N/A |
| 200 | 3D Object post including barrier, footings, and foundation | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Bollard at Road

Bollards refers to posts installed in the works area to control road traffic prevent automotive vehicles from colliding or crashing into pedestrians and structures.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Catchpit

Catchpits are usually constructed to receive stormwater from stream courses or from slopes.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including slab, wall, pipe connector, cover, and frame (if applicable), | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Pavement - Carriage way / Cycle Track / Footpath

A pavement is the durable surface material laid down on an area intended to sustain vehicular or foot traffic, such as a carriageway, cycle track or footpath.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object type | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | 3D Object including road surfacing and openings | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

E&M Pit / Cable Draw Pit

A draw pit is the manhole of the ducting world. They are chambers situated along a line of underground ducting and facilitate access to the ducting to allow for initial installation, maintenance, and monitoring.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Bus Shelter / Terminus

A bus shelter is a roofed structure for people to wait under at a bus stop.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Lines and Symbols | N/A |
| 200 | 3D Object with size and shape of the steel structure/structure, foundation, panels | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Electric Pole

Electric pole refer to one of a series of large, upright poles used to support electric cables.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object including post, footings, and foundations | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Electric Transformer

Transformer is a passive component that transfers electrical energy from one electrical circuit to another circuit, or multiple circuits

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object outline of the housing and footings with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Exterior Benches / Seating

A long seat for several persons in public space.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Fence

A fence is a structure that encloses an area, typically outdoors, and is usually constructed from posts that are connected by boards, wire, rails or netting. A fence differs from a wall in not having a solid foundation along its whole length.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object including post, strut, horizontal railing | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | 3D solid block of each component; barrier fence as a semi-transparent sheet | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Fire Hydrant

A fire hydrant is a connection point by which firefighters can tap into a water supply. It is a component of active fire protection

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Fire Valve

A fire valves stop (or regulate) the flow of supply water of fire hydrant

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Gate

refers to a hinged barrier that can be used to secure an opening in a wall, fence and hedge.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Grating

A grating / manhole cover covering a drain (as illustrated) can be a collection of iron bars (the identical, elongated elements) held together (to ensure the bars are parallel and regularly spaced) by a lighter iron frame. Gratings over drains and air vents are used as filters, to block movement of large particles (such as leaves) and to allow movement of small particles (such as water or air).

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including all iron bars | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Guard Rails/ Railings

Railing, handrail, or guardrails refers to a fence or barrier made of rails to prevent people from falling off.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including railings, bracing, supports, posts and footing | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Gully

A gully is a fitting with a chamber which is designed to collect rainwater, wastewater and groundwater, conveying it to an underground surface-water sewer.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, and cover with approximate size/dimension | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Hoarding

A hoarding is defined as a barrier structure between a construction site and the public.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object including post, beams, bracing, enclosures, and footings | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | 3D solid block of each component | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Inspection Chamber

A manhole is an opening to a confined space such as a shaft, utility vault, or large vessel to support inspection works.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, and cover | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Lamp Post / Lighting column/ Lighting pole

A light fixture, light fitting, or luminaire is an electrical device containing an electric lamp that provides illumination.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including lights/lamp, post, foundation | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Planter

Planter refers to the concrete structure for the planting of vegetations.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Road Marking

Road marking is any kind of device or material that is used on a road surface in order to convey official information.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Road Kerb

Kerb or curb is a vertical or sloping member provided along the edge of a pavement or shoulder to give strength and protect the edge of the pavement

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object type | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Sign Pole / Traffic sign

Traffic signs or road signs are signs erected at the side of or above roads or highways to give instructions or provide information to road users.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Staircase

Staircases refer to a set of steps leading from one level to another.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object including steps and landings | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Street Name Plate

Traffic signs or road signs are signs erected at the side of or above roads to describe the road name.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object with size and width, road name text | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Telephone Pole

Telephone pole refer to one of a series of large, upright poles used to support telephone wires.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object including post, footings and foundations | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Traffic Light

Traffic lights, traffic signals, stoplights or robots are signalling devices positioned at road intersections, pedestrian crossings, and other locations to control flows of traffic.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols | N/A |
| 200 | 3D Object including traffic lights, housing, post, and footings | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

U-Channel/Covered U-Channels

U-shape surface channel used for draining rainwater away from driveways, patios and forecourt areas.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with bottom slab, channel wall | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Surveyed size of component (s) | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Sand Trap

Sand trap is a structure that is constructed to exclude the quantity of sand that is carried by water flowing in the channels or tunnels for power generation or irrigation or some other purposes. Sand trap is provided in the form of chambers that depends upon the discharge that is to be carried by the channel or tunnel.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Stormwater Manhole and cover / Terminal Manhole Terminal manhole

A manhole (alternatively person hole, utility hole, maintenance hole, or sewer hole) is an opening to a confined space such as a shaft, utility vault, or large vessel. Manholes are often used as an access point for an underground public utility, allowing inspection, maintenance, and system upgrades.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Baffle

An element for natural terrain hazard mitigation measure constructed at the uphill of the proposed rigid/flexible barrier to serve as prescribed measures for enhancing robustness of the mitigation scheme.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | N/A | N/A | N/A | N/A | N/A |
| 200 | 3D Object including post, footings and foundations | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Raking Drains

Raking drains are used in lowering the groundwater level and relieving the groundwater pressure at depth or used as contingency measures to cater for uncertainties in the groundwater conditions and possible adverse effects of subsurface seepage (e.g. from leaking services) on slope stability.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Soil Nail

Soil nailing is a remedial construction measure to treat unstable natural soil slopes or as a construction technique that allows the safe over-steepening of new or existing soil slopes.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension/orientation | Assumed / Typical size of component (s) | Object insertion point(s) approximately located and orientated | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Assumed size | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Stepped Channel

Stepped surface channel used for draining rainwater away from driveways, patios and forecourt areas.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with bottom slab, channel wall | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Surveyed size of component (s) | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Flexible barrier net, Base plate, shackle for net, round clip, Post, Footing, shackle on post, running wheel, cable rope, rope clip, braking element

A flexible barriers net is used to trap stream loads from a traversing a stream course.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including post and barrier with approximated size | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Rigid barrier

Rigid debris-resisting barriers are a key example of risk mitigation measures, which catch landslide debris and prevent them from reaching populated or developed areas.

Information Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Hard surface cover

Hand surface cover refers to material (e.g. shotcrete) applies to the slope surface to protect it from landslip.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D hatches | Assumed size | Symbol insertion point(s) approximately located | 2D hatches | N/A |
| 200 | 3D Object with extend and thickness | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Recreation area/facilities

Recreation area refers to surface/area designated to the recreation area or facilities.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D hatches | Assumed size | Symbol insertion point(s) approximately located | 2D hatches | N/A |
| 200 | 3D polylines | Exact Surveyed locations and levels | Object insertion point(s) approximately located | Closed polygons | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Closed polygons | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Vegetation surface cover

Vegetation surface cover refers to vegetation material applies to the slope surface to protect it from landslip.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D hatches | Assumed size | Symbol insertion point(s) approximately located | 2D hatches | N/A |
| 200 | 3D Object with extend and thickness | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid block | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid block | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Tree Ring

Tree ring is an opening in a supporting toe  slab to allow tree roots to extend into the ground.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size | Assumed / Typical size of component (s) | Object insertion point(s) approximately located and orientated | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Assumed size | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Erosion Control Mat

Erosion control mat is a measure to apply directly on exposed soil surface to improve its resistance against soil erosion by slowing down the flow velocity, and by cushioning the impact of raindrops loosening the soil.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with extend and thickness | Assumed / Typical size of component (s) | Object insertion point(s) approximately located and orientated | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Assumed size | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Wire Mesh

Wire mesh is a measure to help prevent or minimise shallow or washout type of slope failures.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with extend and thickness | Assumed / Typical size of component (s) | Object insertion point(s) approximately located and orientated | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Assumed size | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Ground anchors

.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size | Assumed / Typical size of component (s) | Object insertion point(s) approximately located and orientated | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Assumed size | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Concrete Cover for Channels

Concrete cover for channels is a cover made by concrete to cover up the drainage channels.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Cast Iron Grating for Channels

Cast iron grating for channels is a cover made by cast iron grating to cover up the drainage channels.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Existing Underground Utilities Models

Summary of Elements

The following object elements are classified under the Existing Underground Utilities model:

| ITEM | OBJECT ELEMENT | CAT CODE | SUBCAT CODE | OMNICLASS NO | OMNICLASS DESCRIPTION |
| --- | --- | --- | --- | --- | --- |
| 1 | Manhole | UMH | ___ | 23-17 11 27 23 | Manhole Accesses |
| 2 | Manhole Foul Water / Sewer | FMH | Multiple* | 23-39 29 11 13 | Waste Water Storm Drain |
| 3 | Manhole Storm Water | SMH | Multiple* | 23-39 29 13 19 15 | Storm Drainage manholes, frames and covers |
| 4 | Manhole Telephone | MTL | ___ | 23-17 11 27 23 | Manhole Accesses |
| 5 | Valve Gas | PPA | VLV | 23-39 25 11 | Gas Utility Pipeline Equipment |
| 6 | Valve Waterworks | PPA | VLV | 23-39 27 11 | Water Utility Pipeline Equipment |
| 7 | Power cables / lines | EPD | ___ | 23-35 33 25 | Electrical Wireway |
| 8 | Gas pipe | GAP | ___ | 23-39 25 11 | Gas Utility Pipeline Equipment |
| 9 | Telecommunication lines | TED | ___ | 23-37 11 17 | Communication Wireways |
| 10 | Sewage Pipe | FWD | ___ | 23-39 29 11 15 | Waste Water Drainage pipe, couplings, Collectors |
| 11 | Storm Water pipe | SWD | ___ | 23-39 29 13 19 11 | Surface Water Catch Basins, Grates and Frames |
| 12 | Fresh Watermain | WSD | ___ | 23-39 27 11 | Water Utility Pipeline Equipment |
| 13 | Salt Watermain | SSD | ___ | 23-39 27 11 | Water Utility pipeline equipment |
| 14 | Box Culvert | SBP | ___ | 23-11 21 21 11 13 15 | Concrete Box Culverts |
| 15 | Chamber | SBH | Multiple* | 23-39 29 13 19 17 | Surface water retention chamber |
| 16 | Inlet | SIH | ___ | 23-39 29 13 19 13 | Combination storm drain and underdrain inlets |
| 17 | Nullah | SNP | ___ | 23-11 21 21 15 | Channels |

Manhole

A manhole is an opening to a confined space such as a shaft, utility vault, or large vessel. Manholes are often used as an access point for an underground public utility, allowing inspection, maintenance, and system upgrades.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Manhole Foul Water/ Sewer

A manhole is an opening to a confined space such as a shaft, utility vault, or large vessel. Manholes are often used as an access point for an underground public utility, allowing inspection, maintenance, and system upgrades.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Manhole Storm Water

A manhole is an opening to a confined space such as a shaft, utility vault, or large vessel. Manholes are often used as an access point for an underground public utility, allowing inspection, maintenance, and system upgrades.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Manhole Telephone

A manhole is an opening to a confined space such as a shaft, utility vault, or large vessel. Manholes are often used as an access point for an underground public utility, allowing inspection, maintenance, and system upgrades.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Valve Gas

Valves stop (or regulate) the flow of supply gas.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Valve Waterworks

Valves stop (or regulate) the flow of supply water.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Power cables / lines

Power distribution systems that rely on copper or aluminium busbars to channel electricity, while being enclosed in a housing for protection.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension of cable duct, and concrete surroundings dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of cable containment | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Gas pipe

Pipe used to supply gas.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Telecommunication lines

Telecommunication/networking systems that rely on copper or aluminium busbars to channel signals, while being enclosed in an housing for protection.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of cable containment | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Sewage Pipe

A sewage pipe is a conduit to remove unwanted sewage.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Storm Water Pipe

Conduits used to carry off rainwater.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Fresh watermain / Salt water main

Pipe used to supply water services.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object with approximate size/dimension and dynamic envelope to indicate approximated spaces if applicable | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Box Culvert (Sewerage) / Tunnel (Sewerage)

A culvert is a structure that channels sewerage water past an obstacle or to channel a subterranean waterway. Typically embedded to be surrounded by soil, a culvert may be made from a pipe, reinforced concrete, or other material

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including the outline of slab and box | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Chamber

A manhole (alternatively person hole, utility hole, maintenance hole, or sewer hole) is an opening to a confined space such as a shaft, utility vault, or large vessel. Manholes are often used as an access point for an underground public utility, allowing inspection, maintenance, and system upgrades.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including wall, slab, frame and cover with approximate size/dimension and dynamic envelope to indicate approximated spaces | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid block | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid block | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Inlet

Inlet is an opening structure to allow incoming of stormwater to a stormwater system from a larger body of water.

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including the outline of slab and box | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

Decked Nullah / Nullah

Nullah refers to an open, usually concrete-lined channel designed to allow rapid drainage of storm precipitation or industrial wastewater from high ground, to prevent flooding of urbanised coastal areas.

Geometrical Requirement

| LOD-G | Content | Size and Shape | Location & Orientation | Appearance | Behavior |
| --- | --- | --- | --- | --- | --- |
| 100 | Conceptual/Schematic 2D symbol | Assumed size | Symbol insertion point(s) approximately located | Symbols/lines | N/A |
| 200 | 3D Object including the outline of slab and box | Assumed / Typical size of component (s) | Object insertion point(s) approximately located | Overall shape 3D solid blocks | N/A |
| 250 | Same as 200 | Same as 200 | Object insertion point(s) is located at / derived from the onsite survey point(s) | Overall shape 3D solid blocks | N/A |
| 300 | N/A | N/A | N/A | N/A | N/A |
| 400 | N/A | N/A | N/A | N/A | N/A |

[Note: To facilitate the review purpose, the codes not available in the BIM harmonisation guideline are highlighted in Yellow]

The BIM Manager should further detail the Omniclass classification to Level 3 according to the actual type of finishes used on site

The BIM Team should further detail the Omniclass classification to Level 3 according to the actual type of sensors installed on site

The BIM Team should further detail the Omniclass classification to Level 3 according to the actual type of finishes used on site